3GPP TSG-RAN WG2 Meeting #97	R2-1701807

Athens, Greece, 13-17 February 2017

Source: 	Huawei, HiSilicon, Qualcomm Incorporated, Sony, Telefonica

Title: 	Proposed Way Forward on Uplink-Based Mobility for UEs in RRC_INACTIVE

Agenda Item:	10.3.1.2

Document for:	Discussion and decision

# Introduction

This document proposes a way forward on uplink-based mobility for NR for UEs .

# Discussion

The following principles and assumptions are proposed.

Proposal 1: Concerning UL-based mobility in inactive state:

- For connected inactive state, DL-based cell reselection is the baseline but the network can alternatively configure the UE to perform UL-based mobility (if supported)

- Earlier phase/release design should not prevent from introducing UL-based mobility in inactive state.

Proposal 2: Uplink-based mobility in inactive state operates on the following principles:

- The UE maintains downlink sync as usual; uplink sync is assumed within some tolerance

- The network configures the UE (via broadcast or unicast signalling) to transmit an UL signal.

  - FFS if an ACK for the RS transmission would be required

- The UE signal allows the network to uniquely identify the UE within a geographical area, by using a UE specific configuration or including the RAN based UE-ID in the UL signal.

- The network measures the UE UL signal at various TRPs and determines which TRP(s) best serve the UE

- UE monitors a DL signal to check if the UL signal configuration is still valid.

  - FFS type of the DL signal (e.g. "regular" sync signal or another signal)..

- DL-based measurements may be enabled by the UE when necessary, e.g. when the UE approaches the edge or has left the area where the UL signal configuration is valid.

- The UE transmits the UL signal once per paging cycle. FFS if any further optimization is applied for the uplink reference signal transmission interval.

- The UE transmits an uplink reference signal

  - E.g. based on some configuration given by the network via either unicast message or system information.

- The UE can receive paging on its regular POs in inactive state

- No special requirement to read SI for the case of UL based mobility, compared with the DL based mobility

Proposal 3: The following items remain FFS for inactive state (some requiring RAN1 input):

- How the network configures potential target gNBs for uplink measurement

- Design (format/channel) of the uplink RS, choice of signatures, etc.

  - Related to how many signatures can be distinguished, and how often signature changes are needed

- Level of synchronisation required between TRPs that measure a UE UL signal

- If there would be differences in the mechanism for high frequency, e.g. relaxing some of the above requirements on UE to deal with the effects of beam sweeping