3GPP TSG RAN WG2 #97	R2-1702001

Athens, Greece, 13th to 17th February 2017

Agenda Item:		10.3.1.1.2

Souce:		Samsung

Title:		Beam Refinement Considering RRM Measurement based on

Idle Mode RS

Document for:	Discussion/Decision

Motivation

※ Note: The validity of the issue dealt with in this contribution depends on the RAN1 study on additional RS. As a result, we believe that it will be possibly discussed during the WI phase based on the final RAN1 decision.

RAN2 is now actively discussing RRM measurement for both idle and connected mode UEs in order to support cell-level mobility in NR. While RAN2 agreed that the RRM measurement framework of NR is based on that of LTE, RAN1 are still studying which RS (Reference Signal) will be used for the RRM measurement in NR. With regard to this aspect, the following agreements were made in the RAN2 #96 meeting.

Agreements for connected active: - The scope of the RRM measurement should mainly be to facilitate the RRC driven ‘cell’ level mobility. - RAN2 working assumption to be confirmed by RAN1: - Connected active mode RRM measurement and reporting based on at least the signals used by idle mode RRM measurement should be supported in NR. - Additional RS may need to be used for RRM measurement in the connected active mode besides the signals used by idle mode RRM measurement, which is dependent on RAN1’s decision. - The RRM measurement framework (measurement object, measurement id, report config) in LTE as a baseline in NR.

According to the RAN2 agreements above, it is expected that more than one RS may be used by a connected mode UE to perform RRM measurement. RAN1 further studied this aspect in their NR ad-hoc meeting on January 2017 and made the following agreements.

Agreements: - The following always-on signals are used for RRM measurement for L3 mobility in IDLE mode: - NR synchronization signal, or - NR synchronization signal, and additional DM-RS for PBCH if DM-RS is supported for PBCH, or - Note: How to use DM-RS for RRM measurement is up to UE implementation - DM-RS for PBCH if DM-RS is supported for PBCH - Note that down selection will be needed if DM-RS for PBCH is supported - For CONNECTED mode RRM measurement for L3 mobility, the following RS can be used if needed, in addition to IDLE mode RS: - FFS: CSI-RS, - FFS: RS separately designed from CSI-RS - Note that possibility of multiplexing of wideband RS in SS block is not precluded

The RAN1 agreements above mentioned that the always-on signals (e.g., synchronization signals and/or DM-RS for PBCH) are used by both idle and connected mode UEs for the purpose of RRM measurement. Moreover, the use of additional RSs is now being considered for a connected mode UE.

Note that both idle and connected mode UEs in LTE perform RRM measurement using CRS (Cell-specific RS), which is transmitted on the same analog beam pattern with data (i.e., PDSCH). If the analog beam pattern for the idle mode RS (i.e., the RS used by an idle mode UE for RRM measurement) is different from that for the connected mode RS in NR, this aspect can be considered to be a noticeable difference from LTE. In this context, we focus on the following issue.

- If the analog beam pattern for the idle mode RS is different from that for the connected mode RS, what is the impact of this on mobility procedures?

Please note that this issue is valid only when the following assumptions are held.

- Additional RS for a connected mode UE (i.e., the connected mode RS) is introduced by RAN1.

- The analog beam pattern for the connected mode RS is different from that for the idle mode RS.

- The connected mode RS is not an “always-on” signal, regardless of cell-specific or UE-specific.

  - It means that triggering cell (re)selection or handover can be solely based on the idle mode RS without measuring the connected mode RS.

Discussion

First of all, we remind that NR supports both single beam and multi-beam systems. Furthermore, it was agreed that the RRM measurement in NR should be based on a common framework regardless of network beam configuration (e.g., number of beams) and UE beam configuration. Under this situation, as a gNB is equipped with a larger number of beams, it is more plausible that the analog beam pattern for the idle mode RS is different from that for the connected mode RS due to the following reasons.

- The idle mode RS (e.g., synchronization signals and/or DM-RS for PBCH) is always-on and should be swept to cover all directions within a cell. Due to such “always-on” and “beam sweeping” natures, transmitting the idle mode RS using very narrow beams should cause large overhead in terms of radio resources, which should be avoided.

- If the connected mode RS is introduced, it is mainly used to select the beam(s) for data Tx/Rx. Then, it is desirable that a gNB transmits the connected mode RS using the beams that are also used to transmit data, which may be transmitted using narrow beams to obtain high (analog) beamforming gain.

- Based on ① and ②, using narrow beams is beneficial to the connected mode RS, but not to the idle mode RS, and vice versa. As a result, it is expected that the analog beam pattern for the idle mode RS can be different from that for the connected mode RS.

Observation 1: The analog beam pattern for the idle mode RS can be different from that for the connected mode RS due to the different characteristics of idle and connected mode RSs.

We now focus on how different analog beam patterns of idle and connected mode RSs have an impact on mobility procedures. Let’s investigate the RRC connection establishment and handover procedures as an example.

Example 1: RRC connection establishment procedure

In this example, the idle mode UE first measures the idle mode RS and performs the RA (Random Access) procedure towards a cell for RRC connection establishment. Then, the UE and the gNB exchange the RRC messages, as shown in Fig. 1. If the UE’s state is changed from idle to connected, the UE is anyway required to select a set of beams for data Tx/Rx, which can be performed by measuring the connected mode RS. Obviously, it is desirable that the UE can use these refined beams as soon as possible after the RRC connection is established. One option is to allow the UE to find the refined beams by measuring the connected mode RS in parallel with the RA and RRC connection establishment procedures.

Figure 1 RRC connection establishment procedure

Example 2: Handover procedure

We herein assume that the connected mode UE measures the idle mode RS to trigger measurement reporting, which can happen according to the current RAN1/2 agreements. If the source gNB initiates handover after coordinating with the target gNB, it sends the handover command to the UE. Then, the UE performs the RA procedure and sends the handover complete message to the target gNB.

As observed in the previous example, it is also desirable that the UE can use the refined beams in the target gNB as soon as possible after the handover is complete. One option is to allow the UE to find the refined beams by measuring the connected mode RS in parallel with the handover procedure.

Figure 2 Handover procedure

From the two examples above, we observe that the RRM measurement based on the idle mode RS requires the UE to refine the beams for data Tx/Rx, which can be done by measuring the connected mode RS. In this context, we have the following observation and proposal.

Observation 2: The RRM measurement based on the idle mode RS requires the UE to refine the beams for data Tx/Rx, which can be done by measuring the connected mode RS.

Proposal: It is required to investigate how the UE refines beams in case where RRM measurement is based on the idle mode RS, which can be transmitted using the different analog beam pattern with the connected mode RS and/or data. This issue will be possibly discussed during the WI phase based on the RAN1 decision about additional RS.

Conclusions

Observation 1: The analog beam pattern for the idle mode RS can be different from that for the connected mode RS due to the different characteristics of idle and connected mode RSs.

Observation 2: The RRM measurement based on the idle mode RS requires the UE to refine the beams for data Tx/Rx, which can be done by measuring the connected mode RS.

Proposal: It is required to investigate how the UE refines beams in case where RRM measurement is based on the idle mode RS, which can be transmitted using the different analog beam pattern with the connected mode RS and/or data. This issue will be possibly discussed during the WI phase based on the RAN1 decision about additional RS.