3GPP TSG-RAN WG2 Meeting #97	R2-1702142

# Athens, Greece, 13th - 17th February 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0358 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on reselection for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sequans Communications, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  | Date: |  |  | 2017-02-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The expression “Ranking with cell selection criterion S for enhanced coverage is applied” in 5.2.4.6a is unclear and does not seem correct. It seems there are 2 possible interpretations, both wrong: - perform ranking as in 5.2.4.6 but use the “cell selection criterion S for enhanced coverage”, instead of the “cell-ranking criterion Rs” - perform ranking as in 5.2.4.6 but while doing so, only cells that fulfil cell selection criterion S for enhanced coverage are considered. However cells on frequency layers for which CE is not supported does not have Qrxlevmin_CE and “cell selection criterion S for enhanced coverage” does not exist for them. - The condition “when the current serving cell can only be accessed using enhanced coverage” in 5.2.4.6a is unclear as it can be understood in 2 ways: - the cell can only be accessed using CE mode (A or B). - the cell can only be accessed while being in enhanced coverage (i.e. according to cell selection criterion S). - Use of “cell can be accessed” is not clear |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “with cell selection criterion S for enhanced coverage”, replace by “as defined in sub-clause 5.2.4.6” (which is used at other places when referring to ranking procedure). Also clarify that frequency priorities, if any, are not considered. According to 5.2.4.6 the UE will rank “all cells that fulfil the cell selection criterion S”. This encompasses both cell selection criterion S for normal or enhanced coverage. - Clarify that the 2nd interpretation is the intended one. - Clarify the wording Impact analysis Impacted functionality: Cell reselection. If the network is implemented according to the CR, but the UE is not: No inter-operability issue. This CR clarifies UE behaviour only. If the UE is implemented according to the CR, but the network is not: No inter-operability issue. This CR clarifies UE behaviour only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell reselection may not be performed as expected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.1, 5.2.4.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### *****************CHANGE STARTS*****************

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCConnectionReject unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). While the UE is camped on a suitable CSG cell in normal coverage, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than any of the network configured values), irrespective of any other priority value allocated to this frequency. If the UE capable of sidelink communication is configured to perform sidelink communication and can only perform the sidelink communication while camping on a frequency, the UE may consider that frequency to be the highest priority. If the UE capable of V2X sidelink communication is configured to perform V2X sidelink communication and can only perform the V2X sidelink communication while camping on a frequency, the UE may consider that frequency to be the highest priority. If the UE capable of sidelink discovery is configured to perform Public Safety related sidelink discovery and can only perform the Public Safety related sidelink discovery while camping on a frequency, the UE may consider that frequency to be the highest priority.

#### *****************NEXT CHANGE*****************

#### 5.2.4.6a	Reselection for enhanced coverage

Ranking as defined in sub-clause 5.2.4.6 is applied for intra-frequency and inter-frequency cell reselection (irrespective of configured frequency priorities, if any) while the UE is in enhanced coverage.

#### *****************CHANGE ENDS*****************