3GPP TSG-RAN WG2 Meeting #97bis					R2-1702524

Spokane, USA, 3rd – 7th April 2017

Source:	vivo

Title:	Use cases and signalling for UE initiating radio capability update

Agenda Item:	10.4.1.7

Document for:	Discussion and Decision

# Introduction

At the NR SI agreements, the UE capability signalling was discussed and made the following agreements [1]:

1 The UE reports its UE capability at least when the network requests. 2 The gNB can request what capabilities for the UE to report (e.g. similar band and band combination requests in LTE). Details to be finalised in stage 3. 3 The temporary capability restrict should be transparent to the NG core, i.e. only static capability is stored in the NG core.

Moreover, the common sense is that we should firstly identify some specific use cases for UE capability update. In this contribution, we further discuss the use cases and signaling for UE initiating UE radio capability and provide some suggestions.

# Discussion

## Use cases for UE initiating UE capability update

During the NR SI discussion, some contributions are raised to provide the use cases for UE initiated UE capability update [2][3][4]. Besides the introduced use cases, we think that UE power saving and battery life optimization is also important use case for UE dynamically initiating capability update.

The smart phone power consumption is too fast and users have to charge the smart phone frequently so as to satisfy the more frequent use of smart phone. Battery life becomes one of the pain points of smart phone user experience. Therefore, how to reduce the UE power consumption is becoming more and more important for the terminal vender. Now, much effort has been made from the perspective of the hardware and software to reduce UE power consumption. Besides the mechanism of UE power saving from UE designs itself, it is very important to introduce the flexibility of UE power saving from the air interface perspective. Especially, the smart phone can degrade the air interface performance to reduce the UE power consumption in some cases. For instance, when the battery is very low and cannot be connected to power charger, UE can initiate its radio capability degradation for longer service. When the UE is in charging, UE can initiate UE radio capability update to recover air interface performance.

On the other hand, over-heating is another use case that the UE can initiate radio capability update to degrade the MIMO rank and carrier numbers etc. to cool down the Modem and RF temperature to avoid UE rebooting.   Another use case is the UE as WiFi access point (AP). It requires higher UE radio capability in good battery condition.

Observation 1: UE initiating the radio capability update provides the smart phone venders much flexibility to improve the user experience in many cases.

## UE initiating UE capability update procedure

In LTE, based on the assumption that the UE capability is relatively static and the UE does not change the UE capability frequently, the UE can only update the UE capability by detach and reattach procedure, which leads to service interruption and degrades the user experience. Therefore, we consider that NR UE should support more dynamic updates of UE radio capability without detach and reattach procedures.  In order to improve the user experience, the UE radio capability update should not interrupt UE ongoing service.

Figure 1 shows an example of the signalling procedure of UE initiating UE radio capability update. In Figure 1, the UE firstly reports the full UE capability during attach procedure, and then MME stores it. If UE wants to update its UE radio capability, UE can trigger UE radio capability request, and then gNB sends the confirm information to UE. Then, according to the feedback from gNB, the UE reports the updated UE radio capability to gNB. Based on the updated UE radio capability, gNB can perform reconfiguration for the UE.

Figure 1 UE initiate UE capability update procedure

Moreover, for the UE radio capability update request message, it can contain the cause of UE radio capability update. And it also can tell the gNB which types of UE radio capability will be updated. And the updated UE radio capability is not exceeding UE radio capability which is reported during the attach procedure. For the UE radio capability information, we consider at least that the following parameters can be updated:

- Band/Band combination

- RF

- RX/TX capability, such as CA, DC, CoMP, NAICS and MIMO configurations.

- UE category

Based on the above discussion, we have the following proposals:

Proposal 1: The UE should be able to initiate the UE radio capability update without detach/reattach procedure.

Proposal 2: The three-step procedure for UE initiating radio capability update can be considered.

# Conclusion

In this contribution, we discuss the use case and signaling procedure for UE initiating the UE radio capability update. Based on the discussion, we have the following observation and proposals:

Observation 1: UE initiating the radio capability update provides the smart phone venders much flexibility to improve the user experience in many cases.

Proposal 1: The UE should be able to initiate the UE radio capability update without detach/reattach procedure.

Proposal 2: The three-step procedure for UE initiating radio capability update can be considered.

# References

- R2-1700xxx, Draft Report of 3GPP TSG RAN WG2 Ad Hoc on NR, Athens, Greece, February 13-17, 2017

- R2-1700576,  Considerations on NR UE capability reporting, Qualcomm Incorporated

- R2-1700323, UE capability signaling framework for NR, Intel

- R2-1700293, Further discussion on UE capability signaling, HTC