3GPP TSG-RAN WG2 #97bis	 R2-1702531

Spokane, USA, April 3rd- 7th, 2017

Agenda Item:	10.4.2.2

Source: 	Xiaomi

Title:  	Idle mode mobility

Document for:	Discussion and Decision

Introduction

RAN2 has made the following agreements so far regarding to idle mode cell selection and reselection:

RAN2 #95 Agreements:

1:	In the 5G system, the UE camps on the best cell

FFS how the UE determines the best cell

RAN2 NR Ad Hoc Agreements:

Agreements:

1:	LTE cell selection and reselection mechanisms are the baseline for NR.

2:	In multi-beam operations, measurement quantity of a cell is derived amongst the beams corresponding to the same cell.

FFS how to derive the cell level measurement quantity from multiple beams (may or may not be different from connected).

1	As in LTE, UE can prioritise a frequency based on service. On the selected frequency the UE attempts to camp on the best cell.

2	Suitability criterion: Cell quality is above a threshold; Cell is not barred; Cell belongs to selected/R (E) PLMN. Other conditions (if any) are FFS.

3	Cell broadcasts (e.g. in minimum SI) the service(s) supported by it.

FFS for which services (e.g. MBMS, CSG, V2X, URLLC) we apply this mechanism.

FFS how this might apply for the case of network slices.

FFS whether a cell may also broadcast service(s) supported in neighbouring frequencies.

In this contribution, we will look at issues of cell selection and reselection that needs to be considered, i.e. state and state transition, services support, reselection mechanisms.

Discussion

State and state transitions in Idle Mode

In LTE, different states have been defined for idle mode UE, and different cell selection & reselection procedure may apply for the transition between different states. Fig 1 [1] is an illustration of the state transition. In NR, a new RRC state "inactive state" is defined, and direct state transition from inactive state to idle mode was agreed during last meeting, but the detailed cases and mechanisms are FFS. Fig 1 may need to be updated according to the further detailed progress on RRC state transition between inactive state and idle mode. But any way, NR idle mode states should base on LTE idle mode states. Additional transitions can be added in future to accommodate inactive state.

Fig 1 cell selection and reseletion in idle mode

Proposal 1: NR idle mode supports all the states defined in LTE idle mode and adopts the LTE idle mode state transition as baseline.

support for diverse services e.g. CSG/MBMS/sidelink/URLLC

In LTE, separate cell selection and reselection procedures are defined for NB-IoT and other cases. NR is supposed to support more diverse verticals (eMBB, URLLC, mMTC), a unified cell selection and reselection framework is preferred to accommodate all the services.

Proposal 2: A unified cell selection and reselection framework should be applied for NR to accommodate all the supported services.

During last RAN2 meeting, for the services supported, RAN2 agreed to prioritise a frequency based on the service, and on the selected frequency the UE attempts to camp on the best cell. However, it is unclear whether this mechanism is for cell reselection only or can also be used for cell selection.

In LTE, for cell selection, UE performs suitability check only, there is no service availability check for frequency prioritization. It is checked when UE performs cell reselection, and UE considers the frequency that provides the services UE interests as the highest priority frequency. The underline reason is that cell selection is for UE to acquire normal service as soon as possible. Suitability check is used to guarantee that purpose. Whereas service availability check for frequency prioritization is used for check a specific service UE interests, with or without which will not impede UE from receiving normal service. Thus, we think that agreement 1 is for cell reselection only.

Proposal 3: Clarify that frequency prioritization based on service is only applicable to cell reselection, and can be used for services e.g. MBMS, sidelink, CSG.

In LTE, CSG is supported. A user can obtain normal service from a CSG cell only when the cell is a CSG member cell. Therefore, the check for CSG is included in suitability criterion. For NR, we see no reason to deviate from this principle if CSG is supported.

Proposal 4: suitability criterion includes "For a CSG cell, the cell is a CSG member cell for the UE".

Other cell reselection mechanisms to support

Last RAN2 meeting agreed to use LTE cell reselection mechanisms as baseline. LTE cell reselection mechanisms include:

- intra-frequency cell ranking

- inter-frequency absolute priority

- inter frequency redistribution priority

- neighbor frequency specific common reselection parameters

- neighboring cell lists

- black lists

- speed dependent reselection

- Per UE or UE group specific reselection parameters

Proposal 5: NR cell reselection supports all the LTE cell reselection mechanisms, includes:

- intra-frequency cell ranking

- inter-frequency absolute priority

- inter frequency redistribution priority

- neighbor frequency specific common reselection parameters

- neighboring cell lists

- black lists

- speed dependent reselection

- Per UE or UE group specific reselection parameters

Further analysis of some of the mechanisms are given as follow:

inter frequency redistribution

From operator point of view, it is desirable that UEs' distribution among frequencies and cells can be controlled by operator during cell reselection stage to avoid access overload afterwards. LTE enables this operation by supporting inter-frequency E-UTRAN reselection based on redistribution priority. SIB3 & SIB5 will broadcast the redistribution factor of the serving frequency (cell) and neighbor frequencies (cell) respectively. Redistribution factor determines the ratio of UE distributed to the target frequency or cell. A UE may be redistributed to a frequency or cell based on the redistribution factor and frequency priority. UE will consider it as having the highest priority for a period of time. The redistribution can also be triggered by paging.

For NR, we think redistribution should at least support inter frequency redistribution. For inter-RAT redistribution, we should at least consider E-UTRA with 5G-CN case. As we know, the candidate 5G radio includes evolved LTE and NR. The operator may deploy the eLTE and NR at the same time. eLTE will help support legacy LTE UEs and provide some 5G services at the same time. NR will be used mainly for providing 5G services. The load redistribution between eLTE and NR may be necessary in this case.

Proposal 6: NR cell reselection should support redistributing a UE to a frequency or cell. For inter-RAT redistribution, NR cell reselection should at least support E-UTRA with 5G-CN.

Neighbouring cell lists and black lists

LTE provides neighbouring cell lists in SIB4 and SIB5 to indicate cell specific reselection parameter (i.e. cell reselection offset) for intra- and inter- frequency cell reselection. Black lists are also provided in SIB4 and SIB5 to prevent UE from reselecting to these cells. Inter-RAT neighbouring cell lists and black lists are not supported. For NR cell reselection, it should at least support intra- and inter- frequency neighbouring cell lists and black lists. Considering the tight interworking between LTE and NR, NR cell reselection should consider the support of LTE neighbouring cell lists and black lists.

Proposal 7: NR cell reselection should support intra frequency and inter frequency neighbouring cell lists and black lists, and should at least support LTE neighbouring cell lists and black lists for inter-RAT case.

Mobility state based reselection parameter scaling

In LTE, cell reselection takes into account the UE moving speed. The faster UE moves, the smaller reselection timer and the lower neighbor cell threshold should apply so as to quickly reselect to the new cell before too late. Three mobility state are defined for LTE idle mode: High-mobility, Medium-mobility and Normal-mobility. For High-mobility and Medium-mobility state, separate speed dependent Scaling Factors are applied to reselection parameters, i.e. Qhyst, Treselection. For NR, we think cell reselection should also consider the mobility state based scaling mechanism as the baseline.

Proposal 8: NR cell reselection should consider the mobility state based scaling mechanism as the baseline.

Conclusions

Based on the discussion, we have the following proposals:

Proposal 1: NR idle mode supports all the states defined in LTE idle mode and adopts the LTE idle mode state transition as baseline.

Proposal 2: A unified cell selection and reselection framework should be applied for NR to accommodate all the supported services.

Proposal 3: Clarify that frequency prioritization based on service is only applicable to cell reselection, and can be used for services e.g. MBMS, sidelink, CSG.

Proposal 4: suitability criterion includes "For a CSG cell, the cell is a CSG member cell for the UE".

Proposal 5: NR cell reselection supports all the LTE cell reselection mechanisms, includes:

- intra-frequency cell ranking

- inter-frequency absolute priority

- inter frequency redistribution priority

- neighbor frequency specific common reselection parameters

- neighboring cell lists

- black lists

- speed dependent reselection

- Per UE or UE group specific reselection parameters

Proposal 6: NR cell reselection should support redistributing a UE to a frequency or cell. For inter-RAT redistribution, NR cell reselection should at least support E-UTRA with 5G-CN.

Proposal 7: NR cell reselection should support intra frequency and inter frequency neighbouring cell lists and black lists, and should at least support LTE neighbouring cell lists and black lists for inter-RAT case.

Proposal 8: NR cell reselection should consider the mobility state based scaling mechanism as the baseline.

References

[1] TS36.304, User Equipment (UE) procedures in idle mode.