3GPP TSG-RAN WG2 #97bis	 R2-1702532

Spokane, USA, April 3rd- 7th, 2017

Agenda Item:	10.4.1.5

Source: 	Xiaomi

Title:  	Consideration on the transition between inactive state and idle state

Document for:	Discussion and Decision

Introduction

RAN2 has made the following agreements regarding to the state transition from INACTIVE to IDLE:

The RRC state transition from INACTIVE to IDLE is supported

FFS For what cases this transition is supported (e.g. reject from network, other failure cases, other cases, etc)

In this contribution, we will address the cases that UE needs to transit from INACTIVE state to idle mode, and that UE may stay in inactive state.

Discussion

We list some of the cases that need to be considered for UE in inactive state.

Cases that UE may need to transfer to idle state

During previous meeting, it was agreed that for the inter-RAT mobility between NR and RATs other than E-UTRA, UE moves to idle state. And for the case that E-UTRAN connects to EPC and/or does not support RRC_INACTIVE/light connected, UE also moves to idle state. In this section, we will consider other cases that UE needs to move to idle state.

# Case 1: Network rejects UE's request for transition to connected mode

When an inactive state UE initiate the request for transition to connected mode, network may reject the request with the indication of leaving inactive state for reasons like unable to identify the UE or acquire the UE context, as network locally deleted UE context as a consequence of UE contexts overload or unable to reach the UE. In this case, UE should be able to transfer to idle mode directly.

# Case 2: Network rejects UE's request for updating RAN notification area

In RAN2 #96 meeting, the concept of RAN based notification area was agreed, the UE may initiate the RAN notification area update request to the network upon area change. The network may reject the request with the cause that notification area is not allowed, or UE can not be identified, etc. In this case, UE should be able to transfer to idle mode directly.

# Case 3: AS and NAS state mismatch in UE side

Similar to the ongoing discussion in light connection, an inactive state UE may be modelled to EMM-idle or EMM-connected state in NAS. It is inevitable that NAS state may mismatch with RAN state. As we know, EMM-idle and EMM-connected state corresponds to UE registered to network (EMM-REGISTERED). In certain cases (e.g. UE deactivates all EPS bearer context locally), UE may locally detach from the network and enter EMM-DEREGISTERED state. If such a situation happens, it is unnecessary for UE to keep in inactive state, storing the AS context and performing RAN notification area update, or initiating resuming procedure (network may even accept the resuming request if there is state mismatch between UE and network). There should be a mechanism to handle the mismatch, by moving UE to idle mode and/or by initiating connection setup procedure.

# Case 4: USIM is pulled out or NAS indicates PLMN is not allowed

For this case, UE should move to idle state.

Proposal 1: For the following cases, RAN2 should also consider support for UE moving from inactive state to idle mode:

- Network rejects UE's request for transition to connected mode

- Network rejects UE's request for updating RAN notification area

- AS and NAS state mismatch in UE side

- USIM is pulled out or NAS indicates PLMN is not allowed

Cases that UE may keep in inactive state

# Case 1: UE selects to a cell providing limited service

In this case, the UE can only initiate emergency service. But if UE does not need to access to the network (in LTE, TAU is not initiated when entering EMM-REGISTERED-LIMITED SERVICE state. UE reattaches to the network when there is emergency service), it might be better for UE to stay in inactive state. Thus, when UE moves out of the limited service cell, it is able to keep in inactive state. When UE stays in the limited service cell, upon upper layer request, UE may enter idle state and initiate RRC connection request.

# Case 2: UE unable to find a cell to camp on

In this case, UE is not able to have any communication with the network. Staying in inactive state enables UE keeping in inactive state when suffering a short period of coverage hole. We see no harm for this operation. Optionally, if UE cannot find a cell to camp on for a long period, UE may discard the stored AS context, which can be implementation based. Then UE returns to idle mode.

# Case 3: Reselection between NR cell and E-UTRA cell

During the discussion in the previous meeting, RAN2 discussed whether to support remaining in RRC_INACTIVE/light connected for the case that E-UTRA connected to NGC and E-UTRAN supports light connection. In our understanding, it has the advantage of avoid frequent configuration.

# Case 4: UE moves into a forbidden RAN notification area/cell

Similar to forbidden Tracking Area, we suppose that there will be the concept of forbidden RAN notification area/cell to stop UE from accessing the cell for inactive state service. UE can also stay in inactive state if there is no communication with the network.

# Case 5: Transmission or reception failure

When occurring data or RRC signalling transmission or reception failure, UE can stay in inactive mode.

Proposal 2: At least for the following cases, RAN2 should consider support keeping UE in inactive state:

- UE selects to a cell providing limited service

- UE unable to find a cell to camp on

- Reselection between NR cell and E-UTRA cell that is connected to NGC and E-UTRAN supports light connection

- UE moves into a forbidden RAN notification area/cell

- Transmission or reception failure

Conclusions

Based on the discussion, we have the following proposals:

Proposal 1: For the following cases, RAN2 should also consider support for UE moving from inactive state to idle mode:

- Network rejects UE's request for transition to connected mode

- Network rejects UE's request for updating RAN notification area

- AS and NAS state mismatch in UE side

- USIM is pulled out or NAS indicates PLMN is not allowed

Proposal 2: At least for the following cases, RAN2 should consider support keeping UE in inactive state:

- UE selects to a cell providing limited service

- UE unable to find a cell to camp on

- Reselection between NR cell and E-UTRA cell that is connected to NGC and E-UTRAN supports light connection

- UE moves into a forbidden RAN notification area/cell

- Transmission or reception failure

References