3GPP TSG-RAN WG2 #97bis	R2-1702569

Spokane, USA, 3rd-7th April 2017

Agena Item:     8.25

Source:	Apple

Title:	Report of email discussion [97#60][LTE/TEI14] Asymmetric RoHC (Apple)

Document for:		Discussion

# 1	Introduction

During the RAN2#97, the need for relaxing 3GPP PDCP layer limitation of symmetric ROHC DL/UL configuration parameters has been discussed [1][2]. A consensus has been reached on considering uplink-only RoHC operations as potential enhancement for Release 14.

For LTE, RAN2#97-Bis the chairman requested an email discussion to progress the details around enabling uplink-only RoHC configuration and also to prepare required draft CRs.

[97#60][LTE/TEI14] Asymmetric RoHC

Discuss how asymmetric RoHC would operate and how it could be configured. Focus on UL only. Draft CRs can be discussed.

Intended outcome: Email discussion report

Deadline: Thursday 16/03/2017

# 2    Discussion

RoHC framework has been introduced in LTE Release 8 specification with Voice Over LTE (VoLTE) as main use-case. The benefit of RoHC compression is to reduce the header size of uplink and downlink VoLTE IP-packets, and thereby significantly improve the overall voice calls capacity of the Network.

Because of the bidirectional nature of VoLTE traffic, ROHC configuration for the same PDCP entity has been restricted to be similar between downlink and uplink channels. However, ROHC framework already supports multiple traffic’s profiles such as TCP/IP in addition to RTP/UDP/IP VoLTE profile.

During the RAN2#97, the topic of allowing ROHC configuration for uplink-only and downlink-only channels has been discussed and a consensus has been reached to focus discussions around:

- The details of uplink-only RoHC compression operations (feedback packets included).

- Required configuration parameters for uplink-only RoHC channel operation’s mode.

- Drafting CRs to introduce uplink-only RoHC compression.

## 2.1 Uplink-Only RoHC operations

As discussed during RAN2#97, uplink-only RoHC operation will be a subset of Bidirectional RoHC channels defined today in the 3GPP specification [3]. The main difference is the flexibility to configure RoHC parameters for uplink channel only in addition to legacy procedure that use a single set of parameters which apply for both uplink and downlink channels belonging to the same PDCP entity.

Companies are invited to provide details regarding additional aspects/problems in UE or network side that need specific attention when uplink-only channel compression is enabled (uplink RoHC channel feedback included).

| Company | Description of Uplink-Only RoHC UE/NW Operations | Comments |
| --- | --- | --- |
| MediaTek |  | Figure assums that network can decide whether to turn on UL-only RoHC. |
| Intel | Due to performance constraints in the UE it may be necessary to restrict the use of ROHC to one direction from the UE side. Therefore, the UE capabilities in terms of PDCP parameters shall be extended to indicate the UE support of ROHC for UL direction. In addition, a maximum data rate shall be added to the unidirectional ROHC capabilities in order to reflect the additional performance constraints added by ROHC on top of the UE category. The value shall define the maximum data rate the UE is capable to process on all DRBs on which ROHC is configured by the network. The NW configures the ROHC channels per DRB in PDCP-Config according to the UE capabilities and the expected use case. In general, to our understanding the restriction to apply ROHC in UL only does not influence the existing ROHC configuration parameters, i.e. there is no need to specify two separate sets of parameters if bidirectional ROHC is configured. In this case, the configuration shall still be the same for both directions like stated in TS 36.323, 5.5.3. Furthermore, using unidirectional ROHC channels does not change the way how feedback packets are sent, i.e. feedback control PDUs are sent in the other direction as with the current bi-directional ROHC configuration. |  |
| Apple | It is proposed that the UE inform the eNB about its support of RoHC profile 6 uplink-only Operations. When the eNB configure the UE with RoHC uplink Only operation in profile 6: 1. The UE compresses the uplink TCP data flow in uplink channel while the eNB does not compress the downlink TCP data flow in the downlink channel. 2. The eNB could use the downlink uncompressed ROHC channel to transmit feedback packets for the compressed uplink ROHC channel. |  |
| Motorola Solutions | We share the same view as MediaTek and Apple but with a bit more clarification details as follows: UE indicates its capability of uplink-only RoHC operation support to eNB. eNB decides if it wants to invoke an uplink-only RoHC operatiron or not. If yes, sends RRC Reconfiguration to UE to configure a DRB or a set of DRBs for uplink-only RoHC operation. The RoHC configuration is per DRB in PDCP-Config. Otherwise, no RoHC configuration is sent to UE. Also, we see the development between uplink-only and downlink-only RoHC solution be symetteric. To be future-proofing, we suggest constructing the downlink-only RoHC solution at the same time. |  |
| Nokia | We share the view of MediaTek, Apple and Motorola Solutions. In addition the following description from 36.331 should be taken into account: 36.331 states: headerCompression E-UTRAN does not reconfigure header compression for an MCG DRB except for upon handover and upon the first reconfiguration after RRC connection re-establishment. | We assume that the same applies to UL only RoHC reconfiguration. |
| NTT DOCOMO | For initiation of UL only ROHC, we share the views from MediaTek, Apple and Motorola Solution. Furthermore, it would be better to have common understanding on the modelling of “no compression in DL”. There may be 2 interpretations: 1) There are no compressor and decompressor for DL (or all the DL packet will bypass compressor and decompressor) 2) There are compressor and decompressor even for DL, and all the DL packets will be mapped to uncompressed session. If companies see such possibile interpretations, it may be safer to have common understanding for good inter-operability. Also, it may be better to have some guidline on how eNB should transmit DL packet when UL only ROHC is configured, e.g., only uncompressoin session should be used or only specific context ID should be used, etc. |  |

Observation 1: 8 companies provided feedback in the email discussion

Observation 2: There is consensus to support uplink only ROHC based in following procedure:

- UE to report uplink-only ROHC capability indication.

- eNB to reconfigure the DRB for uplink-only ROHC

- uplink ROHC channel feedback is supported which imply that the legacy behavior is preserved. i.e ) There are compressor and decompressor even for DL, and eNB will be responsible of the mapping of downlink packet to uncompressed session.

Observation 3: One company expressed interest for the UE ro provide the maximum data rate

Observation 3: One company expressed interest on investigation the benefits of DL-Only RoHC.

Observation 4: Two companies expresse the need to introduce some clarification on network behaviour in the specification.

## 2.2 Uplink-Only RoHC Configurations

Companies are invited to describe the parameters (UE capability included) that should be introduced to configure and enable uplink-only RoHC channel operations.

| Company | Parameters for configurations | Motivation for the configuration parameters | Comments |
| --- | --- | --- | --- |
| Qualcomm | One bit in UE capability to indicate whether the UE supports this R14 UL-only RoHC operation for each RoHC profile. eNB RRC reconfiguration parameter to allow UE to use this R14 UL-only mode RoHC operation for a given logical channel. | UE capability: To allow eNB to understand whether the UE supports this new feature or not. We may need a new IE similar to the IE supportedROHC-Profiles in 36.331. eNB configuration: to allow eNB to control when and whether to start using this new feature. We may need a new IE similar to the current IE headerCompression in 36.331. |  |
| MediaTek | Agree with QC on UE capability and RRC reconfiguration. In addition to that, optionally eNB can configure UE to request UL-only RoHC in UEAssistanceInformation. UE then request UL-only RoHC when there is a lot of TCP ACK. Further comment Since there is no use case provided by DL only RoHC, we suggest to only provide ignaling for UL nly RoHC, i.e. 1 bit for enabling. Right now, RoHC does not specify max data rate which is handled by RoHC implementation. Therefore, we do not see the need to specify max date rate for RoHC. | Since the performance of UL-only RoHC depends on UL traffic mix, if needed, UE can provide UE assistant information to trigger UL-only RoHC. |  |
| Intel | - 36.306, 36.331: UE-EUTRA-Capability, pdcp-Parameters: - ROHC direction support per supported ROHC profile - Max ROHC data rate B) 36.331: PDCP-Config, headerCompression: - Add configuration for ROHC direction | To A) With these capabilities the UE can indicate to NW its support of unidirectional ROHC per ROHC profile and the maximum supported aggregated data rate of all DRBs configured with ROHC. To B) The NW can configure the direction in which ROHC has to be used by the UE according to the UE capabilities. Configuring the legacy fields will enable bidirectional usage. The NW can select the best configuration for the expected use case of the connection, minimizing the overhead for the UE and maximizing the compression gain. The potential specification modifications to 36.331, 36.306 and 36.323 are shown below (highlighted in red). Just as an example and to have a complete picture of the modifications, the signalling covers unidirectional ROHC support in UL and DL direction. |  |
| Ericsson | UE Capability of UL only support should be part of UE capability Information. Besides Apple CR R2-1701763 for 36.331 (corrected by Ericsson and attached below) can be used. We do not see the need to explicitly mention the data rates. Moreover, data rates may not reflect the granularity of DL processing complexity that UE needs to carry out. Regarding a UE indication (in UEAssistanceInformation) that UL only RoHC should be enabled, today no such indication exists and we do not see a strong need for such if UL only RoHC is added. It is good to minimize extra ignaling for these scenarios; hence as Qualcomm and others have also proposed, it is sufficient to only have a UE capability. The NW, depending upon various factors can decide when to enable/disable RoHC. | We should aim to have as simple approach as possible without too many parameters. |  |
| Apple | - UE capability: Uplink-Only RoHC - PDCP config: configuration of uplink-only RoHC profile 6. | - UE to inform the eNB about its support of uplink only RoHC profile 6 operations. - eNB to configure the UE with uplink-only RoHC profile 6. |  |
| Motorola Solutions | - Only UE capability and PDCP-Config for uplink-only RoHC operation are necessary. The decision of uplink-only RoHC operation invocation is up to eNB implementation. - Only UE capability and PDCP-Config for downlink-only RoHC operation are necessary. The decision of downlink-only RoHC operation invocation is up to eNB implementation. | - UE capability to indicate uplink-only RoHC operation - New IE in PDCP-Config for eNB to configure UE for uplink-only RoHC operation - UE capability to indicate downlink-only RoHC operation - New IE in PDCP-Config for eNB to configure UE for downlink-only RoHC operation |  |
| Nokia | We see UL-only header compression beneficial in some cases. Furthermore, it could be beneficial to support bidirectional header compression for some profiles while supporting UL-only header compression for some other profiles. This should be taken into account in both capability and configuration signalling. |  |  |
| NTT DOCOMO | We share the view from Qualcomm. 1 bit for UE capability 1bit for PDCP configuration |  |  |

Specification modifications:

36.331:

UE-EUTRA-Capability information element

UE-EUTRA-Capability-v14xy-IEs ::= SEQUENCE {

pdcp-Parameters-v14xy				PDCP-Parameters-v14xy					OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

PDCP-Parameters-v14xy ::=			SEQUENCE {

supportedROHC-Direction-r14			SEQUENCE {

profile0x0001-r14					UniDirectRohcSupport-r14,

profile0x0002-r14					UniDirectRohcSupport-r14,

profile0x0003-r14					UniDirectRohcSupport-r14,

profile0x0004-r14					UniDirectRohcSupport-r14,

profile0x0006-r14					UniDirectRohcSupport-r14,

profile0x0101-r14					UniDirectRohcSupport-r14,

profile0x0102-r14					UniDirectRohcSupport-r14,

profile0x0103-r14					UniDirectRohcSupport-r14,

profile0x0104-r14					UniDirectRohcSupport-r14

},

ue-ROHC-maxDataRateDL-r14 ENUMERATED {ffs} OPTIONAL,
ue-ROHC-maxDataRateUL-r14 ENUMERATED {ffs} OPTIONAL

}

UniDirectRohcSupport-r14 ::=	ENUMERATED { no, ul-only, dl-only, ul-or-dl}

Note: The data rates may be defined according to typical data rate values like e.g. mb1, mb2, mb5, mb10, mb15, mb20, mb25, mb50 in Mbps. Exact values are ffs.

PDCP-Config information element

-- ASN1START

PDCP-Config ::=						SEQUENCE {

discardTimer						ENUMERATED {

ms50, ms100, ms150, ms300, ms500,

ms750, ms1500, infinity

}															OPTIONAL,			-- Cond Setup

rlc-AM								SEQUENCE {

statusReportRequired				BOOLEAN

}															OPTIONAL,			-- Cond Rlc-AM

rlc-UM								SEQUENCE {

pdcp-SN-Size						ENUMERATED {len7bits, len12bits}

}															OPTIONAL,			-- Cond Rlc-UM

headerCompression					CHOICE {

notUsed								NULL,

rohc								SEQUENCE {

maxCID								INTEGER (1..16383)				DEFAULT 15,

profiles							SEQUENCE {

profile0x0001						BOOLEAN,

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0101						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

...,

[[	profiles_unidirect-r14			SEQUENCE {

profile0x0001-r14					UniDirectRohcConfig-r14,

profile0x0002-r14					UniDirectRohcConfig-r14,

profile0x0003-r14					UniDirectRohcConfig-r14,

profile0x0004-r14					UniDirectRohcConfig-r14,

profile0x0006-r14					UniDirectRohcConfig-r14,

profile0x0101-r14					UniDirectRohcConfig-r14,

profile0x0102-r14					UniDirectRohcConfig-r14,

profile0x0103-r14					UniDirectRohcConfig-r14,

profile0x0104-r14					UniDirectRohcConfig-r14

} 														OPTIONAL	-- Need ON

]]

}

},

...,

[[	rn-IntegrityProtection-r10		ENUMERATED {enabled}	OPTIONAL	-- Cond RN

]],

[[	pdcp-SN-Size-v1130				ENUMERATED {len15bits}	OPTIONAL	-- Cond Rlc-AM2

]],

[[	ul-DataSplitDRB-ViaSCG-r12		BOOLEAN		OPTIONAL,	-- Need ON

t-Reordering-r12				ENUMERATED {

ms0, ms20, ms40, ms60, ms80, ms100, ms120, ms140,

ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300,

ms500, ms750, spare14, spare13, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3,

spare2, spare1}					OPTIONAL	-- Cond SetupS

]],

[[	ul-DataSplitThreshold-r13		CHOICE {

release						NULL,

setup						ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800,

b25600, b51200, b102400, b204800, b409600, b819200,

spare1}

}																OPTIONAL,	-- Need ON

pdcp-SN-Size-v1310				ENUMERATED {len18bits}	OPTIONAL,	-- Cond Rlc-AM3

statusFeedback-r13				CHOICE {

release						NULL,

setup						SEQUENCE {

statusPDU-TypeForPolling-r13		ENUMERATED {type1, type2}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Type1-r13		ENUMERATED {

ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90,

ms100, ms150, ms200, ms300, ms500, ms1000, ms2000, ms5000,

ms10000, ms20000, ms50000}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Type2-r13		ENUMERATED {

ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90,

ms100, ms150, ms200, ms300, ms500, ms1000, ms2000, ms5000,

ms10000, ms20000, ms50000}		OPTIONAL,	-- Need ON

statusPDU-Periodicity-Offset-r13	ENUMERATED {

ms1, ms2, ms5, ms10, ms25, ms50, ms100, ms250, ms500,

ms2500, ms5000, ms25000}	OPTIONAL	-- Need ON

}

}																OPTIONAL	-- Need ON

]]

}

UniDirectRohcConfig-r14 ::=	ENUMERATED { no, ul-only, dl-only }

-- ASN1STOP

36.306

Capability changes have to be added in chapter 4.3.1.

36.323

### 5.5.3	Protocol parameters

RFC 4995 has configuration parameters that are mandatory and that must be configured by upper layers between compressor and decompressor peers [7]; these parameters define the ROHC channel. The ROHC channel is a unidirectional channel, i.e. there is one channel for the downlink, and one for the uplink. There is thus one set of parameters for each channel, and the same values shall be used for both channels belonging to the same PDCP entity if bidirectional ROHC operation is configured for that entity. In case unidirectional ROHC operation is configured the configuration is only applicable to the configured ROHC channel.

Observation 5: There is a consensus to introduce a new UE capability and PDCP configuration for Uplink-only RoHC operations

# 3    Conclusion

Observation 1: 8 companies provided feedback in the email discussion

Observation 2: There is consensus to support uplink only ROHC based in following procedure:

- UE to report uplink-only ROHC capability indication.

- eNB to reconfigure the DRB for uplink-only ROHC

- uplink ROHC channel feedback is supported which imply that the legacy behavior is preserved. i.e ) There are compressor and decompressor even for DL, and eNB will be responsible of the mapping of downlink packet to uncompressed session.

Observation 3: One company expressed interest for the UE to provide the maximum data rate

Observation 3: One company expressed interest on investigation the benefits of Downlink-only RoHC operations

Observation 4: Two companies expresse the need to introduce some clarification on network behaviour in the specification.

Observation 5: There is a consensus to introduce a new UE capability and PDCP configuration for Uplink-only RoHC operations

Recommendation 1: It is recommended to enable uplink-only RoHC for Release 14 LTE based on the procedure described in “observation 2”. (including necessary descriptive text).

Recommendation 2: It is recommended to extend UE capability and PDCP configuration to signal uplink-only RoHC operations.

# 4    References

[1]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[2]	R2-1701761, “RoHC Symmetric DL/UL Parameters Limitation in LTE”, Apple, LG

[3]			TS 36.323, “Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification”.