3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702656

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.4

Source:	Ericsson

Title:	Indicating Message 3 size in NR Random Access

Document for:	Discussion, Decision

# Introduction

This contribution addresses schemes to indicate the message 3 size via message 1 transmission in NR. It discusses aspects which are needed for providing a procedure description that can be used as a basis for the NR MAC specification.

In RAN2-96, there are the following agreements:

  - The design of RA procedure in NR needs to support flexible Msg3 size (as already supported in LTE).

  - FFS whether the eNB can be provided with more information (compared to LTE) from the UE on the Msg 3 size to provide.

# Discussion

To facilitate the discussion of indicating of message 3 size using message 1, we define the following terms:

-	PRACH radio resources: time-frequency resources for the PRACH transmission

-	PRACH preamble resources: PRACH preamble sequence set

-	PRACH resource = PRACH radio resources + PRACH preamble resources

For PRACH resource partition, we mean that either or both the PRACH radio resources and PRACH preamble resources are partitioned. Since different PRACH resource partitioning means different complexity, RA performance and PRACH resource consumption, care must be taken when configuring and partitioning these for various purposes. The procedure to indicate message 3 size via PRACH transmission using PRACH resource partitioning must be thoroughly studied to ensure it operates accurately and to identify the preferred PRACH resource partition schemes.

## PRACH resource partitioning to indicate Message 3 size

For indicating message 3 size via PRACH transmission, the rationale is that if the gNB knows the size of the small data, it can assign a grant which is sufficient to carry the available data with message 3. If the size is not known, it may happen that only part of the data can be multiplexed with message 3, and the remaining will need to wait for a new grant, which means not only additional overhead but also increased data transmission delay, or even triggering RRC_inactive to RRC_activate state transfer for a UE, which means signalling overhead increase. It may also happen that the gNB provides a too large grant, which will waste radio resources.

In LTE, CCCH SDU (RRCConnectionRequest or RRCConnectionResumeRequest) is not segmented. This means that the network needs to provide at least 56 bits grant for Msg3. On the other hand, the initial RRC message cannot be extended very easily as the network is not expected to provide very large grant. Also large initial message size can limit the coverage.

### Preamble partition

In LTE, two preamble groups, Random Access Preamble group A and Random Access Preamble group B, have been defined which can indicate if message 3 is above or below a configurable threshold. Random access preamble partition has the advantage of low complexity and low overhead for indicating message 3 size. Hence this should be still used in NR. More discussion is needed to decide the number of preamble groups and if a higher granularity than LTE is needed. However, since the number of PRACH preambles are limited, defining more than 2 (or 3) Random Access Preamble groups can result in too small preamble group size and considerably increase the collision probability.

Defining Random Access Preamble groups for flexible message 3 size indication should be supported for efficient small data transmission in UL.

The number of Random Access Preamble groups and corresponding message 3 size threshods should be configurable by the network.

To configure Random Access Preamble groups similar parameters as in LTE could be used. For example sizeOfRA-PreamblesGroupX could be used to derive which preambles belong to which Random Access Preamble groups. For example, with 3 Preamble groups we could define:

-	Random Access Preamble group A has preambles 0 to sizeOfRA-PreamblesGroupA – 1,

-	Random Access Preamble group B has preambles sizeOfRA-PreamblesGroupA to sizeOfRA-PreamblesGroupA + sizeOfRA-PreamblesGroupB – 1.

-	Random Access Preamble group C has preamble sizeOfRA-PreamblesGroupA + sizeOfRA-PreamblesGroupB to sizeOfRA-PreamblesGroupA + sizeOfRA-PreamblesGroupB + sizeOfRA-PreamblesGroupC – 1.

Also other encodings could be possible.

If n Random Access Preamble groups are configured, each Random Access Preamble group X (0 ≤ X ≤ n-1) should have a parameter sizeOfRA-PreamblesGroupX.

In order to map a message size to preamble group, each Random Access Preamble group except the last must have a parameter Message3-ThresholdX, which indicates the maximum size of the Message 3 size of Random Access Preamble group X. For example, for three Random Access Preamble groups we need two thresholds Message3-ThresholdA and Message3-ThresholdB, where if message 3 size is less than or equal to Message3-ThresholdA, a preamble from Random Access Preamble group A is used. If message 3 size is larger than Message3-ThresholdA but less than or equal to Message3-ThresholdB, a preamble from Random Access Preamble group B is used. In all other cases a preamble from Random Access Preamble group C is used.

If n Random Access Preamble groups are configured, there are n – 1 Message 3 size thresholds for Random Access Preamble group selection.

Since both the Random Access Group definition and the corresponding message 3 size thresholds can be common for all users in a cell and there seems no benefit to have user-specific configurations. It is preferred to convey these parameters via System Information.

Both sizeOfRA-PreamblesGroupX and Message3-ThresholdX are conveyed via System Information.

Since the PRACH preambles are a limited, use of fine grained partitions will further fragment the PRACH resources, which will lower the PRACH resource sharing efficiency and increase the preamble collision probability. Furthermore, more indicators must be included in either minimum SI or other SI, which means overhead increase for broadcast transmission. In considering this, the pros and cons of other schemes which require the further PRACH resource partition for other purposes shall be carefully evaluated and should only be used if a real need is foreseen. Our view is that there is currently no need for additional PRACH resource partition for other purposes.

More PRACH preamble partitions for other reasons than indication of message 3 size is FFS and should be low prioritized in first release of NR.

### PRACH partition

In addition to preamble groups, use of partitioning PRACH radio resources to indicate Message 3 size could also be feasible. In LTE, the prach-ConfigIndex is used to indicate where in time and frequency the PRACH transmissions may take place. Using different prach-ConfigIndex configurations, the UE may transmit PRACH preamble in more than one subframs in a radio frame with odd, even or any system frame number (SFN). For instance, when prach-ConfigIndex is configured to 6, a UE can choose to transmit PRACH in Subframe 1 or 6 in any radio frame [1]. Irrespective on how the final PRACH design is done in RAN1, one could use PRACH transmission occurance in time (subframe number and or system frame number) to indicate the message 3 size. As an example, one can divide the subframe numbers for PRACH transmissions into groups and each group indicates one message 3 size range. As another example, one can use odd or even SFN when the PRACH is transmitted to indicate message 3 size. By such grouping schemes, the message 3 size can be indicated without clear increase of the receiver complexity.

NR should support a configuration using the System Frame Number and/or subframe number for PRACH transmission to signal Message 3 size.

One way would be to combine this with Random Access Preamble groups is to use two Random Access Preamble groups, where only one of the groups, e.g. Random Access Preamble group B, is used to indicate Message 3 size. If a preamble from Random Access Preamble group A is used, no size indication is used. Within Random Access Preamble group B, one example is to use different subframe number and or SFN of PRACH transmission to indicate different Message 3 size. More PRACH transmission groups can be realized without increase of the receiver complexity for PRACH monitoring and collision probability. This method will result in optimal grant allocation for UL small data transmission. Hence we propose:

NR should support configuration of using the SFN and/or subframe number for PRACH transmission in combination with Random Access Preamble groups to signal Message 3 size.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Defining Random Access Preamble groups for flexible message 3 size indication should be supported for efficient small data transmission in UL.

Proposal 2	The number of Random Access Preamble groups and corresponding message 3 size threshods should be configurable by the network.

Proposal 3	If n Random Access Preamble groups are configured, each Random Access Preamble group X (0 ≤ X ≤ n-1) should have a parameter sizeOfRA-PreamblesGroupX.

Proposal 4	If n Random Access Preamble groups are configured, there are n – 1 Message 3 size thresholds for Random Access Preamble group selection.

Proposal 5	Both sizeOfRA-PreamblesGroupX and Message3-ThresholdX are conveyed via System Information.

Proposal 6	More PRACH preamble partitions for other reasons than indication of message 3 size is FFS and should be low prioritized in first release of NR.

Proposal 7	NR should support a configuration using the System Frame Number and/or subframe number for PRACH transmission to signal Message 3 size.

Proposal 8	NR should support configuration of using the SFN and/or subframe number for PRACH transmission in combination with Random Access Preamble groups to signal Message 3 size.

# References

3GPP TS 36.211-a10, Table 5.7.1-2