3GPP TSG-RAN WG2 Meeting #97bis	Tdoc R2-1702662

Spokane, USA, 3th –7th April, 2017

Agenda Item:	10.2.3.1

Source:	Ericsson

Title:	Impact of mixed numerologies on UEs in idle mode

Document for:	Discussion

# Introduction

During RAN1#88, signals needed for synchronization, initial access and paging were discussed [1]. In general, the approach in RAN1 has been to try to specify a default subcarrier spacing for each carrier frequency in order to avoid excessive UE complexity, as demonstrated by the agreement listed below:

- RAN1 considers following parameter sets with associated default subcarrier spacing and possible maximum transmission bandwidth for NR-SS design

  - Parameter set #W associated with 15 kHz subcarrier spacing and NR-SS transmission bandwidth no larger than 5 MHz

  - Parameter set #X associated with 30 kHz subcarrier spacing and NR-SS transmission bandwidth no larger than 10 MHz

  - Parameter set #Y associated with 120 kHz subcarrier spacing and NR-SS transmission bandwidth no larger than 40 MHz

  - Parameter set #Z associated with 240 kHz subcarrier spacing and NR-SS transmission bandwidth no larger than 80 MHz

  - Note that association between a frequency band and single set of default parameters (SCS, sequence length, NR-SS transmission bandwidth) will be defined in RAN4

  - Note that each subcarrier spacing is associated with single sequence length and transmission bandwidth

  - Note that additional  parameter set or further down selection of  parameter set is not precluded

  - This agreement does not preclude any subcarrier spacing for data channel

In this contribution, we analyze the complications caused by supporting mixed numerologies on a carrier from RAN2 point of view, with special focus on random access and paging. We also propose to simplify the system design to avoid these issues.

# Discussion

Recently, the concept of a default subcarrier spacing per carrier frequency has been discussed in RAN1. Currently RAN1 is evaluating parameter sets associated with default subcarrier spacing for 15, 30, 120 and 240 kHz subcarrier spacing. This approach clearly reduces the UE complexity, as the UE does not have to blindly detect the subcarrier spacing(s) used on a carrier.

As analysed in [2], there are several RAN2 related consequences of supporting mixed numerologies on a carrier. Assuming that not all terminals support all numerologies (e.g. due to need to support low complexity terminals for machine type communication and other use cases), the network needs to ensure the different terminals are operated with a supported numerology. For connected mode UEs, this can be handled relatively easily. However, for idle mode UEs, the network is not aware of the UE capabilities, and in practice would need to duplicate all information (such as system information broadcast, random access and paging) using all numerologies supported by a carrier. In our opinion this duplication leads to excessive amount of system resources being used to support mixed numerologies. We also note that this in contrast to the default subcarrier design approach taken in RAN1.

Supporting mixed numerologies for idle mode UEs leads to duplication of system information, random access and paging resources

Given that there is likely to be a default subcarrier spacing for each carrier frequency, we would like to explore a similar approach from RAN2 point of view. In practice, this would mean that for each carrier frequency, the UE will need to be able to at least

1)	Read the minimum system information using a default configuration.

2)	Receive paging using a default configuration.

In addition, the network does not know the UE full set of capabilities during the initial access (until the reception of RRC Connection Setup Complete message). However, it’s possible to use special mechanisms to provide partial UE capabilities (such as supported numerologies) during initial access. For example, the NW may partition the random access preamble space to different numerologies. This allows the NW to know one supported numerology already in time for random access response. Alternatively, the UE may include information on preferred numerologies in RRC Connection Request, allowing the NW to use a preferred numerology for RRC Connection Setup. However, these mechanisms create significant extra complexity for the system, and would require significant gains for a relevant use case to justify them. Currently, we would prefer to continue using a default configuration also during initial access:

3)	Complete the random access procedure (transmission of random access preamble, reception of random access response, transmission of message 3 and reception of message 4) using a default configuration.

It is expected that these default configurations are hard-coded in the specifications, and are specified per carrier frequency, similar to RAN1 default subcarrier spacing.

Note that the assumption to use default configuration initially does not prevent a later introduction of additional mechanisms to support e.g. narrow-band operation for new use cases. This can be done by introducing additional signals on existing carrier similar to e.g. NB-IOT introduction on LTE carriers (though one can hope that the forward compatible lean design of NR will make introducing new signals easier than for LTE).

After completion of the random access procedure, the network can configure the UE to operate with any subcarrier spacing supported by the UE. Since the UE is likely to be able to receive the SS block using default subcarrier spacing, this does not appear to bring a significant increase in the UE complexity, especially since the UE is not required to transmit or receive simultaneously. Based on this, we propose

Mixed numerologies on a single carrier are only supported in connected mode. Idle mode procedures (system information broadcast, paging and random access) use default configuration per carrier frequency.

# Conclusion

Based on the analysis in section 2, we propose the following:

Proposal 1	Mixed numerologies on a single carrier are only supported in connected mode. Idle mode procedures (system information broadcast, paging and random access) use default configuration per carrier frequency.

# References

Report of RAN1#88;

R2-1700097, “RRC Support of Multiple Numerologies”, Huawei, HiSilicon