3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702739

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.2.2

Source:	Ericsson

Title:	RLC PDU design

Document for:	Discussion, Decision

# Introduction

In previous RAN2 meetings, segmentation and concatenation functionality had been discussed and lead to the following agreements and working assumptions:

RAN2#95bis

Agreements

1: In NR, the segmentation function is only placed in the RLC layer as in LTE.

Agreement

=>	 SO-based segmentation can be considered for both segmentation and resegmentation as a baseline in NR user plane to support high data rate. (Does not imply anything about location of concatenation). At least overhead for the low data rate case should be analysed further.

RAN2#96-AH

Agreements

1 MAC sub-headers are interleaved with MAC SDUs.

Agreements

1	As in LTE the UEs shall not send padding if there is data available and the remaining TB size is greater than X bytes (actual number can be discussed later when header sizes are known. In LTE X = 7 bytes)

2	MAC CEs are not placed in the middle of the MAC PDU but at the beginning or at the end (placement can be decided later)

3	Working assumption on no RLC concatenation taken at RAN2#96 is confirmed (i.e. concatenation of RLC PDUs is performed in MAC)

RAN2#96-AH

Agreements

1: PDCP and RLC layers have separate PDCP SN and RLC SN.

# Discussion

In RAN2#AdHoc several views on how the RLC headers should be updated to allow, as much as possible, pre-processing of user plane packets by layer-2. To reduce the overhead caused by the introduction of a Segmentation Offset (SO) when segmentation of a RLC SDU is required, two alternatives have been proposed. In [1] and [2] it is proposed to use a two-bit approach like the LTE RLC FI field; meanwhile in [3] and [4] it is advocated for a one bit indication.

## RLC segmentation and header length

Figure 1 illustrates how the MAC PDU is constructed, which is also further discussed below. It is highlighted in green, which header and payload can be pre-processed and stored in memory. It is shown in red, which headers must be created or modified on the fly i.e. are dependent on TB size.

Figure 1: MAC PDU construction.

Since RLC does no longer support concatenation of RLC SDUs, the variable length train of L-fields in RLC is not needed anymore. The MAC L field points to the end of the RLC PDU and thereby to the beginning of the next MAC sub-header.

The RLC header must indicate whether a complete PDU or just a PDU Segment follows. If it is a segment, the Segment Offset (SO) field should indicate the position of the first payload octet in the SDU. When the RLC transmitter pre-generates RLC headers, it will typically assume that all RLC PDUs can be sent without segmentation and will hence not include the SO field. When the RLC transmitter is unable to send the entire RLC PDU due to limited space in the MAC PDU, it must at least indicate that the last payload octet of the RLC PDU is not the last octet of the RLC SDU. But since the first octet of the RLC PDU payload contains the first octet of the RLC SDU, the segment offset is 0 and should preferably be omitted. This avoids having to update the RLC header size for the first segment of an PDU.

The header of the RLC PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Any subsequent segment (not starting with the first octet of the RLC SDU) must carry the additional SO field. To inform the RLC receiver whether the SO field follows the RLC “base header”, an additional bit is needed in the AMD PDU header:

The header of the RLC PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

By using two bits to indicate the presence of the SO, it is possible to further support Layer-2 pre-processing and therefore pre-generation of packets requiring segmentation. This is possible because when the transport block size is determined by the transmitter, and if segmentation is unavoidable, it is not required to interleave any additional header field in an already pre-constructed PDU in memory, but only to update the segmentation flags for the first segment. Only the subsequent segments must be re-generated to include the segmentation offset and only in case of middle segments, the segmentation flags might require update (see Figure 2). Besides the reduced processing effort this approach also avoids that the size of a header inside a TB changes during the generation of that TB. In other words, the size of the header does not depend on whether the PDU is being segmented when included in the TB.

With Proposal 2 and Proposal 3 the size of all RLC PDU headers is known prior to the creation of the transport block.

Figure 2 Two bits aproach segmentation indication working with pre-generation (example of transmission requiring three segments)

In contrast, a one-bit solution requires interleaving the SO field even for the first transport block, i.e., within the actual multiplexing operation. This would also re-introduce the problem that the size of a header depends on how much of that PDU can be included in this transport block. And this may lead to conditions where the initial intent to include another PDU (segment) must be reverted due to the sudden increase of the header. Similarly, as in the previous solution, there is need for fields update for the subsequent TB (unless the last-one), as shown by Figure 3.

Figure 3 One bit Segmentation indication impact in pre-generation (example of transmission requiring three segments)

This is a clear advantage of the two-bit indication and would contribute to a faster processing of packet transmissions.

## RLC Segmentation baseline for RLC AM and RLC UM

In RAN2 email discussion [6], some companies anticipated that segmentation should be optional or configured dynamically for RLC UM and RLC AM mode. We think that such behaviour would not only add complexity to the UE and Network implementation but would also result waste of radio resources when an UL grant is too small for RLC PDUs to fit into transport block and the transport block must be filled with padding. Rather than filling the TB with padding, UE should fill the TB(s) with data and not send padding if there is more data to send. This option should not increase the UE processing significantly as the removal of the concatenation was removed from the RLC. As for the SO based segmentation, we think that the processing overhead caused by the SO based segmentation is relatively small. Therefore, segmentation should be enabled as a baseline for RLC AM and RLC UM mode.

Email discussion [6] suggests that there could be a need for option to disable the segmentation in URLLC or high in data rate scenarios. However, in high data rate scenarios, it may be beneficial to use large PDU size. When large PDU size is used and grant is not enough to serve the whole PDU, disabling the segmentation may led to situation where over the air overhead caused by the padding is significant. In URLLC case the added complexity to disable the segmentation may not be justified because URLLC may already use SPS based scheduling and small packet size. By adjusting the packet and grant size, URLLC traffic may be configured so that the RLC segmentation becomes rare case. To summarize, we do not see benefit in disabling the segmentation for URLLC or High data rate scenarios.

Segmentation is always enabled for RLC AM and RLC UM.

## RLC Sequence Number length

The RAN2 #97-AH agreed that “PDCP and RLC layers have separate PDCP SN and RLC SN”. Additionally, working assumption of concatenation removal was confirmed. This results that only one PDCP PDU may exist per RLC SDU. Because of this one-to-one relationship, PDCP and RLC SN ranges should be equal.

RLC uses the same Sequence Number ranges as PDCP.

# Conclusion

Based in the previous section discussion we made the following proposals

Proposal 1	The header of the RLC PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Proposal 2	The header of the RLC PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

Proposal 3	If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

Proposal 4	Segmentation is always enabled for RLC AM and RLC UM.

Proposal 5	RLC uses the same Sequence Number ranges as PDCP.

# References

R2-1700253. Further details on NR segmentation. Nokia, Alcatel-Lucent Shanghai Bell

R2-1700429	. Further design aspects of NR User Plane stack. Ericsson

R2-1700281. Further considerations on SO based segmentation. LG Electronics Inc.

R2-1700561. Further details of RLC protocol without concatenation. Qualcomm Incorporated

R2-1701987. PDCP/RLC Sequence Number Size for NR. Samsung

R2-1702646. Email discussion report on Segmentation. NTT DOCOMO, INC.

R2-1702741 PDCP SN. Ericsson