3GPP TSG-RAN WG2 #97bis	R2-1702741

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	PDCP SN length

Document for:	Discussion, Decision

# Introduction

This contribution looks at the PDCP sequence number length.

# Discussion

## Data bearers

In Rel13, 3GPP introduced Further enhancement of Carrier Aggregation (FeCA), which is also referred to as massive carrier aggregation or carrier aggregation beyond 5 carriers. For FeCA, up to 32 component carriers (CC) can be aggregated, which means up to 640 MHz bandwidth and up to 25 Gbps peak rate in air interface. The PDCP SN space was extended from 15 bits to 18 bits [1] to meet the high capacity. It has been assumed that the PDCP RTT is 50 ms, the supportable data rate by PDCP increases from 4Gbps to 31 Gbps.

PDCP data rate= (1500×8×215-10.050×10-9) Gbps ≈4 Gbps  or  (1500×8×218-10.050×10-9) Gbps ≈31 Gbps

The same methodology can be used to evaluate the PDCP SN length needed for NR. To determine the PDCP SN length, the following parameters need to be set: PDCP RTT, maximum bit rate, and PDCP PDU size.

The PDCP RTT is compound of three parts: the delay of HARQ retransmissions, MAC/RLC delay, and the transport network delay. In the examples shown in the tables below, the PDCP RTT are 30 ms and 50 ms

The PDCP PDU sizes may vary depending on the application. IP typically delivers 1500 bytes, while PDCP can support up to 8190 bytes (PDCP PDU), similar size to the jumbo frame size.

Table 1	PDCP bit rates (Gbps) for different PDCP PDU sizes and SN lengths for RTT = 30 ms

|  |  | PDCP SN length (bits) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 7 | 12 | 15 | 18 |
| PDCP PDU (bytes) | 512 | 0,008 | 0,28 | 2,24 | 17,9 |
|  | 1502 | 0,025 | 0,82 | 6,56 | 52,5 |
|  | 8190 | 0,139 | 4,47 | 35,78 | 286,3 |

Table 2	PDCP bit rates (Gbps) for different PDCP PDU sizes and SN lengths for RTT = 50 ms

|  |  | PDCP SN length (bits) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 7 | 12 | 15 | 18 |
| PDCP PDU (bytes) | 512 | 0,005 | 0,17 | 1,34 | 10,74 |
|  | 1502 | 0,015 | 0,49 | 3,94 | 31,50 |
|  | 8190 | 0,084 | 2,68 | 21,47 | 171,76 |

NR is expected to achieve bit rates of in the order of some Gbps and in deployments with large amounts of bandwidth, it could arrive up several dozens of Gbps (~20 Gbps) for a single UE. From the table above, it can be concluded that 15 bits is sufficient to cope with the initial expected bit rates. However, 18 bits seem to be a more future proof value which may be able to cope with extreme cases and allows some flexibility in the back-haul design.

PDCP SN length should be up to 18 bits (for data bearer)

While 18 bits is a SN length needed to achieve the maximum bit rates, lower SN lengths may still be needed. There may be situations in which the signaling overhead needs to be limited such as, for example, in coverage limited situations or for low data rate services. In these situations, the maximum bit rates may be considerably reduced and, therefore, a short SN space is more suitable to limit overhead.

Another aspect to consider is the LTE-NR interworking and integration (e.g. CA, DC, and handovers). The MCG (having the PDCP layer) may be in either of LTE or NR depending on the potential bit rates, coverages, expected robustness, etc. However, in a mobile system, handovers may be common. These should be performed as seamless as possible. Having the same set of PDCP SN length values in LTE and NR may avoid having to reconfigure the PDCP layer to change the SN length when doing handover between the two RATs.

LTE supports a set of SN lengths for data bearers. When RLC UM is configured, PDCP may be configured with a SN length equal to 7 or 12.  When RLC AM is configured, PDCP may be configured with a SN length equal to 12, 15, 18. The LTE default length value (if the SN length value is not included in the setup message) is 12 bits. Exceptionally, NB-IoT can only use 7-bit PDCP SN length. Yet, there is one more SN length value for sidelink, 16 bits.

We think that having such a diverse number of SN length does not provide any major advantage and, therefore, we propose to reduce this set to a minimum set.

For the general case, the PDCP SN length is limited to two values: 12 and 18 bits.

Discuss if 7-bit PDCP SN length should already be introduced in Release 15 to be used for specific devices e.g. eMTC, NB-IoT.

In LTE, high PDCP SN length values were only specified for those data bearers mapped onto RLC AM. The reason argued in 3GPP by some companies to exclude the support PDCP SN of 15 or 18 bits for RLC UM is that most of the services which transmit little data are mapped over RLC UM e.g. voice.

In 5G, many more services are expected to be supported. However, it is not possible to predict whether these services may require high bit rates or whether they may need to run on e.g. AM or UM mode.

Following the same path as in LTE (i.e. increasing the SN space for only AM) might hamper the introduction of future services to be supported by NR. If the complexity of supporting these values for both AM and UM does not increase compared to the complexity of implementing it only for AM, it would be then beneficial to support them for both cases.

PDCP SN length set (12, and 18 bits) should be supported for data bearers mapped on RLC AM and UM.

## Signaling radio bearers

Signaling radio bearers do not need a large SN space as the number of RRC messages which may be sent to the UE are limited. Thus, the same SN length as in LTE, 5 bits, would be sufficient. On the other hand, minimizing the set of SN lengths that the network must handle has its advantages. Thus, aligning the SN space to 12 bits could be a good approach. The drawback is the increased, 1 byte, overhead, but we do not see this is a big concern.

For NR signaling bearers, the PDCP SN length should be 12 bits

# Conclusion

Here there are two lists, one listing observations, and one listing proposals. Listing observations are not always needed, but listing proposals are always useful. To update a list, place the cursor inside the list and press F9.

Based on the discussion in section 2 we propose the following:

Proposal 1	PDCP SN length should be up to 18 bits (for data bearer)

Proposal 2	For the general case, the PDCP SN length is limited to two values: 12 and 18 bits.

Proposal 3	Discuss if 7-bit PDCP SN length should already be introduced in Release 15 to be used for specific devices e.g. eMTC, NB-IoT.

Proposal 4	PDCP SN length set (12, and 18 bits) should be supported for data bearers mapped on RLC AM and UM.

Proposal 5	For NR signaling bearers, the PDCP SN length should be 12 bits