3GPP TSG-RAN WG2 #97bis	R2-1702742

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	Introduction of Jumbo frames in NR

Document for:	Discussion, Decision

# Introduction

This contribution looks at the PDCP SDU size.

# Discussion

Ethernet standard frames may carry up to a payload of 1500 bytes. However, larger sizes are starting to be adopted. “jumbo frames” and “super jumbo frames” are the two examples of the frames that are/may be commonly adopted. “Jumbo frames” can carry up to 9000 bytes while “super jumbo frames” can carry up to ~64000 bytes.

There are few aspects which need to be considered for defining support for large frame payloads in NR:

- The support of super jumbo frames should not be mandatory.
A node should not enforce or configure another device in the network with a larger PDCP SDU size than the PDCP SDU size which was indicated to be supported.

- loss rate and performance: It can be argued that configuring super jumbo frames may reduce upper layer performance when there are losses/problems in lower layers. This type of concern applies similarly to other type of features which require a link quality e.g. Higher Order Modulation, or MIMO. These features require a good link quality and are typically configured in such occasions. Therefore, super jumbo frames are likely to be configured in good radio conditions. In these scenarios, there are also obvious benefits to be able to transmit large PDCP SDUs.

- Another aspect to consider is security. There used to be an upper bound of 8188 octets in previous securities (ZUC) versions, but recently this limitation has been amended [1, 2]. Encryption algorithms should thus be able to handle the PDCP SDU size. Even though SA3 has not yet decided details and which algorithms will be used for NR, it is likely that these will be able to handle jumbo frames.

These considerations lead to think that the PDCP SDU size could be larger and it would not cause any issues in neither the network nor the UE.

Currently, PDCP can only carry PDCP SDUs up to 8188 bytes. It would, then, be natural to increase this size up to ~64KB too to cope with the future frame sizes.

Current specifications have a PDCP PDU header length of at most 3 bytes for data bearers (not considering feedback formats). This means, therefore, that the PDCP SDU size should be (PDCP PDU size Bytes – 3 Byes). If a PDCP PDU size of 65536 bytes is agreed (2^16), then the PDCP SDU should be 65533 bytes.

Introduce in the PDCP specifications the support of PDCP SDU sizes of up to 65533 bytes (considering 3 bytes of PDCP headers)

The UE support of super jumbo frames is optional

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	PDCP specifications should support PDCP SDU sizes of up to 65533 bytes (considering 3 bytes of PDCP headers)

Proposal 2	The support of super jumbo frames is optional

# References

S3-111093, LS on EEA3 output length, SA3, 3GPP TSG SA WG3 Security, SA3#65, 7-11 November 2011

S3-120023, EEA3 output length, ETSI SAGE, ETSI SAGE, 30 December 2011