3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702745

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	SR and BSR signalling content in NR

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions. We prepare two contributions focusing on different aspects. This contribution discusses what actual information should be carried by SR and BSR. Another contribution addresses when to trigger SR and BSR [1].

# Discussion

We first briefly discuss LTE SR/BSR as background information, to motivate some necessary enhancements for SR and BSR focusing on the signalling content in NR.

## Role of SR and BSR

In LTE, scheduling requests (SRs) are used for requesting UL-SCH resources for new transmissions when the UE has no valid grant. If SRs are not configured for the UE, the UE initiates a Random Access procedure in order to get scheduled in UL. In LTE SRs consists of only one bit of information and indicates only that the UE needs an UL grant. This means that upon the reception of SR, the gNB knows neither which logical channel (associated with certain QCI) has data available for transmission, or the amount of data available for transmission at UE. Furthermore, in view of the latest RAN2 agreements, it should be noted that the numerology/TTI duration should be conveyed in the grant. This implies that the gNB must also be made aware of what numerology/TTI duration is desired by the UE for the upcoming transmission. In short, in NR an accurate grant cannot be provided to the UE only based on the one-bit information of the LTE type of SR.

LTE scheduling request saves physical layer resources but does not provide sufficient information for efficient grant allocation in NR.

Buffer Status Reports (BSRs) on the other hand carry more detailed information compared to SR. A BSR indicates buffer size for each LCG. However, the BSR requires a grant for transmission so it may take longer time until the gNB receives it since it may need to be preceded by an SR. The interaction between SR, BSR and grant is exemplified in Figure 1.

Figure 1: Scheduling procedure for dynamic scheduling in LTE

LTE BSRs provide more detailed information than an SR, but it brings additional delay due to the need for a grant.

We believe the framework with SR/BSR from LTE in general works well but there may be room for improvements considering the above observations.

LTE SR/BSR framework can be reused in NR as a baseline, however further improvement is necessary for NR.

## Aspects for improvements in NR

The scheduling entity should be aware of the information including

- An indication that UE has data to transmit

- Buffer size for each logical channel

- Priority indication for each logical channel

- Indication of a set of the associated numerologies/TTI durations for each logical channel

To reduce the latency due to dynamic scheduling, and improve the resource utilization efficiency, the above information must be provided on time and accurately by UE via SR and BSR.

### SR aspects

#### Extensional bits for SR

A possible improvement is to extend the SR to not only indicate whether data is available not. With more bits used in SR it would be possible for a UE to provide more detailed information such as the type/priority of the logical channel which has data available.

By knowing the type/priority of the logical channel, the gNB can provide grants for the traffic that should be scheduled. This enables a more correct priority handling. This is beneficial for RRC signalling or URLLC data.

More efficient priority handling is achievable by extending additional bits for SR

How many bits that SR should be extended with, is a question of how to achieve a good trade-off between the increased L1 control channel issues (overhead, design complexity etc) and the possible achieved gain in terms of UP latency reduction.  The SR without HARQ acknowledgement could probably be relatively easier generalized to support 2 bits with the modulation and coding scheme QPSK. The extension of more than 2 bits would require more changes than that in LTE. However, the final decision should be made by RAN1. It is suggested to send LS to RAN1 to check the feasibility that SR is extend to support 2 bits.

Extend SR to support 2 bits pending on RAN1’s confirmation

Send LS to RAN1 to check the feasibility that SR can be extended to 2 bits

It is natural for SR to convey the priority information with more bits. Assuming SR can be extended to 2 bits, an example of the SR signaling content is shown in the below table.

| Priority level | bit 1 | bit 2 |
| --- | --- | --- |
| Priority level 1 | 1 | 1 |
| Priority level 2 | 1 | 0 |
| Priority level 3 | 0 | 1 |
| Priority level 4 | 0 | 0 |

The network configures the mapping between priority levels and logical channels. It can be cell-specific or UE-specific. UE may be configured with more than 4 LCHs at the same time. In this case, there may be more than 1 LCHs mapped to the same priority level.

More than 1 logical channels may be mapped to the same priority level

Support extended SR to not only indicate that a UE has data to transmit, but also indicate the priority information of associated logical channels

On the other hand, if there is a need to simultaneously send HARQ acknowledgement and SR information, or in situations where additional information is not needed, then an SR with more bits (e.g., 2 bits) may not always be useful. In these cases, a 1-bit SR is preferred. This means that UEs may need to support 1-bit SR in addition. If a NR cell support both 1-bit SR and 2-bits SR, the gNB shall be able to configure a UE to use either 1-bit SR or 2-bits SR.

The network can configure a UE to use either 1-bit SR or 2-bits SR

### Enhancement to BSR

#### BSR content

Regarding data structure of BSR, there are two types of BSRs in LTE, including Short BSR and Long BSR. With short BSR, UE can inform the amount of data in UL buffer only for one certain LCG (Logical Channel Group). While with long BSR, UE can inform the UL buffer information for all LCGs (up to 4 LCGs supported in LTE). There are no LCG ID fields in Long BSR but multiple 'Buffer Size' field, each of which represent one LCG.

As we proposed in [1], it is beneficial to report BSR per LCH, instead of LCG. It gives better granularity hence improve the accuracy for BSR. Another benefit is to simplify the management complexity for the network. Therefore, the data structure of BSR must be changed accordingly. It is unnecessary to keep both short BSR and long BSR. It is sufficient to define a unified BSR data structure covering scenarios where one or more than one LCHs that trigger BSR.

NR should support a unified BSR format consisting buffer sizes (BS) for one or more than one LCHs

Keeping the gNB informed of the status of many logical channels at the same time would mean considerable signaling overhead. A large grant may be also required to carry a BSR with a big size. Consequently, the gNB can configure a maximum number of LCHs that a UE can report in a single BSR.

NW configures the maximum number of LCHs that can be reported in a single BSR

A BSR consists of fields for each reported LCH, including an indicator for the presence or absence of a BS field, and a BS field (if the indicator conveys its presence)

Upon reception of BSR, the gNB must be made aware of what numerology/TTI duration that is preferred or wanted the data. Since we assume that a mapping of LCH to numerology/TTI duration will make it possible to infer which numerology/TTI duration to use given the LCH indicated in the BSR, no additional information than the field buffer size (BS) is needed in the BSR

In BSR, no additional information than buffer size (BS) is needed to indicate the wanted numerology/TTI duration for the associated logical channel

# Conclusion

In section 2 we made the following observations:

Observation 1	LTE scheduling request saves physical layer resources but does not provide sufficient information for efficient grant allocation in NR.

Observation 2	LTE BSRs provide more detailed information than an SR, but it brings additional delay due to the need for a grant.

Observation 3	LTE SR/BSR framework can be reused in NR as a baseline, however further improvement is necessary for NR.

Observation 4	More efficient priority handling is achievable by extending additional bits for SR

Observation 5	More than 1 logical channels may be mapped to the same priority level

Based on the discussion in Section 2 we propose the following:

Proposal 1	Extend SR to support 2 bits pending on RAN1’s confirmation

Proposal 2	Send LS to RAN1 to check the feasibility that SR can be extended to 2 bits

Proposal 3	Support extended SR to not only indicate that a UE has data to transmit, but also indicate the priority information of associated logical channels

Proposal 4	The network can configure a UE to use either 1-bit SR or 2-bits SR

Proposal 5	NR should support a unified BSR format consisting buffer sizes (BS) for one or more than one LCHs

Proposal 6	NW configures the maximum number of LCHs that can be reported in a single BSR

Proposal 7	A BSR consists of fields for each reported LCH, including an indicator for the presence or absence of a BS field, and a BS field (if the indicator conveys its presence)

Proposal 8	In BSR, no additional information than buffer size (BS) is needed to indicate the wanted numerology/TTI duration for the associated logical channel

# References

R2-1702746, SR and BSR triggering aspects, Ericsson