3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702747

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	Aspects of grant-free and contention based UL transmission in NR

Document for:	Discussion, Decision

# Introduction

Autonomous, grant-free and contention based UL transmission for NR has been discussed in RAN1. In those discussions, the following agreements have been made

Agreements:

- At least an UL transmission scheme without grant is supported for URLLC

  - Resource may or may not be shared among one or more users

    - FFS: resource configuration details

  - FFS other details of design

Furthermore, in RAN2#97, the following agreement was made with respect to UL scheduling

Agreements

1	NR supports an SPS scheme similar to LTE

2	NR supports skipping UL grant scheme similar to LTE

Based on the status of those RAN1/RAN2 discussions, this contribution discusses RAN2 aspects for grant-free and contention-based UL scheduling. It should be noted that in this paper, we do not differ between SPS and ”grant free” transmissions although this may need changes based on input from RAN1 etc.

# Discussion

## Background

Autonomous or grant-free transmissions refer to the case when no explicit grant needs to be signalled using downlink control channels for every UL transmission, i.e. opposite to LTE dynamic scheduling via PDCCH. Instead, UEs are assigned resources that can be used for UL transmissions whenever the UEs receive UL data in their buffers. These resources could either be assigned in a one-to-one mapping between UE and resource such as in the SPS mechanism in LTE or a many-to-one mapping. In case of many-to-one mapping between UE and resource, the transmissions will be subject to contention, implying that there is a risk of collision if more than one UE would use the same time and frequency resource.

## Aspects in grant-free and contention based UL transmission

### Resource Allocation

In LTE, an SPS like mechanism allows the transmission of data packets without scheduling requests, which is beneficial to reduce latency compared to ordinary dynamic scheduling. The UE is pre-configured by the eNB with an SPS-RNTI and a SPS periodicity. First the network activates SPS resources via PDCCH.  Then, during every SPS period, the UE can transmit using the configured grant. Compared to dynamic scheduling, pre-scheduling with SPS may result in less efficient resource utilization due to its semi-static nature, for example, PRB positions and MCS setting are rather semi-static, and thus has the issue of resource over-provisioning; especially when there are many UEs with SPS resources at the same time.

SPS like resource allocation, i.e., explicitly pre-allocated resources to UEs have a risk of resource over-provisioning

In LTE, contention based PUSCH transmission has been studied in SI LATRED [1]. According to this study, for contention based PUSCH transmission, multiple UEs may share the same PUSCH resource and this may allow more efficient PUSCH resource utilization compared to the existing pre-scheduling scheme. Collision will happen if two or more UEs that share the same PUSCH resource perform a transmission at the same time, and in this case the eNB may not be able to successfully decode all the PUSCH transmissions. Because of collision, potential retransmissions can result in increased latency for colliding UEs. It should be noted that it was proposed to include an indication of the DMRS cyclic shift as part of the SPS configuration, which would allow the network to identify the colliding UEs.

Allowing multiple UEs to share the same time and frequency resources may result in more efficient resource utilization but is sensitive to collisions

SPS like transmission is beneficial for latency reduction but require that a large amount of resources is set aside for each UE. This is doable when the system load is low, while contention based transmission will be a better alternative from resource efficiency when the load is high. Hence, both mechanism should be supported to complement each other.

NR support grant-free/SPS contention based UL transmissions in addition to SPS.

### Contention resolution

Since collisions will happen in the contention based case, efficient mechanisms for contention resolution also need to be studied. Since the gNB needs to know which UE has transmitted on the shared resource, some form of UE identity must be linked to the transmission. Several approaches are possible for the UE to indicate its identity. For connected state, one possibility would be for the UE to attach its C-RNTI in the transmission on the shared resource. However, this approach would lead to a high unnecessary overhead as the C-RNTI is two bytes. It would also have the drawback that if several UEs transmit simultaneously, it would be hard or impossible to detect more than one UE identity and even more difficult to also detect and decode the data. This approach will lose many of the benefits of contention based transmissions except in certain narrow load conditions.

From RAN2 point of view, increased resource efficiency for shared pre-allocated grant free resources are beneficial.

Another more efficient approach would be as discussed in RAN1, to overlay the UE identity on orthogonal DMRS patterns. The gNB would then deduce which UE has transmitted based on the received DMRS. Successful contention resolution relies on that both the UE identity and the data have been successfully decoded. In case only the UE identity is decoded, the UE should have the opportunity to retransmit the data on a contention free resource. This approach would result in substantial benefits compared to attaching a C-RNTI, since once the UE identity is known, lost data can be retransmitted on contention free resources.

UE identification based on DMRS should be studied by RAN1 to enable efficient usage of contention based UL transmissions.

# Conclusion

In section 2 we made the following observations:

Observation 1	SPS like resource allocation, i.e., explicitly pre-allocated resources to UEs have a risk of resource over-provisioning

Observation 2	Allowing multiple UEs to share the same time and frequency resources may result in more efficient resource utilization but is sensitive to collisions

Based on the discussion in Section 2 we propose the following:

Proposal 1	NR support grant-free/SPS contention based UL transmissions in addition to SPS.

Proposal 2	From RAN2 point of view, increased resource efficiency for shared pre-allocated grant free resources are beneficial.

Proposal 3	UE identification based on DMRS should be studied by RAN1 to enable efficient usage of contention based UL transmissions.

# References

TR 36.881, Study on latency reduction techniques for LTE, v0.6.0

R1-17xxxx, SPS for NR, RAN1#88bis, April 2017