3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702756

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.2.4.3 – NR – Stage-2 – Standalone - QoS

Source:	Ericsson

Title:	QoS stage-2 aspects

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss some stage-2 aspects that were not concluded during the study item phase and that should be concluded before entering the stage-3 design phase of RRC and the QoS protocol layer itself.

# Discussion

## Updating QoS-Flow to DRB filters

At RAN2-96 it was discussed how the network can change a mapping of UL flows to DRBs and RAN2 agreed that “The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly”.

The word “continuously” was put in quotation marks since companies wanted to study whether really each and every DL packet needs to be analysed.

We believe that this is the simplest way to allow the eNB to update the mapping by redirecting the packets of a DL flow onto a different DRB. For example, if the UE observes initially a downlink packet with Flow ID X on DRB 1, it creates an “Flow-to-DRB” filter that maps uplink packets with Flow ID X to DRB 1. But if the UE later observes a downlink packet with Flow ID X on DRB 2, it should change the filter for Flow X so that also the UL packets are mapped to DRB 2.We therefore suggest confirming and clarifying the previous agreement as follows:

The UE monitors the Flow ID (if present) in all downlink packets and updates the corresponding reflective “uplink QoS Flow ID to DRB mapping” accordingly.

For completeness, we also suggest to capture that the NW may change a configured uplink flow to DRB mapping by means of an RRCConnectionReconfiguration:

The RAN may configure, modify and release a configured “Uplink QoS Flow to DRB” mapping (a pair of “QoS Flow ID” and “DRB ID”) by an RRCConnectionReconfiguration.

Some companies observed in the last meeting that the re-mapping of a QoS Flow to a different DRB may cause out-of-sequence packet delivery. This may happen when initial packets of the flow ended up in a low priority DRB and subsequent packets are mapped to a high priority DRB due to an updated Flow-to-DRB mapping. We agree with this observation but believe that the network can avoid this when performing the re-mapping at an occasion where the queues are empty. It may not always be possible to ensure this for the uplink direction. But at least for initial re-mapping from a default DRB to another DRB, it is likely that higher layers are still in the initial handshaking phase (e.g. TCP SYN/SYN-ACK, TLS security setup, HTTP GET) and hence there will typically be very few packets in flight that could overtake each other.

When the NW re-maps a flow to a different DRB during the initial transaction phase of the flow, packet re-ordering is unlikely due to few packets being in flight.

When the NW re-maps a flow to a different DRB it can minimize the risk of re-ordering by postponing it to occasions when buffers are empty or at least small.

It was also mentioned that packet re-ordering upon Flow re-mapping could be avoided by means of an additional re-ordering function per QoS Flow (above PDCP). However, in accordance with the observations above, we don’t see a need for such (pretty complex) functionality on the UE side.

If RAN2 believes the risk of packet re-ordering upon QoS-Flow remapping (in uplink direction) is unacceptably large, we suggest to seek for a relatively simple solution such as the following: Upon detecting a remapping of a flow to a different DRB (reflectively or explicitly) the PDCP transmitter copies all the not-yet-RLC-ACKed PDCP PDUs to the target DRB’s PDCP entity. This may result in some duplicates but those don’t matter for higher layers. Since we anyway assume that there will usually be only few packets in flight during the initial phase of a file transfer, the inefficiency due to the (few) duplicates would be negligible for the initial reflective QoS remapping described above. Of course, the approach would also avoid re-ordering on IP level if the network re-maps a flow during handover.

Moving (instead of copying) the data to another DRB would avoid the overhead but would require re-processing already pre-processed the PDCP PDUs of the source DRB.

Additional UE functionality for avoiding possible out-of-order delivery when re-mapping a QoS-Flow to a different DRB (by explicit signalling or by update reflective QoS mapping) should not be introduced.

If Proposal 3 is not agreeable (i.e., if RAN2 believes that re-ordering due to QoS flow re-mapping shall be avoided), the PDCP transmitter shall copy all the not-yet-RLC-ACKed PDCP PDUs to the target DRB’s PDCP entity.

## Precedence order of reflective and configured mapping

RAN2-96 did not yet agree on “The precedence of the RRC configured mapping and reflective QoS”. There are basically three options:

1) An RRC configured mapping overrides any reflective mapping for that flow.

2) A newly derived reflective mapping overrides a mapping configured previously by RRC.

3) The UE applies always the most recent mapping, i.e., either provided by RRC or derived by reflective QoS

We think that the second option would introduce an undesirable dependency between the RRC configuration and the user plane. For example, the RRC configuration (AS-Config) would not represent the mapping that the UE applies. This would be undesirable in case of mobility since the UE would not behave as expected by the target node. It would also remove the possibility to override a previous reflective QoS mapping by a dedicated configuration.

The third option suffers from possible race conditions since it may not be fully predictable whether the UE received a DL data packet or the RRCConnectionReconfiguration first. Also, just as the second option, ambiguities exist upon mobility.

Generally, we think that RRC signalling should always have precedence over L2 and L1 control signalling. It would ensure a clean split and avoid any ambiguity. Also during mobility this principle ensures that the target eNB is aware of all configured UL QoS mappings applied by the UE. Besides that, it would also allow the eNB to map a DL QoS flow onto a different DRB than the UL QoS flow with the same ID.

If the eNB configures the UE with an “uplink QoS Flow to DRB filter”, it overrides any reflective mapping for this QoS flow.

## Maintaining QoS mapping during handover

In the context of inter-cell mobility it should be discussed whether the UE maintains the reflective UL QoS filters. As mentioned above, the target eNB does not know the UE’s reflective QoS filters from the AS-Config. One could consider that the source eNB provides the reflective UL QoS mappings to the target eNB (e.g. in AS-Context). Alternatively, the target node can change the QoS mapping and send the new mapping to the UE in the HO command (RRCConnectionReconfiguration). But we consider this being unnecessarily complex and it would also introduce risk of state mismatch. It appears simpler that the UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

The UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility and upon bearer-type change. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

## Omitting Flow ID in QoS header

To enable reflective QoS, the RAN marks downlink packets over Uu with a QoS flow ID. The UE marks uplink packets over Uu with the QoS flow ID for the purposes of marking forwarded packets to the CN.

RAN2-96 discussed whether these fields should always be present or whether the eNB should configure their presence explicitly. In our view, the reflective QoS concept intends to reduce the signalling overhead but the need to include the flow ID in each QoS header increases the user plane protocol overhead. The relative overhead is marginal for large IP packets but could be considered significant for services such as VoIP.

One could consider omitting the flow ID in all but the first DL packet of a certain flow. Or one could compress the flow ID on the radio interface. But since such schemes increases the complexity (book-keeping) in UE and eNB they don’t come for free either.

There are however still cases when the Flow ID does not need to be conveyed: Whenever the eNB configures a dedicated DRB for a Flow (1:1 mapping) and if it configures the Flow to be mapped to the DRB (explicitly configured UL “AS filter”), neither the UE nor the eNB need to include the flow ID. Since this may be a typical case for IMS Voice as well as for latency critical services where the relative overhead due to the Flow-ID is significant, the eNB should have means to configure for each DRB whether the Flow-ID is conveyed in the header.

As discussed during RAN2-96, the NAS level will not provide a QoS Flow ID in deployment scenario where NR connects to EPC (scenario 3). Also in such cases, it should be possible not to include the Flow ID over the Uu interface.

The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink headers.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The UE monitors the Flow ID (if present) in all downlink packets and updates the corresponding reflective “uplink QoS Flow ID to DRB mapping” accordingly.

Proposal 2	The RAN may configure, modify and release a configured “Uplink QoS Flow to DRB” mapping (a pair of “QoS Flow ID” and “DRB ID”) by an RRCConnectionReconfiguration.

Proposal 3	Additional UE functionality for avoiding possible out-of-order delivery when re-mapping a QoS-Flow to a different DRB (by explicit signalling or by update reflective QoS mapping) should not be introduced.

Proposal 4	If Proposal 3 is not agreeable (i.e., if RAN2 believes that re-ordering due to QoS flow re-mapping shall be avoided), the PDCP transmitter shall copy all the not-yet-RLC-ACKed PDCP PDUs to the target DRB’s PDCP entity.

Proposal 5	If the eNB configures the UE with an “uplink QoS Flow to DRB filter”, it overrides any reflective mapping for this QoS flow.

Proposal 6	The UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility and upon bearer-type change. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

Proposal 7	The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink headers.

# Annex: QoS-related agreements in previous meetings

RAN2-95 discussed the basic principles of the NR QoS framework and reached the following agreements:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS. RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

At RAN2-95bis some further agreements were achieved and the first of the FFSs above was resolved:

Agreements

1:	RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL.

1a	RAN can map multiple QoS flows to a DRB.

2	Specification will not forbid a GBR flow and non-GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

3	Specification will not forbid more than one GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

FFS: Whether traffic from different PDU sessions can be mapped to one DRB or not.

Agreements

1	Default DRB is established by eNB at PDU session establishment (or an existing DRB may be used if mapping of more than one session to a DRB is allowed)

2. 	If the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network.

FFS How and when the network can remap the flow to more appropriate DRB.

FFS the first packet is handled in the case that pre-authorised QoS is configured

FFS whether the pre-authorised QoS applies to RAN or only to the UE.

FFS whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB.

RAN2#96:

Agreement

1:	Traffic from different PDU sessions are mapped to different DRBs

2:	In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping (confirmation of SA2 agreement status).

3:	In UL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping.

4	DL packets over Uu are marked in band with QOS-flow-id for the purposes of reflective QoS

5	UL packets over Uu are marked in band with QOS-flow-id for the purposes of marking forwarded packets to the CN.

FFS for bullets 4 and 5 whether it can be semi-statically configured to not include the QOS flow ID in some cases.

FFS for bullets 4 and 5 whether it might be possible to use a shorter id over the radio compared to that received from the CN. This is a stage 3 issue.

Agreements

1	For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

2	The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

3	RRC can configure an uplink mapping

FFS The precedence of the RRC configured mapping and reflective QoS (e.g. can reflective QoS update an RRC configured mapping)

Working assumption:

If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

=>	FFS whether the QoS field is added by PDCP or a new protocol layer above PDCP.

RAN2 Ad-Hoc January 2017:

Agreements

1: A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including:

-	QOS flow->DRB routing;

-	QoS-flow-id marking in DL packets;

-	QoS-flow-id marking in UL packets;

2	The new protocol layer is applicable for all cases connecting to the 5G-CN

3:	Single protocol entity is configured for each individual PDU session.

RAN2-97 Athens:

1 RAN2 to confirm that the timing of non-default DRB establishment (RAN to UE) for QoS Flow configured during PDU Session Establishment could be done NOT at the same time as PDU Session Establishment. (up to eNB implementation) 2 Working assumption from RAN2#96 is confirmed. i.e. First UL packet that doesn't have a mapping to a DRB, is mapped to a default DRB.

1 “Lossless HO”, that is, lossless, in sequence without duplication to upper layers, should be supported in specification for intra-NR. FFS whether we support QoS flow remapping at handover and, if supported, whether the handover is lossless for this case.