3GPP TSG RAN WG2 Meeting #97-Bis	R2-1703002

Spokane, USA, 3rd-7th April 2017

Agenda item:	8.10

Source: 	Samsung

Title: 	Clarification on MBMS Notification Configuration

Document for:	Discussion and Decision

- Introduction

This contribution discusses about a possible ambiguity for MBMS-NotificationConfig in case of FeMBMS and provides clarification.

- Discussion

Following is the structure for MBMS-NotificationConfig information element [1]

MBMS-NotificationConfig information element -- ASN1START MBMS-NotificationConfig-r9 ::= SEQUENCE { notificationRepetitionCoeff-r9 ENUMERATED {n2, n4}, notificationOffset-r9 INTEGER (0..10), notificationSF-Index-r9 INTEGER (1..6) } MBMS-NotificationConfig-v14xy ::= SEQUENCE { notificationSF-Index-v14xy INTEGER (7..8) OPTIONAL } -- ASN1STOP MBMS-NotificationConfig field descriptions. notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset.. notificationRepetitionCoeff Actual change notification repetition period common for all MCCHs that are configured= shortest modification period/ notificationRepetitionCoeff. The ‘shortest modificaton period’ corresponds with the lowest value of mcch-ModificationPeriod of all MCCHs that are configured. Value n2 corresponds to coefficient 2, and so on.. notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7 and 8 correspond with subframe #0 and #5 respectively. If notificationSF-Index-v14xy is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively..

As per the definition of notificationOffset-r9, its range varies from 0 to 10. For MBMS-dedicated cell or/ FeMBMS/Unicast-mixed cell, all the values are not valid as notification can only be transmitted over the non-MBSFN subframe (CAS subframe) which has a periodicity of 40ms. Thus network has limited possible configuration values as 0, 4 and 8 only. Reuse of legacy notificationOffset-r9 can be ambiguous and confusing to network implementer without clarifying this limitation.

Option 1: Explicitly specify that usable values of parameter notificationOffset-r9 for MBMS-dedicated cell or/ FeMBMS/Unicast-mixed cell is 0, 4 and 8 only.

MBMS-NotificationConfig field descriptions

notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset. For MBMS-dedicated cell or FeMBMS/Unicast-mixed cell, only possible values are 0,4 and 8.

Alternatively, it is possible to use the range (0..10) with a different interpretation of notificationOffset-r9 for MBMS-dedicated cell or FeMBMS/Unicast-mixed cell. In this case, each value indicates the notification in terms of non-MBSFN subframes (i.e. CAS subframes) which are 40 ms apart i.e. 0 indicates  no offset, 1 indicates offset by one CAS subframe (40 ms), 2 indicates offset by 2 CAS subframes (80 ms) and so on.

Option 2: For MBMS-dedicated cell or FeMBMS/Unicast-mixed cell, SFN mod notification repetition period = 4* notificationOffset is used to identify the SFN on which MBMS notification is to be transmitted

MBMS-NotificationConfig field descriptions

notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset. For MBMS-dedicated cell or FeMBMS/Unicast-mixed cell, SFN mod notification repetition period = 4* notificationOffset

In both options, it is possible to reuse the legacy notificationOffset-r9 for FeMBMS in a clear and unambiguous way. Option 2 seems better as it provides usage of all existing values of the range for notificationOffset-r9 parameter.

Proposal 1: RAN2 should discuss and adopt option 2 for reusing notificationOffset-r9 for FeMBMS

- Conclusion

Proposal 1: RAN2 should discuss and adopt option 2 for reusing notificationOffset-r9 for FeMBMS

An associated CR

- References

[1] 3GPP 36331 RRC spec v14.2.1