3GPP TSG-RAN2 Meeting #97bis			     R2-1703204

Spokane, Washington, USA, 3rd – 7th April, 2017

Agenda item:		8.21 (LTE_eFDMIMO-Core)

Source:	LG Electronics Inc.

Title: 	Remaining issues in Activation/Deactivation of CSI-RS resources MAC CE for eFD-MIMO

Document for:	Discussion and Decision

1.	Introduction

For Activation/Deactivation of CSI-RS resources, as the result of email discussion [97#20], Activation/Deactivation of CSI-RS resources MAC CE is introduced. This document addresses remaining issues regarding Activation/ Deactivation of CSI-RS resources MAC CE.

2.	Remaining Issues in MAC CR

RAN1 introduced two CSI-RS modes called “aperiodic CSI-RS” and “multi-shot CSI-RS” to overcome periodic CSI-RS mode when using UE-specific beamformed CSI-RS. Accordingly, RAN2 introduced an Activation/Deactivation of CSI-RS resources MAC CE which controls aperiodic and multishot CSI-RS resources per CSI process.

Activation/Deactivation of CSI-RS resources MAC CE includes CSI-RS commands for all CSI processes configured for a UE, where each CSI-RS command indicates activation/deactivation status of each CSI-RS resource configured for a CSI process.

Observation 1. The Activation/Deactivation of CSI-RS resources MAC CE shall include CSI-RS commands for all CSI processes configured for the UE.

Basically, each CSI process may be configured with Rel-13 periodic CSI-RS mode or Rel-14 CSI-RS mode which is either aperiodic CSI-RS or multi-shot CSI-RS resource. As shown in Table 1 below, when a set of CSI processes are configured, a part of CSI processes may be configured with Rel-13 periodic CSI-RS mode and the others of configured CSI processes may be used for Rel-14 CSI-RS mode configured with either aperiodic or multi-shot CSI-RS resources.

In this case, with observation 1, Activation/Deactivation of CSI-RS resources MAC CE shall include a CSI-RS command even for a CSI process which is configured with neither aperiodic CSI-RS nor multi-shot CSI-RS resource. This would consume radio resource uselessly by including unnecessary CSI-RS command. Furthermore, it is not clear what the UE shall do if the UE receives a CSI-RS command for a CSI process which is configured with neither aperiodic CSI-RS nor multi-shot CSI-RS resource.

Observation 2. The Activation/Deactivation of CSI-RS resources MAC CE shall include a CSI-RS command even for a CSI process which is configured with neither aperiodic CSI-RS nor multi-shot CSI-RS resource.

Table 1. TS36.331 for configuring CSI Process

CQI-ReportConfig information elements -- ASN1START << skip unrelated part >> CSI-ProcessToAddModList-r11 ::= SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-Process-r11 CSI-ProcessToReleaseList-r11 ::= SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF CSI-ProcessId-r11 << skip unrelated part >> -- ASN1STOP csi-ProcessToAddModList For a serving frequency E-UTRAN configures one or more CSI-Process only when transmission mode 10 is configured for the serving cell on this carrier frequency. CSI-Process information elements -- ASN1START CSI-Process-r11 ::= SEQUENCE { csi-ProcessId-r11 CSI-ProcessId-r11, csi-RS-ConfigNZPId-r11 CSI-RS-ConfigNZPId-r11, << skip unrelated part >> [[ cqi-ReportAperiodicProc-v1310 CHOICE { release NULL, setup CQI-ReportAperiodicProc-v1310 } OPTIONAL, -- Need ON cqi-ReportAperiodicProc2-v1310 CHOICE { release NULL, setup CQI-ReportAperiodicProc-v1310 } OPTIONAL, -- Need ON eMIMO-Type-r13 CSI-RS-ConfigEMIMO-r13 OPTIONAL -- Need ON ]], [[ eMIMO-Type-v14xy CSI-RS-ConfigEMIMO-v14xy OPTIONAL, -- Need ON eMIMO-Type2-r14 CSI-RS-ConfigEMIMO2-r14 OPTIONAL, -- Need ON eMIMO-Hybrid-r14 CSI-RS-ConfigEMIMO-Hybrid-r14 OPTIONAL, -- Need ON advancedCodebookEnabled-r14 BOOLEAN OPTIONAL -- Need ON ]] } -- ASN1STOP eMIMO-Type Parameter: eMIMO-Type, see TS 36.213 [23], TS 36.211 [21]. If eMIMO-Type is set to nonPrecoded, the codebooks used for deriving CSI feedback are in TS 36.213 [23, Table 7.2.4-10 to Table 7.2.4-17]. Choice values nonPrecoded and beamformed correspond to ‘CLASS A‘ and ‘CLASS B’ respectively, see TS 36.212 [22] and TS 36.213 [23]. CSI-ProcessId information elements -- ASN1START CSI-ProcessId-r11 ::= INTEGER (1..maxCSI-Proc-r11) -- ASN1STOP

There could be two possible options to resolve this issue:

- Option 1. Activation/Deactivation of CSI-RS resources MAC CE includes CSI-RS command only for a CSI process which is configured with either aperiodic CSI-RS or multi-shot CSI-RS resources.

- Option 2. Specify the UE behaviour when the UE receives a CSI-RS command for a CSI processes which is configured with neither aperiodic CSR-RS or multi-shot CSI-RS resources, e.g., to ignore the CSI-RS command for that CSI process

Option 1 would avoid useless radio resource consumption while Option 2 consumes more radio resources unnecessarily without any purpose. Thus, we prefer Option 1.

Proposal 1: Activation/Deactivation of CSI-RS resources MAC CE includes CSI-RS command only a CSI process which is configured with either aperiodic CSI-RS or multi-shot CSI-RS resources.

The current Activation/Deactivation of CSI-RS resources MAC CE uses Ri field to indicate the activation/deactivation status of the CSI-RS resources associated with CSI-RS-ConfigNZPId i for the CSI process. However, R is the conventional notation to express reserved bit. Almost all or maybe all of specifications in 3GPP use R for this meaning. In our view, A can be a proper notation for Activation/Deactivation of CSI-RS resources MAC CE.

Proposal 2: In Activatoin/Deactivation CSI-RS command, notation R is replaced by A in order to follow conventional representation in 3GPP.

3.	Conclusion

In this document, we explained remaining issues in Activation and Deactivation of CSI-RS resources MAC CE, and propose followings:

Observation 1. The Activation/Deactivation of CSI-RS resources MAC CE shall include CSI-RS commands for all CSI processes configured for the UE.

Observation 2. The Activation/Deactivation of CSI-RS resources MAC CE shall include a CSI-RS command even for a CSI process which is configured with neither aperiodic CSI-RS nor multi-shot CSI-RS resource.

Proposal 1: Activation/Deactivation of CSI-RS resources MAC CE includes CSI-RS command only a CSI process which is configured with either aperiodic CSI-RS or multi-shot CSI-RS resources.

Proposal 2: In Activatoin/Deactivation CSI-RS command, notation R is replaced by A in order to follow conventional representation in 3GPP.

References

[1] R2-1702084, TS 36.321 CR1021R1 on Introducing Activation/Deactivation CSI-RS MAC CE for eFD-MIMO, Samsung