3GPP TSG-RAN WG2 #97bis                   								R2-1703379

Spokane, Washington, USA, 3rd-7th April 2017

Agenda item:	10.2.4.4

Source: 	Huawei, HiSilicon

Title: 	Inter-RAT mobility for inactive UE

Document for:	Discussion and Decision

# Introduction

In RAN2#97 meeting, two solutions were provided for case NR RRC_INACTIVE/LTE light connected UE performs cell reselection between NR and E-UTRA connected to NextGen Core (NGC), where the target state after cell reselection are still FFS. In this paper, we show our view on the target state for this two solution and provide potential procedures.

# Discussion

## Mobility between LTE and NR connecting to NR CN

For case NR RRC_INACTIVE/LTE light connected UE performs cell reselection between NR and E-UTRA connected to NextGen Core (NGC), the following two solutions were proposed in [1]:

1)	At every inter-RAT cell reselection, the UE initiates a CN tracking/registration area update procedure;

2)	At inter-RAT cell reselection, the UE does not always perform the update procedure. Registration/tracking area update and/or RAN area update is only performed when the reselected cell does not belong to the area where the UE is registered.

Solution 2 implies that RAN notification areas (RNA) may consist of cells from both NR and E-UTRA, so that inactive UE can autonomously move to light connected without network involved procedure after reselection of an E-UTRA cell (and vice versa). Solution 2 provides benefits in terms of signalling. However, we think this should be carefully examined:

- We are wondering the probability of reselection from NR to E-UTRAN. If a UE is in the INACTIVE state of NR, it should camp on NR as far as possible. When the UE finds the E-UTRAN has better quality, it doesn’t mean NR will have worse quality when the UE has data to be transmitted so this UE should not perform reselection to E-UTRAN. In our view, the cell reselection criteria should be that NR has higher priority for camping.

- If, in spite of higher priority for NR as compared to LTE, there would be frequent reselections from NR to LTE, this means that UE battery consumption is rather high, even though signalling is avoided at inter-RAT cell reselection. If there are deployments where such a risk is identified, RAN2 should attempt to minimize inter-RAT cell reselections in such deployments.

- Even for frequent reselections from NR to LTE case, solution 2 is not the only way to reduce signalling, i.e. the UE could be moved to idle state after inter-rat cell reselection.

- Another consideration is the number of UE in INACTIVE state. The number of INACTIVE state UE may be large, this is one important objective of NR and NR is expected, at some point, to allow direct data transmission in INACTIVE state, without RRC signalling. If many NR UEs perform reselection to E-UTRAN, since transmission in light connection requires using the resume procedure, there will be extra signalling as compared to the direct data transmission in INACTIVE state in NR.

- Another point is the ID allocation consideration. It’s not clear for us whether the ID allocation should be coordinated between NR and E-UTRAN because the same procedure is considered. If the resume ID can be allocated by either NR or E-UTRAN, it means NR and E-UTRAN need to do some coordination, extra signalling will be introduced. Otherwise, UE/RNA ID among RATs should be aligned, which needs to be standardized.

- Whether the UE in inactive state can move to light connected autonomously after reselection to E-UTRAN is also unclear since the features of NR inactive and light connected are not aligned. For lightly connected state, it is agreed that “idle mode features covered by TS 36.304 are applicable for UEs in RRC_CONNECTED with light RRC connection” and “when any MO data/signaling is triggered, the UE will get back to be connected to eNB”, e.g., the suspended DRB in light connected state should be resumed by RRC signalling to transmit data (i.e., transmit data after necessarily performing a full state transition to connected state), whereas for inactive state, UE can transmit UL data without necessarily performing a full state transition to connected state. Without network involved procedure, it is difficult for UE to autonomously achieve state from a state with suspended RBs to another state transition that can directly transmit data.

- Furthermore, NR and E-UTRA will have different L2, e.g., concatenation is removed in NR RLC and the MAC format of NR may also different from E-UTRA, so that UE may be required to store both NR and E-UTRA L2 configurations to support solution 2, which will increase complexity of UE.

In summary, solution 2 wouldn't solve UE battery consumption problems, some of its technical aspects are still unclear and it may increase signalling due to UE activity to send small data. Besides, the light connection is not finished in Release 14, and the E-UTRA connected to NGC may not support the light connection.

Observation 1:  Solution 2 wouldn't solve UE battery consumption problems in case of frequent reselections from NR to E-UTRAN, some of its technical aspects are still unclear and it may increase signalling due to UE activity to send small data.

When the reselected new RAT cell does not belong to the area where the UE is registered, registration/tracking area update and/or RAN area update will be triggered. In [1], for both solution 1 and solution 2, it is still not decided which state the NR RRC_INACTIVE/LTE light connected UE will enter after reselecting into a new RAT cell. Regarding to the target state, there are two options:

Option 1: Inactive UE enters lightly connected state in case of moving from NR to E-UTRAN connected to NGC and vice versa

This option can be achieved by the resume procedure similar as in intra-RAT mobility case. However, it requires extra efforts in the RAN to coordinate UE IDs and fetch the UE context while the purpose is only to do a TAU.

Option 2: Inactive UE enters idle state in case of moving from NR to E-UTRAN connected to NGC and vice versa

This options does not require any special design, the UE will perform a TAU/RNA update and the target RAN has the possibility to select the UE state. If the RAN moves the UE to inactive/light connected, the UE will have a context ready with AS configuration of the current RAT.

From standardization point of view, transition to IDLE state has minimum impact to standard and save time for stage 1 study. Hence, we prefer to take option 2 as the baseline.

Proposal 1: UE in the NR inactive state transits to idle state in case of moving from NR to E-UTRAN connected to NG Core should be the baseline.

## Possible solutions for NR to clear the related UE information

Inactive UE in NR can move into different RATs, e.g., GERAN, UTRAN and legacy LTE connected to EPC or NG Core. Inactive UE will enter idle if re-selection into “legacy RAT” (Here, “legacy RAT” refers to GERAN, UTRAN and legacy LTE connected to EPC or NG Core).

Fig.2 inter-RAT mobility for inactive UE

Normally, after inactive UE leaves the NR and enters idle in legacy RAT, it is reasonable for NR to release all possible information of the UE. There may be three possible solutions for NR to clear the related UE context.

Opt 1: Indication from target RAT CN to NR CN

Generally, when UE moves into legacy RAT, it will notify the RAT by initiating TAU or similar procedure. If UE indicates the target RAT with identifier related to the source RAT. After interaction between two RATs, the source RAT will know that the inactive UE has moved out of the coverage and clear all possible UE contexts in RAN and CN [4].

Opt 2: Notification from UE to the NR RAN

Besides, it is considered for the leaving inactive UE to inform the NR RAN of an indication to release the stored UE context. After the inactive UE completes the related procedure, e.g. TAU and totally camps on the target RAT. The UE can return to the former NR RAT and initiate a release procedure to notify the NR RAT to release the RAN/CN connection and clear the stored AS context. As a possible way, the inactive UE can send the release notification via grant free scheme.

Fig.3 UE initiated release notification to NR RAT

Opt 3: Detecting by NR RAN based on timer/counter

If inactive UE is configured with periodically RAN Notification Area (RNA) Update, we can rely on NR RAN to decide when to clear all UE related contexts. For example, if the NR RAN doesn’t receive the RNA Update message at the predefined occasion. NR RAN starts a timer or counter. If no RNA Update is received from the inactive UE, when timer or counter expires, the NR RAN considered the UE out of its service or coverage. Thereby, the NR RAN can clear the stored UE context and release the CN/RAN connection.

Based on the discussion in the aforementioned section, NR RAT should clear the stored inactive UE context. To align with the legacy interaction between UTRAN and E-UTRAN, option 1 is highly anticipated as the baseline.

Proposal 2: Source RAT (NR) should clear the stored UE context after knowing the UE moving into legacy RAT, e.g., GERAN, UTRAN, legacy LTE connected to EPC or NG Core.

# Conclusion

In this paper, we analysed the state transition for inter-RAT mobility for inactive UE. Therefore we made the following proposals:

Observation 1: Solution 2 wouldn't solve UE battery consumption problems in case of frequent reselections from NR to E-UTRAN, some of its technical aspects are still unclear and it may increase signalling due to UE activity to send small data.

Proposal 1: UE in the NR inactive state transits to idle state in case of moving from NR to E-UTRAN connected to NG Core should be the baseline.

Proposal 2: Source RAT (NR) should clear the stored UE context after knowing the UE moving into legacy RAT, e.g., GERAN, UTRAN, legacy LTE connected to EPC or NG Core.

# Reference

- R2-1702425 TP on inter-RAT inactive mobility, Ericsson

- R2-1700493 Mobility between LTE and NR for inactive UEs, Ericsson

- 3GPP TR 38.804 TR for Study on New Radio Access Technology Radio Interface Protocol Aspects3GPP TS 23.401 General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access