3GPP TSG-RAN WG2 #97bis	Tdoc R2-1703396

# Spokane, WA, USA, 3rd – 7th April 2017

Agenda Item:	10.4.1.1

Source:	Ericsson

Title:	Discussion about TS 38.331 skeleton

Document for:	Discussion, Decision

# Introduction

The purpose of this contribution is to discuss TS 38.331 skeleton structure.

# Discussion

All 3GPP specifications have basically the same structure for clauses 1, 2, 3 and the differences start from clause 4. On a general level, both 36.331 and 25.331 have very similar structure. One essential difference between 36.331 and 25.331 is that 36.331 does not contain separate clauses for tabular descriptions. It is expected that there are no reasons to include tabular descriptions in 38.331 either and hence it is proposed to reuse the structure of 36.331 as much as possible.

TS 38.331 should reuse the structure of TS 36.331 as much as possible and tabular descriptions (as in TS 25.331) should not be used.

The structure of 36.331 could be reused with the following exceptions and remarks;

- Clause 5 has on general level similar structure as 36.331 but some sub-clauses are removed because they are not within the scope of NR work item

- Sub-clause 5.2 system information procedures can be reused but it can be considered to restructure them slightly as the current 36.331 structure has become somewhat complex as more features are added. E.g. sub-clause 5.2.1 includes descriptions that are rather UE procedures and requirements than a short introduction. Instead, new procedures for triggering acquisition of system information could be introduced in sub-clause 5.2.3. In addition, the content of sub-clause 5.2.3 acquisition of system information message should appear earlier in the procedures.

- Sub-clause 5.3 is reused but 5.3.14 proximity indication is not included so far (should be FFS?)

- The order of 5.6 headings is slightly different compared to 36.331 where UE information is now on 5.6.4 and SCG failure information is on 5.6.5. The rest of the sub-clauses are not within the scope of the work item and hence not included in the skeleton

- Sub-clauses 5.8 MBMS and 5.9 RN procedures are not included because they are not within the NR work item scope

- Clause 6 can be reused but without 6.3.7 and 6.3.8 because MBMS and SC-PTM are not within the NR work item scope

- The order to headings in clause 7 follow the same order as in 25.331, i.e. first timers, counters, constant and UE variables as last. There will be many UE variables and therefore it seems reasonable to start with the short sub-clauses

- Clauses 8 and 9 are needed but without sidelink and NB-IoT parts

It proposed to discuss if this principle is acceptable.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	TS 38.331 should reuse the structure of TS 36.331 as much as possible and tabular descriptions (as in TS 25.331) should not be used.

# References

R2-17xxxxx, TS 38.331 skeleton, Ericsson, 3GPP TSG-RAN WG2 #97bis, Spokane, WA, USA, 3rd – 7th April 2017

TS 36.331, Radio Resource Control protocol

TS 25.331, Radio Resource Control protocol