3GPP TSG-RAN WG2 #97bis	R2-1703398

# Spokane, WA, USA, 3rd – 7th April 2017

Agenda Item:	15.1

Source:	Ericsson

Title:	DL Interference Mitigation

Document for:	Discussion

# Introduction

A work item to mitigate potential downlink interference has been introduced in Rel-15 [1]. When UMTS and GSM RATs are deployed co-sited, there could be possible interference in UMTS cells from GSM and vice-versa. In such scenario, it would be beneficial to indicate to the UE in UMTS cells about potential GSM interference so that the UE can take necessary actions.

This paper aims to identify a possible solution as how Network can inform to the UE about such prevailing interference.

# Discussion

SIB5/SIB5Bis/SIB6 are used to broadcast physical channel parameters to the UE in idle and connected mode [2]. A simple solution, which allows the Network to indicate to the UE that GSM interference is prevalent in a certain cell, would be to add a new indicator field in these SIB messages, as below.

| Adjacent channel interference indication | OP |  | Enumerated (TRUE) | The presence of this IE indicates that adjacent channel interference may be present. | REL-15 |
| --- | --- | --- | --- | --- | --- |

One advantage of this SIB Signalling solution is that it can be made release independent, i.e.; If this IE is included in existing release version, the UE would simply ignore the message if it is not understood. Thus, the impact of the solution is minimum.

Detailed SIB impacts are depicted below in the Appendix, Section 3.

Include a new IE in SIB5/SIB5Bis/SIB6 to indicate to the UE about the potential DL Interference.

# Conclusion

Based on the discussion in section 2 we propose the following:

- Include a new IE in SIB5/SIB5Bis/SIB6 to indicate to the UE about the potential DL Interference.

# Appendix

Detailed SIB impacts is depicted below. New IE is highlighted in blue.

##### Section: 10.2.48.8.8 From 25.331 RRC Spec

##### System Information Block type 5 and 5bis

The system information block type 5 contains parameters for the configuration of the common physical channels in the cell. System information block type 5bis uses the same structure as System information block type 5. System information block type 5bis is sent instead of system information block type 5 in cells that use Band IV or Band IX or Band X if it is broadcasted.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB6 Indicator | MP |  | Boolean | TRUE indicates that SIB6 is broadcast in the cell. When the UE receives SIB5 in the System Information Container message, this IE is interpreted as FALSE |  |
| SIB22 Indicator | MP |  | Boolean | TRUE indicates that SIB22 is broadcast in the cell. | REL-11 |
| PhyCH information elements |  |  |  |  |  |
| PICH Power offset | MP |  | PICH Power offset 10.3.6.50 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>AICH Power offset | MP |  | AICH Power offset 10.3.6.3 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| >TDD |  |  |  |  |  |
| >>PUSCH system information | OP |  | PUSCH system information 10.3.6.66 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| >>PUSCH system information VHCR | OP |  | PUSCH system information VHCR 10.3.6.66a | Only for 7.68 Mcps TDD If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-7 |
| >>PDSCH system information | OP |  | PDSCH system information 10.3.6.46 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| >>TDD open loop power control | MP |  | TDD open loop power control 10.3.6.79 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| Primary CCPCH info | OP |  | Primary CCPCH info 10.3.6.57 | Note 1 |  |
| PRACH system information list | MP |  | PRACH system information list 10.3.6.55 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| Common E-DCH system info | OP |  | Common E-DCH system info 10.3.6.9a |  | REL-8 |
| Secondary CCPCH system information | MP |  | Secondary CCPCH system information 10.3.6.72 | Note 2 If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| CBS DRX Level 1 information | CV-CTCH |  | CBS DRX Level 1 information 10.3.8.3 |  |  |
| Frequency band indicator | OP |  | Frequency band indicator 10.3.6.35b |  | REL-6 |
| Frequency band indicator 2 | OP |  | Frequency band indicator 2 10.3.6.35c |  | REL-6 |
| Frequency band indicator 3 | OP |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| Frequency Bands Indicator Support | OP |  |  |  | REL-10 |
| >CHOICE mode |  |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Frequency bands indicator for redirection | MP | 1 to< maxFreqBandsIndicatorSupport> |  | The presence of this IE indicates one or two bands for redirection. | REL-10 |
| >>>>CHOICE frequency bands indicator |  |  |  |  | REL-10 |
| >>>>>Frequency bands indicator 1 for redirection |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>>>>Frequency bands indicator 2 for redirection |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>>>>Frequency bands indicator 3 for redirection |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| >>TDD |  |  |  |  |  |
| >>>Frequency bands indicator for 1.28Mcps TDD | MP | 1 to< maxFreqBandsIndicatorSupport> |  | The presence of this IE indicates one or two bands for RF capability. | REL-10 |
| >>>>Frequency bands indicator for TDD | MP |  | Frequency band indicator for TDD 10.3.6.35d |  | REL-10 |
| Multiple Frequency Band indicator list | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | A list of additional frequency bands which the cell belong to. The order of appearance of the additional supported frequency bands in the list indicates their priority i.e from higher to lower. If the UE supports the frequency band indicated explicitly or implicitly in accordance with subclause 8.1.1.6.5, it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the IE "Multiple Frequency Band indicator list". | REL-10 |
| >CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| HSDPA cell Indicator | MD |  | Enumerated (HSDPA Capable Cell) | Default is ’HSDPA capability not indicated’. ’HSDPA Capable Cell’ means that the UE may consider this cell as part of the HSDPA coverage area for display indication only. This indication shall not be used for any other purpose. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. | REL-6 |
| E-DCH cell Indicator | MD |  | Enumerated (E-DCH Capable Cell) | Default is ’E-DCH capability not indicated’. ’E-DCH Capable Cell’ means that the UE may consider this cell as part of the E-DCH coverage area for display indication only. This indication shall not be used for any other purpose. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. | REL-6 |
| Secondary CCPCH system information MBMS | OP |  | Secondary CCPCH system information MBMS 10.3.6.72a | Included if MCCH is on an S-CCPCH used only for MBMS. Note 2 | REL-6 |
| CHOICE mode | OP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>HS-DSCH common system information | MP |  | HS-DSCH common system information 10.3.6.36c | Included if cell supports HS-DSCH reception in CELL_FACH and during IDLE to RRC Connection state transition. | REL-7 |
| >>HS-DSCH paging system information | OP |  | HS-DSCH paging system information 10.3.6.36d | Included if cell supports for UEs in RRC Connected state paging message reception on HS-DSCH. | REL-7 |
| >TDD |  |  |  |  | REL-7 |
| >>HS-DSCH common system information | MP |  | HS-DSCH common system information 1.28Mcps TDD 10.3.6.36ca | Included if cell supports HS-DSCH reception in CELL_FACH and during IDLE to RRC Connection state transition. | REL-8 |
| >>HS-DSCH paging system information | OP |  | HS-DSCH paging system information 1.28Mcps TDD 10.3.6.36da | Included if cell supports for UEs in RRC Connected state paging message reception on HS-DSCH. | REL-8 |
| TDD MBSFN information | OP |  | TDD MBSFN Information 10.3.6.78b | TDD only: included only if some timeslots are designated to MBSFN. | REL-7 |
| HS-DSCH DRX in CELL_FACH Information | OP |  | HS-DSCH DRX in CELL_FACH Information 10.3.6.36g |  | REL-8 |
| HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD | OP |  | HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD 10.3.6.36h |  | REL-8 |
| Second Frequency info | OP |  | Integer (0 .. 16383) | Note 3 | REL-8 |
| Treset Usage Indicator | OP |  | Enumerated (TRUE) | Only for 1.28 Mcps TDD. The presence of this IE means the timer Treset is not valid when the dedicated H-RNTI is configured in CELL_FACH and CELL_PCH. | REL-8 |
| UpPCH Position Info | CV-Frequency |  | Integer (0 .. 127) | Only for 1.28 Mcps TDD. | REL-8 |
| Cell Update message with optimised encoding | OP |  | Enumerated (TRUE) | Only for FDD. The presence of this IE means that UE shall use the Rel-11 asn1 message type for Cell Update, if the UE supports this message type. | REL-11 |
| Adjacent channel interference indication | OP |  | Enumerated (TRUE) | The presence of this IE indicates that adjacent channel interference may be present. | REL-15 |

NOTE 1:	DL scrambling code of the Primary CCPCH is the same as the one for Primary CPICH (FDD only).

NOTE 2:	There is only one MCCH in a cell, which may either be mapped on to an S-CCPCH also used for non- MBMS purposes or to an S-CCPCH dedicated to MBMS. In the first case the MCCH configuration is specified within the IE "Secondary CCPCH system information", in the latter case the MCCH configuration is provided within the IE "Secondary CCPCH system information MBMS".

NOTE 3:	This IE is used in 1.28 Mcps TDD multi-frequency cell to indicate the secondary frequency at which enhanced E-DCH transmission and HS-PDSCH reception for 1.28 Mcps TDD is supported and to indicate that corresponding IEs: "Common E-DCH system info", "HS-DSCH common system information" (TDD) and "HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD" can apply for this frequency. The absense of "Frequency info" means that enhanced E-DCH access transmission and HS-PDSCH reception and above IEs apply for primary frequency. For 1.28 Mcps TDD only.

| Condition | Explanation |
| --- | --- |
| CTCH | The IE is mandatory present if the IE "CTCH indicator" is equal to TRUE for at least one FACH, otherwise the IE is not needed in the message |
| Frequency | The IE is optional if the IE "Frequency info" is present, otherwise the IE is not needed. |

##### 10.2.48.8.9	System Information Block type 6

The system information block type 6 contains parameters for the configuration of the common and shared physical channels to be used in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PhyCH information elements |  |  |  |  |  |
| PICH Power offset | MP |  | PICH Power offset 10.3.6.50 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>AICH Power offset | MP |  | AICH Power offset 10.3.6.3 |  |  |
| >TDD |  |  |  |  |  |
| >>PUSCH system information | OP |  | PUSCH system information 10.3.6.66 |  |  |
| >>PUSCH system information VHCR | OP |  | PUSCH system information VHCR 10.3.6.66a | Only for 7.68 Mcps TDD | REL-7 |
| >>PDSCH system information | OP |  | PDSCH system information 10.3.6.46 |  |  |
| >>TDD open loop power control | MP |  | TDD open loop power control 10.3.6.79 |  |  |
| Primary CCPCH info | OP |  | Primary CCPCH info 10.3.6.57 | Note 1 |  |
| PRACH system information list | OP |  | PRACH system information list 10.3.6.55 |  |  |
| Secondary CCPCH system information | OP |  | Secondary CCPCH system information 10.3.6.72 |  |  |
| CBS DRX Level 1 information | CV-CTCH |  | CBS DRX Level 1 information 10.3.8.3 |  |  |
| Frequency band indicator | OP |  | Frequency band indicator 10.3.6.35b |  | REL-6 |
| Frequency band indicator 2 | OP |  | Frequency band indicator 2 10.3.6.35c |  | REL-6 |
| Frequency band indicator 3 | OP |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| Multiple Frequency Band indicator list | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | A list of additional frequency bands which the cell belongs to. The order of appearance of the additional supported frequency bands in the list indicates their priority i.e from higher to lower. If the UE supports the frequency band indicated explicitly or implicitly in accordance with subclause 8.1.1.6.6, it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the IE "Multiple Frequency Band indicator list". | REL-10 |
| >CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| Adjacent channel interference indication | OP |  | Enumerated (TRUE) | The presence of this IE indicates that adjacent channel interference may be present. | REL-15 |

# References

RP-170703, New WID on NW assisted DL interference mitigation, Qualcomm, RAN Meeting #75, March 6 - 9, 2017

25.331, Radio Resource Control (RRC) Protocol specification, Version 14.2.0, 2017-03-21