3GPP TSG-RAN WG2 #97bis	Tdoc R2-1703589

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.2.3.2

Source:	Ericsson

Title:	Mobility performance in heterogeneous networks

Document for:	Discussion, Decision

# Introduction

It has been discussed that the homogeneity of the TRP beam patterns in a network might influence the mobility performance, and particularly how a cell level measurement algorithm (i.e., cell level quality derivation based on individual beam measurements, e.g. “strongest beam”, “average of N beams”) performs against other algorithms. It is likely that taking a power average over more beams may lead to a different mobility performance when different neighbour cells use different multi-beam transmission patterns, e.g., different number of SS blocks in a burst set through wider beams per block. One of the cell quality derivation algorithms may be able to cope with more sophisticated network topologies better than others.

In this paper, we present simulation results for networks with both homogeneous and heterogeneous beam patterns to seek answers to some of these questions.

# Discussion

Our simulation is based on a regular 7-site (21-cell) deployment. To be able to make comparisons, we simulate both heterogeneous and homogeneous topologies. In the heterogeneous case each cell has a random number of beams, which can be between 1 and 16, whereas in the homogeneous case each cell has a fixed number of 16 beams. We study three different options for cell quality derivation from individual beams, namely “strongest beam” (cell quality is the quality of the strongest beam), “average 4 beams” (cell quality is derived by taking the average of the 4 strongest beams), and “average all beams” (cell quality is derived by taking the average of all available beams). A full list of simulation parameters is presented in Appendix A.

As in a companion contribution [1], we evaluate mobility performance in terms of ping-pong rate and handover failure rate.

Ping-pong occurs for example when the handover is triggered too early, so that the UE upon arrival in the target cell realizes that the old source cell has still higher quality, and immediately goes back (X  Y  X, where X and Y refer to different cells). The ping-pong rate is evaluated by counting the number of ping-pong occurrences within one second, divided by the total number of (successful and unsuccessful) handover events:

PPP=NPPNHO_success + NHO_fail

where NPP is the number of ping-pong occurrences, NHO_success is the number of successful handovers, and NHO_fail is the number of failed handovers. Simulation results are shown in Figure 1.

Figure 1: Ping-pong rates for homogeneous and heterogeneous networks.

A handover failure, on the other hand, might occur if the handover decision is delayed too much, or if the quality of the target cell drops too much before the handover can be completed. In this paper we determine if a handover failure happens by comparing the RSRP of the target cell to a cell detection threshold during the duration of TTT. If the measured RSRP falls below the threshold, it is assumed that the target cell can no longer be detected, and hence a handover failure is registered. Throughout this study, we assume a TTT of 100 ms. The handover failure rate is given by the ratio of the number of failed handover events to the total number of (successful and unsuccessful) handover events:

PHO_fail=NHO_failNHO_success + NHO_fail

where NHO_success is the number of successful handovers, and NHO_fail the number of failed handovers. Simulation results are shown in Figure 2.

Figure 2: Handover failure rates for homogeneous and heterogeneous networks.

These results show that none of the studied beam processing algorithms is particularly suited to handle heterogeneous networks. In general, the mobility performance in heterogeneous networks is simply worse overall.

In heterogeneous networks the mobility performance is generally worse than in homogeneous networks.

One notable exception is that the “average all beams” option shows better HOF performance in heterogeneous networks. The reason is likely that “all beams” means something very different in homogeneous and heterogeneous networks: In the homogeneous case the averaging is always done over 16 beams (there is no beam detection threshold). In the heterogeneous case on the other hand, most cells will have fewer than 16 beams, and thus also the averaging is done over a smaller number of beams.

Averaging over more beams to derive the cell quality leads to better ping-pong rates but more handover failures.

In summary, these results confirm our results presented earlier [2]: Averaging over more beams leads to slightly better ping-pong rates, but also to worse handover failure rates. It should be noted, though, that for both the “strongest beam” as well as the “average 4 beams” option both ping-pong and handover failure rates are rather small, even in the heterogeneous case. Also, these results do not preclude that more sophisticated handover algorithms might be able to reduce the number of handover failures without increasing the number of ping-pongs (or vice versa).

# Conclusion

We made the following observations:

Observation 1	In heterogeneous networks the mobility performance is generally worse than in homogeneous networks.

Observation 2	Averaging over more beams to derive the cell quality leads to better ping-pong rates but more handover failures.

# References

R2-1702797, Determination of serving cell quality in NR, RAN2, 3GPP TSG-RAN WG2 #97bis, Spokane, USA, 17th – 19th January 2017.

R2-1700681, Filtering of RRM measurements in CONNECTED, RAN2, 3GPP TSG-RAN WG2 NR AdHoc, Spokane, USA, 17th – 19th January 2017.

# Appendix A. Simulation Parameters

| Parameter name | Value |
| --- | --- |
| Operating frequency | 2GHz |
| Deployment | 7-site urban-macro with wrap-around (21 sectors) |
| Duplex | FDD |
| Tx antenna configuration | 1x16 (row array), up to 16 beams per sector |
| System Bandwidth | 40 MHz |
| TTI length | 0.125 ms |
| Subcarrier spacing | 60KHz |
| UE speed | 30 kmph / 120 kmph |
| UE position update period | 2 ms |
| UE ports | 2 |
| Propagation mode | 3GPP TR 36.873 |
| Channel estimation | Ideal |
| Cell detection threshold | -110 dBm |
| Hysteresis margin | 3 dB |
| Filter coefficients | L1 filter λ = 0.8, L3 filter k = 4 |
| Beam processing algorithm | Strongest beam / average 4 beams / average all beams |
| Beam grouping | Between L1 and L3 filtering |
| Handover measurement period | 20 ms |
| Handover update period | 20 ms |
| TTT | 100 ms |