3GPP TSG-RAN WG2 #99bis	Tdoc R2-1710199

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.1.3.7

Source:	Ericsson

Title:	Size of MSG3 in NR

Document for:	Discussion, Decision

# Introduction

This contribution evaluates the content and size of MSG3 in NR to be able to provide a size estimate to RAN1 for the physical layer design.

# Discussion

MSG3 is the third message in the NR random access procedure and carries the RRC connection request, RRC connection re-establishment request, and RRC connection resume request messages. As these RRC messages are sent on SRB0 using RLC TM and hence lacks segmentation, the whole RRC PDU must be sent within a single transport block which restricts its size. The size of the transport block is limited by the number of bits that can be reliably delivered to a UE at the cell edge, and is typically determined through simulations in RAN1. In the following, we evaluate the contents and the size of the different RRC messages to be able to provide a size estimate to RAN1 for their simulations.

RRC connection request

The RRC connection request message is used when transitioning from idle to connected mode. Here we assume that the 5G-S-TMSI used as UE identity has the same size as S-TMSI in LTE, i.e., 40 bits. Also note that the establishment cause is increased from 3 to 6 bits compared to LTE. This is because we assume that there will be a direct mapping between the access categories defined as part of the NR access control scheme and establishment causes, see [2].

| RRCConnectionRequest |  |
| --- | --- |
| IE | Size (bits) |
| ue-Identity | 40 |
| establishmentCause | 6 |
| Total | 46 |

RRC connection resume request

The RRC connection resume request message is sent when transitioning from inactive to connected mode. Both the UE context identifier, contained in the UE identity field, and the short-MAC-I are assumed to have the same size as in LTE Rel-13 suspend/resume, i.e. 40 and 16 bits respectively, see [7] and [8]. We also expect the resume cause to have the same size as the establishment cause in the RRC connection setup request. In addition, two new fields have been added compared to LTE:

- The plmn-Identity contains an index to the PLMN list in SIB1 and indicates the PLMN that allocated the resume identity. It is included to avoid coordination of gNB IDs (which is part of the resume identity) between PLMNs sharing the same RAN, see [3]. The index is assumed to be 4 bits allowing up to 16 PLMNs to be referenced.

- The resumedSlice indicates which of the up to eight 8 slices that the UE wishes to resume. This field is included to enable different treatment and early prioritization of network slices, see [4]. If several slices are resumed then the UE indicates the one with highest priority.  This field may be possible to omit if a buffer status report (BSR) is included on MAC level, in which case the slice instead can be determined from the the Logical Channel Group (LCG) associated with the BSR. (The LCG maps to a DRB which in turn maps to a slice).

| RRC connection resume request |  |
| --- | --- |
| IE | Size (bits) |
| ue-Identity | 40 |
| resumeCause | 6 |
| shortMac-I | 16 |
| plmn-Identity* | 4 |
| resumedSlice* | 3 |
| Total | 69 |

RRC connection re-establishment request

The RRC connection re-establishment request message is sent to recover the RRC connection after radio link failure, for example after a failed handover attempt. No changes to the content are expected compared to LTE. Note that the UE identity in this case is comprised of a 9 bit PCI and a 16 bit C-RNTI.

| RRC connection re-establishment request |  |
| --- | --- |
| IE | Size (bits) |
| ue-Identity | 25 |
| reestablishmentCause | 2 |
| shortMac-I | 16 |
| Total | 43 |

The minimum MSG3 size that needs to be supported is determined by the largest of the three messages above, which is the RRC connection resume request.

RRC connection resume request is largest among the initial RRC messages and hence determines the minimum size of MSG3 (i.e., the size that must be supported in all deployments of NR).

So far in the size calculations we have only considered the actual content and ignored the overhead resulting from the structure of the RRC message, such as the choice bits required to indicate the message type or the bits used to signal the presence of optional fields. Since the RRC message structure largely depends on the outcome of the message harmonization discussion (see [5]), it is difficult to calculate the exact message size now. However, based on the experience from LTE, we can expect the overhead to be small (the overhead is only 5 bits for the RRC connection resume request message in LTE). Also note that the RRC message PDU must be byte aligned which requires padding bits to be inserted to the closest byte. Taking this additional overhead into account, the total size of the RRC PDU for the RRC connection resume request message is around 80 bits, or 10 bytes.

The exact size of the initial RRC messages depends on outcome of the message harmonization discussion but we can approximate the size fairly well.

To calculate the total size of MSG3 we will also need to account for the overhead added by the MAC layer. The minimal possible MAC PDU is shown below. Note that no padding is needed if we assume the MAC PDU fits exactly into the UL grant/transport block. Also note that, unlike in LTE, the MAC SDU subheader always contains a length field, which means that its size is 2 bytes instead of 1 byte.

However, if we want to be able to schedule the UE as quickly as possible at RRC connection resumption, we should also make sure a (short/truncated) BSR and Power Headroom Report (PHR) is included in MSG3. The latter indicates how much transmission power that the UE has left to use in addition to the power being used by the current transmission, and is used to determine the amount of UL bandwidth to assign to the UE. Until the first PHR is received the UE is assumed to use its maximum transmit power and the gNB may therefore assign a smaller UL bandwidth than what is actually possible, which in turn reduces the UL data transfer speed. The resulting MAC PDU with both BSR and PHR included looks as follows.

Based on the above we conclude:

Taking MAC overhead into account, the minimum size of MSG3 (i.e. the size that must be supported in all deployments of NR) is around 12 or 16 bytes, depending on if a BSR+PHR is included or not.

It is proposed to send an LS to RAN1 asking them whether this size can be supported. A draft LS can be found in [6].

Send an LS to RAN1 asking them if a MSG3 transport block size of 12 or 16 bytes can be supported.

# Conclusion

In this contribution we discussed the size of MSG3 in NR. In section 2 we made the following observations:

Observation 1	RRC connection resume request is largest among the initial RRC messages and hence determines the minimum size of MSG3 (i.e., the size that must be supported in all deployments of NR).

Observation 2	The exact size of the initial RRC messages depends on outcome of the message harmonization discussion but we can approximate the size fairly well.

Observation 3	Taking MAC overhead into account, the minimum size of MSG3 (i.e. the size that must be supported in all deployments of NR) is around 12 or 16 bytes, depending on if a BSR+PHR is included or not.

Based on the discussion in section 2 we propose the following:

Proposal 1	Send an LS to RAN1 asking them if a MSG3 transport block size of 12 or 16 bytes can be supported.

# References

R2-1708799, Email discussion report on [98#30][NR] RRC Connection Control, Intel

R2-1710480, Establishment causes for NR, Ericsson

R2-1710473, RAN Sharing and identifier aspects in NR, Ericsson

R2-1710195, Signalling aspects of network slicing, Ericsson

R2-1710826, Harmonizing RRC Connection control messages and procedures, Ericsson

R2-1710200, Draft LS on MSG3 size, Ericsson

R2-1710831, UE context discussion, Ericsson

R2-1710835, Security for RRCConnectionResumeRequest message, Ericsson