3GPP TSG-RAN WG2 Meeting #99bis                                                            	R2-1710285

Prague, Czech Republic, 9 - 13 Oct 2017                              Resubmission of R2-1707906

Source:	CATT

Title:	Indexed SI in NR

Agenda Item:	10.4.1.6.3

Document for:	Discussion and Decision

Introduction

In RAN2 #98 meeting, the issue of the area ID and value tag for indexed SI has been discussed [1][2], and the following agreements were made:

Agreements

There will be at least a value tag and area ID

-	value tag is associated to each SIB

-	value tag can be valid in only one cell or when combined with an area ID to be valid in more than one cell.

FFS whether the area ID and valuetag is separately signalled or as a single identifier

FFS whether the area ID is associated to each SIB/ SI message or associated to a group of SIBs/ SI messages or all SIBs/ SI messages.

In this contribution we analyze the above FFS issues.

Discussion

Analysis of the area ID and value tag

- Signalling overhead

It has been agreed that value tag is associated to each SIB. If value tag and area ID are encoded into a single identifier, it means area ID should be signalled per SIB. If the number of SIBs/SI messages is large, then the total length of the area IDs in the system information will be large and the signalling overhead is high. If the value tag and area ID are separately signalled, it is possible for the network to associate a group of SIBs/SI messages to one area ID to reduce the broadcast overhead.

Observation1: When the area ID and value tag are separately signalled, the signalling overhead is low, for the area ID can be associated to a group of SIBs/SI messages.

- Cell specific system information

We assume that area ID is only necessary for the SIBs which may valid in more than one cell, hence separate area ID and value tag seems more suitable to indicate which SIBs are cell-specific SI. With this information into account, UEs may apply different storage policy for cell-specific SI and cross-cell-valid SI, e.g. different validity time applied for cross-cell-valid SI.

Observation2: Separate area ID and value tag is more suitable to indicate which SIBs are cell-specific SI, assuming area ID is only associated with the SIBs which may valid in more than one cell.

- Further optimization

Some papers [3][4] suggest to include the value tag of updated SIBs in the paging message triggered by system modification notification mechanism. In this way, the UEs which have stored the SIBs with the above value tag are not required to re-read the corresponding SIBs. Obviously, shorter value tag (separate area ID and value tag) will lead to less paging overhead than longer value tag (a single identifier).  Therefore, separate area ID and value tag may provide possibility for further optimization, if required, allowing for forward compatibility.

- Observation3: Separate area ID and value tag provide forward compatibility for further optimisation if required.Network deployment flexibility

When the area ID and value tag are signalled as a single identifier, no matter the version of SIBs is updated or the area of SIBs is re-associated, the whole identifier needs to be changed. However, how many bits allocated to the area ID and value tag is left to network implementation.

Observation4: When the area ID and value tag are signalled as a single identifier, it’s flexible for the network to adjust the sizes of area ID and value tag.

Comparison of the solutions

The above analysis is summarized in Table1.

Table 1: Comparison of the solutions

|  | area ID and value tag are separately signalled | area ID and value tag are signalled as a single identifier |
| --- | --- | --- |
| Signalling overhead | low | high |
| Cell specific system information | area ID is only associated with the SIBs which may valid in more than one cell. | can not indicate which SIBs are cell-specific |
| Further optimization | Easy for optimization, e.g. system information notification enhancement | Hard for optimization |
| Network deployment flexibility | the sizes of area ID and value tag are fixed | flexible for the network to adjust the sizes of area ID and value tag |

According to the analysis above, signaling of value tag and area ID separately leads more benefit than signalling as a single identifier, we propose the area ID and value tag are separately signalled.

Proposal 1: The area ID and value tag are separately signalled.

As discussed above in observation1, associate one area ID to each SIB leads to a lot of broadcast overhead. However, associate only one area ID to all SIBs will limit the network deployment flexibility.

Hence, we prefer to associate one area ID to a group of SIBs, which is very flexible. The network can allocate each SIB in one group or all the SIBs in one group, or several SIBs in one group.

Proposal 2: Associate one area ID to a group of SIBs.

Conclusion

According to the analysis in section 2, we obtain some observations of the area ID and value tag and propose:

Observation1: When the area ID and value tag are separately signalled, the signalling overhead is low, for the area ID can be associated to a group of SIBs/SI messages.

Observation2: Separate area ID and value tag is more suitable to indicate which SIBs are cell-specific SI, assuming area ID is only associated with the SIBs which may valid in more than one cell.

Observation3: Separate area ID and value tag is more convenient for further optimization.

Observation4: When the area ID and value tag are signalled as a single identifier, it’s flexible for the network to adjust the sizes of area ID and value tag.

Proposal 1: The area ID and value tag are separately signalled.

Proposal 2: Associate one area ID to a group of SIBs.

Reference

- R2-1705584	 On SI Index	MediaTek Inc.

- R2-1706133	 Offline discussion summary for Area ID and valuetag

- R2-1704062 System Information Update in NR Samsung

- R2-1705176 SI Update procedure Huawei, HiSilicon