3GPP TSG-RAN WG2 #99bis                                                                                R2-1710300

Prague, Czech Republic, 9 - 13 Oct 2017 	       Revision of R2-1707917

Source:	CATT

Title:	Minimum Size of MAC PDU including Data

Agenda Item:	10.3.1.7

Document for:	Discussion and Decision

# Introduction

In order to instruct UE to assemble the MAC PDU effectively, RAN2 sets the minimum size of MAC PDU including data.

From TS 36.321[1] :

- The UE shall also follow the rules below during the scheduling procedures above: - the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity; - if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible; - the UE should maximise the transmission of data. - if the MAC entity is given an UL grant size that is equal to or larger than 4 bytes while having data available for transmission, the MAC entity shall not transmit only padding BSR and/or padding (unless the UL grant size is less than 7 bytes and an AMD PDU segment needs to be transmitted); - for transmissions on serving cells operating according to Frame Structure Type 3, the MAC entity shall only consider logical channels for which laa-Allowed has been configured.

From TS 38.321[2] :

The UE shall also follow the rules below during the scheduling procedures above: - the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity; - if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible; - the UE should maximise the transmission of data; - if the MAC entity is given an UL grant size that is equal to or larger than [X] bytes while having data available for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

In this contribution, we discuss and try to decide the X bytes in LCP based on RAN2 progress.

# Discussion

In LTE, we calculate the minimum size of MAC PDU including not only padding/padding BSR based on the minimum headers of L2 sub-layers.

For the regulation of “UL grant size that is equal to or larger than 4 bytes”, there could be a 4-byte MAC PDU including 1 byte data.

4 bytes = 1 byte PDCP header +1 byte RLC header +1 byte MAC header + 1 byte data (applicable to UMD only)

Figure 1 4-byte MAC PDU in LTE (for UMD)

For the regulation of “unless the UL grant size is less than 7 bytes and an AMD PDU segment needs to be transmitted)”, there could be a 8- bytes MAC PDU including 1 byte data.

7 bytes = 2-byte AMD PDCP header + 4-byte AMD PDU segment RLC header +1-byte MAC header (applicable to AMD PDU segment)

Figure 2 8-bytes MAC PDU in LTE (for AMD PDU segment)

Actually, we think the rule of “X bytes” is not a mandatory rule but guidance. For example, when the UL grant size is 4 bytes and UE only has an AMD PDU to send, UE can’t assemble a MAC PDU including data because in this case, there are at least 2-byte PDCP header, 2-byte RLC header, and 1-byte MAC header. Moreover, with different lengths of PDCP SN and RLC SN configured by NW, the sum size of the L2 sub-layers headers can be far more than 4 or 7 bytes.

With above observations, we should only consider the minimum overhead of each L2 sub-layer header when calculating the “X bytes” in NR.

The agreements on each L2 sub-layer header format and corresponding analysis are shown below.

SDAP sub-layer:

The SDAP header can be absent. That is, the SDAP header could be 0 byte.

Agreements in AH#2:

- Whether a SDAP header is present or not is configured by RRC per DRB

Agreements in RAN2#99

1.	RAN2 aims at designing a 1 byte SDAP header.  Whether the QFI is 6 bit or 7 bits is FFS.

2.	If configured, SDAP header size for a DRB is static (assuming 1 byte header).  The QFI will always be present.

3. 	No SN will be introduced in SDAP

PDCP sub-layer:

Considering 1 bit D/C filed, 3 bits PDU type field, 12 bits PDCP SN, the minimum PDCP header should be 2 bytes.

Agreements in RAN2#98

- the D/C field is not present in the PDCP data PDU format for SRB.

- the D/C field is present in the PDCP data PDU format for DRB.

- the P field is not present in the PDCP data PDU format for SRB and as a baseline for DRB as well.

- the D/C field is present in all PDCP control PDUs.

- the PDU type field is present in all PDCP control PDUs.

- the PDU type field is 3 bits.

- RAN2 assumes that out-of-order/duplicated reception of ROHC feedback is not an issue to be resolved to RAN2.

- Out-of-order/duplicated reception of PDCP status report is not an issue to be resolved in RAN2.

- As a baseline, PDCP SN number will not be added to the PDCP Control PDUs

- 12 and 18 bit PDCP SN is used for NR

RLC sub-layer:

Because the RLC UM SN for the complete SDU is absent and SI field is included in the RLC header, the minimum RLC header should be 1 byte.

Agreements in RAN2 AH#2

- 12 and 18 bit RLC SN is used for RLC AM NR.  SN for RLC UM is FFS

- Only 16 bit SO is used in NR for both AM and UM.

- NACK_RANGE size is FFS – depends on byte alignment and full RLC STATUS format

1.	RLC UM without SN for the complete SDU is selected.   FFS which options is selected

2.	SI field is included in RLC UM header to differentiate complete RLC SDU, the first SDU segment, the middle SDU segment, and the last SDU segment.

-	The header of unsegmented SDU contains only SI field.

-	The header of first segment contains only SI field and SN.

-	The header of middle and last segment contains SI field, SN, and SO.

Agreements in RAN2#99

- SN RLC UM size of 6 bits and 12 bits

MAC sub-layer:

The size of LCID is 6 bits and the L field is present for every MAC SDU, then the minimum MAC header is 2 bytes.

Agreements in RAN2#98

1.	The E field is not present in NR MAC sub-header.

2.	F2 fields are not present in NR MAC sub-header.

3. 	Variable L fields size with two values will be supported.  The F field is included.  Size is FFS

4.	The size of LCID field is 6 bits

5.	The L field is not present for the fixed-size MAC CE

6.	The L field is present for variable-size MAC CE

7. 	The L field is present for every MAC SDU. FFS if no L field is present for padding

Agreements in RAN2#99

-	8 and 16 bits length field is supported with the single F bit in the second bit.  For fixed size MAC CE there is a two R fields

Therefore, the minimum size of MAC PDU including data should be:

X = 0-byte SDAP header + 2-byte PDCP header + 1-byte RLC header + 2-byte MAC header + 1-byte data = 6 bytes

Proposal: The X bytes in LCP should be 6 bytes.

# Conclusion

In this contribution, we analyze the minimum size of MAC PDU including data based on the latest RAN2 progress on NR L2 sublayers headers and give the below proposal:

Proposal: The X bytes in LCP should be 6 bytes.

# Reference

- 3GPP TS 36.321, E-UTRAN Medium Access Control (MAC) protocol specification, V14.2.0

- 3GPP TS 38.321, NR Medium Access Control (MAC) protocol specification, V0.2.0

# Text proposal to 38.321

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical channel prioritization

The Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

The MAC entity shall maintain a variable Bj for each logical channel j. Bj shall be initialized to zero when the related logical channel is established, and incremented by the product PBR × NR-UNIT for each NR-UNIT, where PBR is Prioritized Bit Rate of logical channel j. However, the value of Bj can never exceed the bucket size and if the value of Bj is larger than the bucket size of logical channel j, it shall be set to the bucket size. The bucket size of a logical channel is equal to PBR × BSD. It is up to UE implementation when to update Bj.

Editor's note: (again) NR-UNIT is used. Editor thinks consistent unit (i.e. NR-UNIT) throughout the MAC would be desirable rather than to use e.g. one millisecond as proposed during the meeting.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels in the following steps:

-	Step 1: Relevant logical channels for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to "infinity", the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

Editor's note: compared to LTE, 'All the logical channels' is replaced with 'Relevant logical channels for the UL grant'.

-	Step 2: the MAC entity shall decrement Bj by the total size of MAC SDUs served to logical channel j in Step 1;

NOTE:	The value of Bj can be negative.

-	Step 3: if any resources remain, all the relevant logical channels are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

Editor's note: the wording 'relevant' needs to be further clarified after having concrete RAN2 agreements (by considering e.g. numerology/TTI, packet duplication, transmission profile, etc.).

The UE shall also follow the rules below during the scheduling procedures above:

- 	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given an UL grant size that is equal to or larger than 6 bytes while having data available for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs, the MAC entity shall not generate a MAC PDU for the HARQ entity in the following cases:

-	in case the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI.

Editor's note:  The term C-RNTI is tentatively used to capture the agreement. Can be changed later.

Editor's note: The condition 'If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs' comes from LTE, and can be discussed later. Also aperiodic CSI condition from LTE is missing.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	…

Editor's note: The detailed priority order (e.g. CCCH/C-RNTI MAC CE -> BSR MAC CE -> PHR MAC CE, …) is not discussed yet, so leave it empty for the time being. Will be filled out later.

Editor's note: The name of RRC parameters priority, prioritisedBitRate, bucketSizeDuration, and skipUplinkTxDynamic are tentatively used to capture the agreement, but can be changed later.