3GPP TSG-RAN WG2 Meeting #99bis	R2-1710362

Prague, Czech Republic, 9th – 13th October 2017                              Resubmission of R2-1708863

Agenda Item:	10.4.2.2

Source: 	Fujitsu

Title:	Inter-RAT measurement for EN-DC

Document for:	Decision

- Introduction

For LTE, inter-RAT NR measurements for NSA operation are discussed in this contribution, including the following FFSs after the discussion of connected mode mobility between NR and E-UTRA at RAN2 NR Ad-hoc#2:

- Whether a measurement trigger event is needed in LTE which is triggered when NR PSCell becomes worse than an absolute threshold.

- A measurement trigger event is needed in LTE which is triggered when NR PSCell becomes worse than absolute threshold1 and NR neighbour cell on a different carrier becomes better than absolute threshold2.

- Discussion

Regarding LTE-NR DC procedure and mobility between LTE and NR, RAN2 has made some agreements [1] [2] [3] [4]. Based on these agreements, it is observed that the MN can initiate SN addition, SN release and inter-frequency SN change for EN-DC. And for MN initiated SN addition, the LTE inter-RAT measurement trigger event B1 for NR will be supported. Then the question is whether there is a need for MN to initiate the SN release/change based on RRM.

For SN release this may be needed if:

- It is not possible for the UE to be served by 2 nodes, e.g. the quality of the connection of current SN becomes poor or the load of current SN becomes very high; or

- There is no need for the UE to be served by 2 nodes, e.g. the throughput requirement for the UE can be met by the MN.

For the SN release due to the poor channel quality, this can be handled by the SN initiated SN release as the SN knows the radio condition. This means that the MN will initiate the SN release based on throughput requirement and load situation and will not initiate the SN release procedure based on the SN’s quality.

Some companies may have concerns on the case where the measurement report cannot be received at the SN and the SN release cannot be initiated by the SN to the MN. So these companies may prefer to support MN initiated SN release based on RRM. However, in our opinion, the UE can transfer the measurement report successfully only if the reporting configuration is reasonable. In rare cases, the measurement reporting failure occurs due to inappropriate configuration or sudden channel quality deterioration. For the former case, the expected behaviour is that the SN is aware of the inappropriate configuration and the reporting configuration can be corrected rather than measurement report sent to the MN and then the SN release procedure initiated by the MN. Therefore, SCG failure reporting procedure should be used in case the measurement report cannot be sent to the SN successfully no matter what the reason is.

From RAN2 efforts point of view, MN initiated SN release based RRM is not preferred. If supported, RAN2 would need to work on the down-selection of all potential measurement trigger events. Then, after the event is identified, the coordination on the reporting configuration between the MN and the SN should be further studied in order to guarantee the MN can receive the NR measurement report in time.

Besides, SCG split SRB (i.e. split SRB3), which can be used to transfer the NR measurement report configured by the SN from the UE to the SN via the MN in case the SN quality becomes poor, has been proposed but  RAN2 has agreed that this will not be supported in Rel-15.

Based on the above analysis, it can be observed that the MN will not initiate the SN release based on RRM and there is no need to introduce a new measurement trigger event.

Proposal 1: For SN release initiated by MN, there is no need to introduce a new measurement trigger event.

For SN change, this is comprised of 2 procedures, i.e. SN release and SN addition. With the similar reasons to SN release, we think the MN will not initiate the SN change based on RRM and there is no need to introduce a new measurement event.

Proposal 2: For SN change initiated by MN, there is no need to introduce a new measurement trigger event.

- Summary

In this contribution, the inter-RAT NR measurements to be added in LTE for NSA operation are discussed and the following proposals are made:

Proposal 1: For SN release initiated by MN, there is no need to introduce a new measurement trigger event.

Proposal 2: For SN change initiated by MN, there is no need to introduce a new measurement trigger event.

# References

- RAN2-NR AH-Spokane-Chairman-Notes

- RAN2#97-Athens-Chairman-Notes

- RAN2-97bis-Spokane-Chairman-Notes

- RAN2-98-Hangzhou-Chairman-Notes