3GPP TSG-RAN WG2 Meeting #99bis								R2-1710400

Prague, Czech Republic, 9 - 13 October 2017

Agenda item:	9.2

Source: 	Huawei, HiSilicon

Title: 	Handling of MAC CE Priority Handling in sTTI

Document for:	Discussion and Decision

- Introduction

In RAN2 #99 meeting, the agreement on MAC control element (CE) for sTTI was achieved[1]:

=>	There is no TTI length restriction for MAC CE

Based on the agreement, the MAC CE can be transmitted on either sTTI grant or legacy TTI grant. Even, how to handle the priority between the MAC CE and data of LCH configured sTTI has been discussed in previous meeting [2] [3] [4] not reach a conclusion, which is focused on in this document.

2	Discussion

In current specification, there are 4 types of MAC CE defined for uplink:

UE identification related: C-RNTI;

Data volume report related: BSR, Pading BSR, Sidelink BSR, Sidelink pading BSR and DPR;

Power Headroom report related: PHR, extended PHR and DC PHR;

Signaling Related: SPS confirmation and recommended bit rate query.

Since sTTI is not applied in sidelink and NB-IoT, the left uplink MAC CEs are C-RNTI, BSR, padding BSR, PHR, extended PHR, DC PHR, and SPS confirmation. The relative priority amongst MAC CEs and the LCH data is:

C-RNTI and data from UL-CCCH > SRS confirmation > BSR > data (except for data from UL-CCCH) > padding BSR.

However, when sTTI is used, how to handle the priority between MAC CE and data needs to be further studied.

Typically, LCHs configured with just sTTI have strict requirements on latency. According to the current specification, the data from LCHs configured with sTTI (sTTI data) has lower priority than the most of the MAC CEs. If the grant for sTTI data is rather small, only the higher priority data can be assembled in the grant, the latency of the sTTI data can’t be guaranteed. In NR, there is some discussion to cope with this issue as well. The possible solutions are provided and can be divided into three alternatives.

Alternative A) Not to introduce enhancement for the priorities for the sTTI data

In this alternative, there will be no specification impact introduced. And the priority of the LCH configured with sTTI will be multiplexed with lower priority than the MAC CE. When the UE has a small grant, the MAC CE is assembled in the grant with high priority. Consequently, it may not meet the latency requirement for the UE.

Alternative B) Using default priorities between and amongst MAC CEs and logical channel

In this alternative, default priorities list or table are specified. On a more detailed level, the relative priority can be specified in the following, C-RNTI and data from UL-CCCH > sTTI data > SRS confirmation > BSR > data (except for data from either UL-CCCH or sTTI data) > padding BSR. On the other hand, if there is no LCH configured with sTTI, the current priority order can be reused.

However, in last meeting, it was agreed that there is no new BSR format is introduced for sTTI. In line with the agreement and the priority order, the sTTI data will have higher priority than the BSR containing the Buffer status for sTTI in this alternative. This may be further studied in the future.

Alternative C) the relative priorities between and amongst MAC CEs and the logical channels are configurable by the network.

In this alternative, default priorities list for MAC PDU are also provided like alternative 1, on the case when there is no signalling related the priorities is sent to the UE. If a configuration of the relative priorities is received by the UE, the specific priority of the LCH, especially for the sTTI data can be set higher than that of the MAC CE. This can guarantee the low latency for the sTTI data, and is more flexible.

The table below tries to highlight the pros and cons of the different options.

Table 1 Pros and Cons Comparison between Different Alternatives

| Alternative | Pros | Cons |
| --- | --- | --- |
| A | With low complexity and no impacts on the specification. | Can’t guarantee the priority of the sTTI data when small grant arriving; |
| B | Complexity: Middle; small impact on the specification; | BSR including buffer status of LCH configured with sTTI will be assembled with lower priority than sTTI data. |
| C | Flexible | BSR including buffer status of LCH configured with sTTI will be assembled with lower priority than sTTI data. Introduce impacts on the specification. Extra signalling overhead. |

As illustrated above, alternative B and alternative C guarantee the sTTI data with higher priority than the legacy MAC CE in the case when the grant is too small that only sTTI data or MAC CE can be multiplexed into the grant. The gain is relative small. Hence, it is proposed that:

Proposal: Not to introduce enhancement of the priorities for the sTTI data.

3	Conclusion

In this document, we analyse the relative priority between sTTI data and MAC CE. And we kindly propose that:

Proposal: Not to introduce enhancement of the priorities for the sTTI data.

4	References

- R2-xxxxxx, Chairman notes, RAN2, #99 meeting.

- R2-1703131, LCP procedure for sTTI, RAN2 # 97 meeting, CATT;

- R2-1703641, Multiplexing and LCP procedure of Different TTIs, RAN2 #97bis, Huawei

- R2-1709646, MAC CE Handling under PUSCH and sPUSCH Collision, RAN2 #99 meeting, Ericcion;