# 3GPP TSG-RAN WG2 Meeting #99bis 	R2-1710468

# Prague, Czech Republic, 9th – 13th Oct 2017 	Resubmisison of R2-1708078

Agenda item:  10.4.4.5

Source: 	Huawei, HiSilicon

Title: 	Speed dependent mobility in Idle and Inactive mode

Document for: Discussion and decision

# Introduction

In this paper, we discuss the issue of speed dependent mobility for NR in idle mode. Since NR supports INACTIVE state in addition to IDLE and CONNECTED state, speed dependent scaling can also apply to INACTIVE state.

# Discussion

## Idle speed dependent mobility in LTE

In LTE, speed dependent mobility is used to adjust parameters (e.g. Treselection, q-Hyst) for cell reselection based on the UE’s mobility state estimation. For instance, in order to avoid high speed UE losing the coverage of serving cell, it’s reasonable to apply a shorter t-Reselection or smaller q-Hyst value so as to reselect to a target cell more quickly. Cell reselection is performed by the UE without explicit network control, so speed dependent reselection parameters need to be broadcasted to UE.

The mobility state dependent scaling of cell reselection for RRC_IDLE state is stated in [2] and summarised below:

- Besides the common cell reselection parameters, the speed state reselection parameters (e.g. evaluation time, thresholds and scaling factors for medium and high speed state) are broadcasted to UE.

- UE compares the number of cell reselections during time period TCRmax with the thresholds (NCR_M and NCR_H) to distinguish the UE mobility state.

- UE applies speed dependent scaling (sf-Medium, sf-High) factor for cell-reselection parameters (Qhyst, Treselection) according to the detected UE mobility state. The cell-reselection parameters are applied with high or medium scaling factors respectively if UE is in the high or medium mobility state. For example, if UE in high mobility state, UE shall apply the scaled parameters to perform cell reselection more quickly.

For NR, the basic mechanism of speed dependent mobility of LTE may apply to NR also to improve NR mobility performance.

Proposal 1: Reuse the LTE mechanism of MSE (i.e. adjust Qhyst and Treselection for cell reselection) as the baseline for RRC_IDLE state.

## Speed dependent consideration for different NR RRC states

In NR, there is a discussion on the new RRC state (RRC_INACTIVE) on top of the IDLE state and CONNECTED state. According to the RAN2 discussion, the characteristics of RRC_INACTIVE can be summarized as below:

-	Cell re-selection mobility;

-	CN – NR RAN connection (both C/U-planes) has been established for UE;

-	The UE AS context is stored in at least one gNB and the UE;

-	Notification is initiated by NR RAN;

-	RAN-based notification area is managed by NR RAN;

-	NR RAN knows the RAN-based notification area which the UE belongs to;

When UE is in INACTIVE state, UE can only have cell re-reselection mobility or receive notifications from NR RAN, so a similar MSE function defined in LTE with speed dependent scaling for cell-reselection can be applied for UE in INACTIVE state.

Proposal 2: 	Speed dependent scaling of cell reselection shall be applied when UE is in NR INACTIVE state.

For a UE transiting from INACTIVE to IDLE, it may include two cases. One case is that NR RAN could configure UE transition from INACTIVE to IDLE if it predicts that a UE has no packet arrival for a long period or some error cases (e.g. rejecting from network). The other case is that UE is out of coverage and the periodical RAN based notification area update timer expires, the UE will not be able to send the notification area update request to NR RAN. Then NR RAN may release NG1 connection if the expected periodical notification area update request is not received. To keep the state consistent with NR RAN, the UE may need to enter to IDLE state autonomously. For the cases of state transition from INACTIVE to IDLE, UE needs to count the sum of cell reselections during evaluation time period for the accurate mobility state estimation across the state transition (i.e. not reset the count or evaluation time period when the state change takes place).

Proposal 3: 	UE needs to count the sum of cell reselections in IDLE and INACTIVE state during evaluation time period for accurate MSE in case of transition from INACTIVE to IDLE.

# Conclusion

In this document, we discussed the mobility state estimation in idle and inactive state of NR.

Proposal 1: 	Reuse the LTE mechanism of MSE (i.e. adjust Qhyst and Treselection for cell reselection) as the baseline for RRC_IDLE state.

Proposal 2: 	Speed dependent scaling of cell reselection shall be applied when UE is INACTIVE state.

Proposal 3: 	UE needs to count the sum of cell reselections in IDLE and INACTIVE state during evaluation time period for accurate MSE in case of transition from INACTIVE to IDLE.

# References

[1] 3GPP TR 36.839: "Evolved Universal Terrestrial Radio Access (E-UTRA);Mobility enhancements in heterogeneous networks".

[2] 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Procedures in Idle Mode".