3GPP TSG-RAN WG2 #99-bis on NR	Tdoc R2-1710841

Prague, Czech, 9th-13th October 2017

Agenda Item:	10.4.1.4.8

Source:	Ericsson

Title:	Mobility states and speed based parameter scaling in NR

Document for:	Discussion, Decision

# Introduction

Mobility related measurements and the corresponding configuration for measuring, report triggering and reporting are being considered for NR in RAN2 discussions. The corresponding discussions in LTE also included, amongst other things, speed dependent scaling of measurement related parameters. In the recent email discussion [3], the support for speed based scaling was discussed but there was no clear preference as to whether to support this feature in NR or not.

In this contribution, we discuss the extent of usage of such a feature in LTE and also, discuss the need for such a feature in NR.

# Discussion

## Speed dependant scaling of measurement parameters

The handover related parameters that are optimized by a SON algorithm, like Mobility Robustness Optimization (MRO),  includes, amongst other parameters, event specific offsets (e.g., A3 offset), cell individual offset (CIO) and time-to-trigger (TTT). Of these parameters, A3 offset and TTT are neighbour cell agnostic and CIO can be specified specifically towards each neighbour.

- In LTE, A3 offset, Cell individual offset (CIO), hysteresis and Time-to-trigger (TTT) are used to tune the handover border using a SON algorithm like MRO.

- In LTE, A3 offset, hysteresis and TTT are configured independent of the neighbour cell and only CIO can be configured specifically towards a neighbour cell.

In LTE, a RRC_CONNECTED mode UE could scale its mobility parameter TTT depending on the speed state as estimated by the UE and details of this scaling is mentioned in RRC specifications [1]. The scaling factors to be used in LTE are 0.25, 0.5, 0.75 and 1. The thought process behind having this feature in LTE was to be able to receive the measurement report from the UE early enough so that the handover command can be sent successfully to the UE.

- In LTE, only TTT can be scaled depending on the mobility state as estimated by the UE.

#### 5.5.6.2	Speed dependant scaling of measurement related parameters

The UE shall adjust the value of the following parameter configured by the E-UTRAN depending on the UE speed: timeToTrigger. The UE shall apply 3 different levels, which are selected as follows:

The UE shall:

1>	perform mobility state detection using the mobility state detection as specified in TS 36.304 [4] with the following modifications:

2>	counting handovers instead of cell reselections;

2>	applying the parameter applicable for RRC_CONNECTED as included in speedStatePars within VarMeasConfig;

1>	if high mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-High within VarMeasConfig;

1>	else if medium mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-Medium within VarMeasConfig;

1>	else:

2>	no scaling is applied;

However, in the RRC_CONNECTED mode the network is aware of the UE’s mobility states and it could configure the UE accordingly with event specific offsets (like A3 offset), TTT and CIO values in order to best optimize the overall handover border compared to scaling only TTT values by the UE itself [2]. The mobility state estimation could be done already at connection establishment based on the UE mobility history information as introduced in Rel-12 in LTE or based on Doppler,  position information etc.. In release 12 of LTE, the UE could be asked to report its mobility history information using mobilityHistoryReportReq IE.

1>	if mobilityHistoryReportReq is set to true:

2>	include the mobilityHistoryReport and set it to include entries from VarMobilityHistoryReport;

2>	include in the mobilityHistoryReport an entry for the current cell, possibly after removing the oldest entry if required, and set its fields as follows:

3>	set visitedCellId to the global cell identity of the current cell:

3>	set field timeSpent to the time spent in the current cell;

When the network uses these detailed information to group the UEs based on their mobility state, and configures the UE with relevant measurement reporting configuration (event thresholds, hyst etc.), it was found to be more useful than just the scaling of one parameter in LTE for the RRC_CONNECTED UEs.

- In LTE, network is aware of the UE mobility state (either via considering the UE handover history or by measuring the Doppler or position information etc.) for the RRC_CONNECTED mode UE.

- In LTE, network configuration of event specific offsets, hysteresis, TTT, CIO values depending on the UE’s estimated speed gives better control of handover border tuning.

- In NR, as in LTE, support mobility history information reporting.

## Scaling of measurement related parameters in NR

In NR, it has already been agreed that event A1-A6 are supported for both SS block based measurements and CSI-RS based measurements. Though it has not been agreed yet, it is reasonable to assume that the HO related parameters could consist of at least event specific offsets like A3 offset, Cell individual offsets, Time-to-trigger and (in addition to LTE) value of ‘N’ to be used for cell quality derivation.

Based on the learnings from LTE, it was found to be useful to adjust not just the TTT but also event specific offsets, hysteresis, cell individual offsets based on the UE speed groups as categorized by the network. Therefore, it is proposed to not have TTT scaling based on its mobility state in NR and rather include flexible configuration of event specific offsets, hysteresis, CIO values and TTT by the network based on the grouping as carried out by the network.

- In NR, as in LTE, allow the network configuration of event specific offsets, hysteresis, TTT, CIO values via measurement reporting configuration.

- In NR, speed state based scaling of Time-to-trigger (TTT) is not supported

In LTE, it was observed that the handover borders are not smooth i.e., there can be regions in the handover borders where the SINR degradation in more gradual and in some other regions SINR degradation can be more severe due the propagation difference in different regions. This observation is relevant for the UE having the same mobility state (either medium mobility state or high mobility state or slow mobility state). This behaviour is expected to be relevant for NR as well, if not more severe due to the beamforming nature of NR.

In order to resolve this, a scaling of HO related parameters needs to be provided which is dependent of the UE’s perceived SINR at the cell border. It is to be noted that when the UE is moving fast, the SINR change will be faster at the cell border, thus automatically scaling the relevant HO parameters accordingly. To support SINR based scaling, a SINR based states could be defined e.g., good SINR, medium SINR, bad SINR etc. The state could be based not just based on the instantaneous SINR but also the rate of change of SINR etc.

- Handover borders between two cells are not smooth meaning there can be regions in the handover borders where the SINR degradation in more gradual and in some other regions SINR degradation can be more severe due the propagation difference in different regions.

- In NR, introduce SINR based scaling of the handover parameters.

In LTE, the speed based scaling was supported, however, there was no possibility to have the scaling of handover parameters per events. This could be useful as the purpose of having different events is different and only in some events the scaling of parameters could be useful. For example, the event A1 could be used to trigger carrier aggregation whereas event A3 could be used for intra-frequency handover. Having the scaling of handover parameters could be beneficial only for intra-frequency handover situations than for carrier aggregation set-up. Therefore, in NR, the handover parameter scaling should be configurable for each events. In order to do so, for each event configured in the reportConfig, there could be an additional flag to indicate if the parameter scaling is applicable or not.

- In NR, whether the scaling of the handover parameters is applicable or not is configured in the reportConfig.

# Conclusion

In the previous section, the following has been observed:

- In LTE, A3 offset, Cell individual offset (CIO), hysteresis and Time-to-trigger (TTT) are used to tune the handover border using a SON algorithm like MRO.

- In LTE, A3 offset, hysteresis and TTT are configured independent of the neighbour cell and only CIO can be configured specifically towards a neighbour cell.

- In LTE, only TTT can be scaled depending on the mobility state as estimated by the UE.

- In LTE, network is aware of the UE mobility state (either via considering the UE handover history or by measuring the Doppler or position information etc.) for the RRC_CONNECTED mode UE.

- In LTE, network configuration of event specific offsets, hysteresis, TTT, CIO values depending on the UE’s estimated speed gives better control of handover border tuning.

- Handover borders between two cells are not smooth meaning there can be regions in the handover borders where the SINR degradation in more gradual and in some other regions SINR degradation can be more severe due the propagation difference in different regions.

Based on these observations, we have proposed the following:

- In NR, as in LTE, support mobility history information reporting.

- In NR, as in LTE, allow the network configuration of event specific offsets, hysteresis, TTT, CIO values via measurement reporting configuration.

- In NR, speed state based scaling of Time-to-trigger (TTT) is not supported unlike LTE .

- In NR, introduce SINR based scaling of the handover parameters.

- In NR, whether the scaling of the handover parameters is applicable or not is configured in the reportConfig.

# References

3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 14)”

K. Zetterberg, P. Ramachandra, F. Gunnarsson, M. Amirijoo, S. Wager, T. Dudda, “On Heterogeneous Network Mobility Robustness”, in Proc. IWSON at VTC Spring, Dresden, Germany, 2013.

R2-1710839, Summary of email discussion [99#32][NR] TP on RRM, 3GPP TSG-RAN WG2 NR#99, Ericsson, October 2017