3GPP TSG-RAN WG2 Meeting #99bis 	R2-1710887

# Prague, Czechia, 9-13 Oct 2017	Revision of R2-1708237

Agenda item:	9.4.4

Source: 	Qualcomm Incorporated

Title: 	Field trial results on handover performance for aerial vehicles

Document for:     Discussion/Decision

# Introduction

Cellular connectivity will be key for coordinated operation and control of Unmanned Aerial Vehicles, more commonly referred to as “Drones”, enabling a growing set of use cases within and beyond the drone operator’s visual line of sight. RAN#75 approved a study item on Enhanced Support for Aerial Vehicles [1]. One of the objectives of the study is [1]:

- Handover: Identify if enhancements in terms of cell selection and handover efficiency as well as robustness in handover signalling can be achieved. [RAN2, RAN1]

In this contribution, we present some field trial results on handover.

# Field Trials

Field trials were performed to analyze handover performance and to compare handover and link reliability between ground and airborne drone UEs.

## Setup

For this trial, data was collected as indicated in Table 1. Connectivity was provided and tested using a commercial cellular network during all flights.

Table 1 Trial Setup

| Data | Description |
| --- | --- |
| Location | Qualcomm UAS Flight Center, San Diego, California |
| Environment | Mixed suburban |
| Altitudes | Ground, 30, 60, 90, 120 meters |
| Test types | Mobility route at 5 m/s with 0.5 Mbps UDP UL throughput requested Mobility route at 5 m/s and periodic RACH every 15 seconds Stop/Start route with 0.5 Mbps UDP UL throughput requested |
| LTE bands (locked to one band per flight) | PCS AWS 700MHz |
| Data collection | On device logging IPerf logs |

## Mobility route

Much of the analysis in later sections is derived from a 2.5 km loop at different altitudes, and each altitude was flown multiple times for data collection in each band, and to provide sufficient data for each case (at least 2 loops).

The ground data was collected by mounting the drone to a car and driving the route on surface streets (the duration of these tests tended to be a bit longer than flying due to some stoplights and traffic.)

Each dataset was trimmed so the final data for analysis only includes samples where the drone is at its desired altitude and underway (i.e., excluding takeoff and landing). This prevents takeoff and landing transition data, as well as data when the drone is stationary on the landing pad, from impacting the analysis results.

## Handover results

Figure 1 shows distributions of the total interruption time during handovers.

Figure 1 Handover events and distributions of delay in handover completion

The figures show that majority of these handover interruptions are between 20-40 ms, but there are some outliers present as high as 800 ms. While these are not large interruption times, it is notable that the outliers are more likely in this data at altitudes 60 meters and higher.

Majority of the handover are completed within 20-40ms.

Some outliers are present which cause handover interruption of as high as 800ms.

This result particularly demonstrates that for the most part, current LTE networks can handle the handover for the aerial devices. However, some HO enhancements are beneficial to keep the interruption lower to guarantee the C&C communication.

Current LTE networks can handle the handover for the aerial devices. However, some HO enhancements are beneficial to keep the interruption lower.

Based on the above observations and discussion, we propose:

Consider the above field trial results for potential mobility enhancements for Aerial Vehicles (Drones).

Capture the results and analysis in the TR.

# Conclusion

In this contribution, based on the simulation results, we made the following observations on mobility performance of drone UEs:

Observation 1.	Majority of the handover are completed within 20-40ms.

Observation 2.	Some outliers are present which cause handover interruption of as high as 800ms.

Observation 3.	Current LTE networks can handle the handover for the aerial devices. However, some HO enhancements are beneficial to keep the interruption lower.

Based on the discussion and observations, we propose:

Proposal 1.	Consider the above field trial results for potential mobility enhancements for Aerial Vehicles (Drones).

Proposal 2.	Capture the results and analysis in the TR.

# References

[1] RP-170779, SID on Enhanced LTE Support for Aerial Vehicles, NTT DOCOMO INC, Ericsson