3GPP TSG-RAN WG2 Meeting #99bis	 R2-1710930

Prague, Czech Republic, 9th - 13th October 2017

Source:	vivo

Title:	Report of SCell-failure of PDCP duplication

Agenda Item:	10.2.7

Document for:	Discussion and Decision

Introduction

In the RAN2#99 meeting [1], RAN2 achieved the following consensus regarding the RLC failure of the CA duplicate bearer:

-	RLC reports maxNumberofRLC retransmissions are reached to RRC.

-	For a logical channel restricted to one or multiple SCell(s) (i.e. logical channel configured for duplication) UE reports the failure to the gNB (e.g. SCell-RLF) but no RRC re-establishment happens.

In this contribution, we discuss the RRC signaling details of the SCell-RLF report.

Discussion

Content of SCell-RLF reporting

Content 1: The indication of SCell-RLF

The UE needs to inform the network of which RLC encounters the SCell-RLF. If only 1bit is used to indicate whether there is SCell-RLF, the network does not know which DRB(s)/SCell(s) encounters the SCell-RLF. Then the network cannot reconfigure the CA duplicate bearer with better SCell(s). As such we could have the following Options for the indication of SCell-RLF:

- Option 1: Logical channel ID of the SCell-RLF

- Option 2: SCell index list of the SCell-RLF

As the network knows which cells are configured for the logical channel of the CA duplicate bearer, reporting the LCH ID can save some signaling overhead, compared with the SCell index list of the SCell-RLF. Given that a logical channel can be configured with more than one SCell(s).

Proposal 1: The UE reports the logical channel ID of the SCell-RLF.

Content 2: Measurement results

Same as the SCG failure report, reporting the available measurement results to the network can help the network to change/release the SCell configured for the CA duplicate bearer. The measurement results to be reported can include:

- Option 1: The measurement results of the failed SCell

- Option 2: The measurement results of the neighbor cells of the failed frequency

- Option 3: The measurement results of non-serving frequency

The above Options are like the measurement results reported in the SCG failure report. We think all measurement results as given in Option 1/2/3 should be reported.

Proposal 2: While reporting the SCell-RLF, the UE reports the valid cell-level measurement results including:

- The measurement results of the failed SCell(s)

- The measurement results of the neighbor cell(s) of the failed frequency

- The measurement results of non-serving frequency

One another measurement result which can be reported is the beam-level measurement result. From our understanding, the SCell-failure could be caused by a specific beam. Furthermore the change of the SCell for the CA duplicate bearer should change to a best beam of the target cell, so as to achieve better performance. The same principle as used for RRM measurement reporting can be reused. This means that while reporting the cell-level measurement results, the beam-level measurement results can be piggybacked in the same message for the reported cell.

Proposal 3: While reporting the cell-level measurement results, the UE reports the valid beam-level measurement results of the reported cell.

Reporting message

According to RAN2 agreement, the CA duplicate bearer can only be configured for NR. Then the SCell-RLF can occur in the following cases:

- Case 1: EN-DC SCG SCell-RLF

- Case 2: NR-DC SCG SCell-RLF

- Case 3: NE-DC MCG SCell-RLF

- Case 4: NR-DC MCG SCell-RLF

Here we have the following table to list the potential reporting messages for the SCell-RLF for different cases.

Table 1: Reporting of SCell-RLF

| Failure Case | Via SRB1 | Via SRB3 | Forwarded from MN to SN |
| --- | --- | --- | --- |
| MCG SCell-RLF for NE-DC or NR-DC | Yes | No | No |
| SCG SCell-RLF for EN-DC or NR-DC | Yes (if SRB3 is not present) | Yes (if SRB3 is present) | Yes (if reported via SRB1) |

As given in Table 1, for NE-DC or NR-DC, the SCell-RLF of MCG can be reported to the MN directly. Here we consider that RAN2 should firstly agree that a new message for SCell-RLF report is required. According to the legacy LTE, the SCGFailureInformation which does not require any cyphering is reported via SRB1. The same principle can be re-used for the SCell-RLF rmessage.

Proposal 4: A new message for reporting SCell-RLF is defined in NR.

Proposal 5: The MCG SCell-RLF is reported via SRB1.

As given in Table 1, for EN-DC and NR-DC case, the SCell-RLF of SCG can only be reported via the MCG SRB1 if SCG SRB3 is not configured. If SRB3 is configured, the SCG SCell-RLF is reported via SRB3. Then the SCell-RLF message can be included in a container in the SCGFailureInformation message of LTE. Furthermore, as the SCell and the CA duplication configuration of SCG is determined by the SN, we think that the SCG-RLF message for the SCG SCell-RLF if reported via SRB1 should be forward to the SN.

Proposal 6: The SCG SCell-RLF is included in a container of the SCGFailureInformation message, if SRB3 is not configured.

Proposal 7: The SCG SCell-RLF is reported via SRB3, if SRB3 is configured.

Proposal 8: The SCell-RLF message reported via SCGFailureInformation is forwarded to SN.

Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: The UE reports the logical channel ID of the SCell-RLF.

Proposal 2: While reporting the SCell-RLF, the UE reports the valid cell-level measurement results including:

- The measurement results of the failed SCell(s)

- The measurement results of the neighbor cell(s) of the failed frequency

- The measurement results of non-serving frequency

Proposal 3: While reporting the cell-level measurement results, the UE reports the valid beam-level measurement results of the reported cell.

Proposal 4: A new message for reporting SCell-RLF is defined in NR.

Proposal 5: The MCG SCell-RLF is reported via SRB1.

Proposal 6: The SCG SCell-RLF is included in a container of the SCGFailureInformation message, if SRB3 is not configured.

Proposal 7: The SCG SCell-RLF is reported via SRB3, if SRB3 is configured.

Proposal 8: The SCell-RLF message reported via SCGFailureInformation is forwarded to SN.

- Reference

[1] RAN2#99 meeting minutes.