3GPP TSG-RAN WG2 #99	R2-1711135

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.2.3

Source:	Ericsson

Title:	Tunnelling of NR messages via LTE

Document for:	Discussion, Decision

# Introduction

In case of MR-DC, it has been agreed that UL NR RRC measurement reports should be possible to send transparently through MN (RAN2 #99).

Agreements

1 For the case of MR-DC, a new Generic Container message carries a transparent container IE to hold an SN MeasurementReport message. Sent on SRB1

In this contribution, it is proposed to add a new LTE RRC message and procedure for tunnelling such messages in the case of EN-DC.

A text proposal to LTE RRC 36.331 is provided in section below.

# Text proposal to 36.331 v14.4.0

============================== Begin Text Proposal ============================

### 5.6.Y	UL NR information transfer

#### 5.6.Y.1	General

Figure 5.6.Y.1-1: UL NR information transfer

The purpose of this procedure is to transfer NR RRC TS 38.331 [80] information from the UE to E-UTRAN.

#### 5.6.Y.2	Initiation

A UE in RRC_CONNECTED initiates the UL NR information transfer procedure whenever there is a need to transfer NR RRC information. The UE initiates the UL NR information transfer procedure by sending the ULNRInformationTransfer message.

#### 5.6.Y.3	Actions related to transmission of ULNRInformationTransfer message

The UE shall set the contents of the ULNRInformationTransfer message as follows:

1>	set the ulNRinformation to include the information received from the NR RRC layer;

1>	submit the ULNRInformationTransfer message to lower layers for transmission, upon which the procedure ends;

#### 5.6.Y.4	Failure to deliver ULNRInformationTransfer message

The UE shall:

1>	if mobility (i.e. handover, RRC connection re-establishment) occurs before the successful delivery of ULNRInformationTransfer messages has been confirmed by lower layers:

2>	inform the NR RRC layer about the possible failure to deliver the information contained in the concerned ULNRInformationTransfer messages;

============================== End Text Proposal ============================

============================== Begin Text Proposal ============================

### 6.2.1	General message structure

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the E-UTRAN or from the RN to the E-UTRAN on the uplink DCCH logical channel.

-- ASN1START

UL-DCCH-Message ::= SEQUENCE {

message			UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

csfbParametersRequestCDMA2000				CSFBParametersRequestCDMA2000,

measurementReport							MeasurementReport,

rrcConnectionReconfigurationComplete		RRCConnectionReconfigurationComplete,

rrcConnectionReestablishmentComplete		RRCConnectionReestablishmentComplete,

rrcConnectionSetupComplete				RRCConnectionSetupComplete,

securityModeComplete						SecurityModeComplete,

securityModeFailure						SecurityModeFailure,

ueCapabilityInformation					UECapabilityInformation,

ulHandoverPreparationTransfer				ULHandoverPreparationTransfer,

ulInformationTransfer						ULInformationTransfer,

counterCheckResponse						CounterCheckResponse,

ueInformationResponse-r9					UEInformationResponse-r9,

proximityIndication-r9					ProximityIndication-r9,

rnReconfigurationComplete-r10				RNReconfigurationComplete-r10,

mbmsCountingResponse-r10					MBMSCountingResponse-r10,

interFreqRSTDMeasurementIndication-r10	InterFreqRSTDMeasurementIndication-r10

},

messageClassExtension	CHOICE {

c2							CHOICE {

ueAssistanceInformation-r11			UEAssistanceInformation-r11,

inDeviceCoexIndication-r11			InDeviceCoexIndication-r11,

mbmsInterestIndication-r11			MBMSInterestIndication-r11,

scgFailureInformation-r12				SCGFailureInformation-r12,

sidelinkUEInformation-r12				SidelinkUEInformation-r12,

wlanConnectionStatusReport-r13		WLANConnectionStatusReport-r13,

rrcConnectionResumeComplete-r13		RRCConnectionResumeComplete-r13,

ulNRInformationTransfer-r15			ULNRInformationTransfer-r15,

spare8 NULL, spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r11	SEQUENCE {}

}

}

-- ASN1STOP

============================== End Text Proposal ============================

============================== Begin Text Proposal ============================

### 6.2.2	Message definitions

#### –	ULNRInformationTransfer

The ULNRInformationTransfer message is used for the uplink transfer of transfer of NR RRC TS 38.331 [80] information.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULNRInformationTransfer message

-- ASN1START

ULNRInformationTransfer-r15 ::=			SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulNRInformationTransfer-r15			ULNRInformationTransfer-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULNRInformationTransfer-r15-IEs ::=	SEQUENCE {

ulNRinformation						OCTET STRING,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

============================== End Text Proposal ============================