3GPP TSG-RAN WG2 #99bis	R2-1711241

Prague, Czech Republic, 9th – 13th October 2017                 Resubmission of R2-1708327

Agenda Item:	10.3.3.7

Source:	Ericsson

Title:	PDCP SN reconfiguration at handover

Document for:	Discussion, Decision

# Introduction

RAN2 has agreed to not address the issue when the SN length is reconfigured and its length is reduced. This type of reconfigurations may result in some losses. Yet, it is still open when the SN may be reconfigured by the network.

# Discussion

A UE will be initially configured with a SN length which may depend on the UE capabilities, type of device, service, network load, achievable bit rates in the cell and other possible cases. In NR, currently only two SN length are allowed: 12 and 18 bits.

While the UE remains in the same cell, there should be no reason for changing the SN length. When the UE changes a cell, at handover, there may be times in which the SN length needs to be reconfigured. It could be that one of the two nodes involved in the handover may support a different SN range set, or that the cell has different configuration characteristics.

We do not think that SN length may be dynamically changed; thus, we consider that it is sufficient to allow the change of PDCP SN length only during handover, i.e. at reconfigurations when the PDCP entity is changed and results in a PDCP re-establishment.

PDCP SN length may be reconfigured only during a handover (at reconfigurations when the PDCP entity is changed and results in a PDCP re-establishment)

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	PDCP SN length may be reconfigured only during a handover (at reconfigurations when the PDCP entity is changed and results in a PDCP re-establishment)