3GPP TSG-RAN WG2 #99bis																		R2-1711400

Prague, Czech, 9th – 13th Octobor 2017

Agenda item:	10.4.1.9

Source:	Samsung

Title:	Measurement result contents for SN addition in MRDC

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

1	Introduction

The measurement result related MN/SN procedures were discussed in last RAN2#99 meeting as follows:

Agreements

1:	Measurement results relating to the target SN can be provided by MN to target SN at MN initiated SN change procedure.

2:	Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure.

3:	Measurement results according to measurement configuration from MN are encoded according to NR RRC when they are provided by MN to SN in SN Addition Request message (EN-DC).

4:	During SN initiates SN change procedure, measurement results according to measurement configuration from SN are encoded according to NR RRC when they are provided by MN to SN in SN Addition Request message.

In this contribution, we discuss on the measurement result contents for SN addition for a remaining and stage 3 issue.

2	Measurement result contents in inter node RRC message for SN addition

For SN addition, the measurement configuration is made by MN and MN might have restricted information to configure on the SN frequency. Still the exact content of the measurement configuration needs to be discussed further. Whatever contents are included in the measurement report from the UE, MN should process this result and select the target SN based on received result, and send this to the target SN.

Even corresponding contents of measurement configuration from UE is unclear, at least the frequency information to be measured must be included as a measurement object. The corresponding measurement report from the UE might include measurement results from multiple SNs on configured frequencies. Since MN has to decide the only one target SN based on received measurement report from the UE, it is meaningless to include the cell measurement results on other SNs than target SN in measurement result inter node message. In LTE DC case, MN gives the measurement result only on cells to be added. Since MN forces the SN to add those cells, there is no reason to provide the whole information on cells to be possibly added. However, in MR-DC MN should give the neutral information so that the SN is able to select and add the preferred cells by itself.

Proposal 1. The measurement result in inter node RRC message which is given by MN to SN for SN addition should only include the measurement result on cells in the target SN.

Proposal 2. The measurement result in inter node RRC message which is given by MN to SN for SN addition should include measurement result as much as inter node message container size admits. (i.e., should include not only some selective cell measurement result).

The above reasoning should be applicable to SN change procedure since the same message of SN addition request is used for SN change procedure too.

Proposal 3. The measurement result in inter node RRC message which is given by SN to MN for SN change should only include the measurement result on cells in the target SN.

Proposal 4. The measurement result in inter node RRC message which is given by SN to MN for SN change should include measurement result as much as inter node message container size admits. (i.e., should include not only some selective cell measurement result).

5.	Conclusion

In this contribution, measurement result content in SN addition procedure are discussed. We have the following proposals.

Proposal 1. The measurement result in inter node RRC message which is given by MN to SN for SN addition should only include the measurement result on cells in the target SN.

Proposal 2. The measurement result in inter node RRC message which is given by MN to SN for SN addition should include measurement result as much as inter node message container size admits. (i.e., should include not only some selective cell measurement result).

Proposal 3. The measurement result in inter node RRC message which is given by SN to MN for SN change should only include the measurement result on cells in the target SN.

Proposal 4. The measurement result in inter node RRC message which is given by SN to MN for SN change should include measurement result as much as inter node message container size admits. (i.e., should include not only some selective cell measurement result).