3GPP TSG-RAN WG2 #99bis	R2-1711459

Prague, Czech Republic, 9 - 13 October 2017

Agenda item:	10.4.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Support of reportOnLeave for E-UTRA B1 and B2

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#99 meeting (August 2017) the following agreements have been taken with respect to reportOnLeave in NR [1]:

Agreements:

1:	Support the ‘reportOnLeave’ measurement report triggering mechanism in NR.

2:	When the measurement report is triggered due to reportOnLeave, the UE just reports the current remaining triggered cells, not the ‘leave’ cell(s), as in LTE.

3:	The ‘reportOnLeave’ is applicable for all the measurement events and is configurable per reporting configuration.

reportOnLeave should be optionally supported for all NR measurement events, including A1-A6, B1 and B2. Nevertheless, it has not been confirmed reportOnLeave is also added to B1 and B2 (i.e. Inter-RAT) events on LTE side. This paper suggests taking such step and points to a CR outlining how to capture this change in LTE RRC specification [2].

# 2	Discussion

The benefits of introducing reportOnLeave to other events than just A3 and A6 (as it has been done in LTE) have been outlined in our paper [3] submitted to RAN2#99. Similar motivation has been captured in [4] which served as a basis for RAN2#99 online discussion. In the course of the meeting, reportOnLeave has been adopted for all NR events. The Inter-RAT NR events: (B1 and B2) would, however, work more efficiently if the reportOnLeave is added also to LTE side. This may be especially beneficial for EN-DC scenarios. Please note that Proposal 2 in our [3] was already suggesting taking such step and is reiterated below:

- Add the option to use reportOnLeave for B1 and B2 also to LTE measurement framework.

If Proposal 1 is found to be reasonable and agreeable then RAN2 should consider related CR [5].

- Adopt associated changes submitted in LTE RRC CR [5].

# 3	Conclusion

This paper briefly focused on reportOnLeave for E-UTRA events B1 and B2. As a result, the following Proposals have been made:

Proposal 1: Add the option to use reportOnLeave for B1 and B2 also to LTE measurement framework.

Proposal 2: Adopt associated changes submitted in LTE RRC CR [5].

# References

- R2-1710001 RAN2#99 Meeting Report

- 3GPP TS 36.331 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, version 14.4.0

- R2-1708589 On measurement events and the need of reportOnLeave 3GPP TSG-RAN WG2 #99	Berlin, Germany, 21 - 25 August 2017

- R2-1708707	Measurement report triggering mechanism in NR 3GPP TSG-RAN WG2 #99	Berlin, Germany, 21 - 25 August 2017

- R2-1711460 36.331 CR:Support of reportOnLeave for E-UTRA B1 and B2, 3GPP TSG-RAN WG2 #99bis	Prague, Czech Republic, 9 - 13 October 2017