3GPP TSG RAN WG2 #99bis	R2-1711599

Prague, Czech Republic, 9th to 13th October 2017            Resubmission of R2-1709602

Agenda Item:		10.2.10

Souce:		Samsung

Title:		The Necessity of T312 in NR

Document for:	Discussion/Decision

Introduction

Handover and RLF (Radio Link Failure) are important mechanisms to support seamless mobility for UEs. In the RAN2 #96 meeting, the following agreements about cell-level mobility (i.e., handover) were made.

Agreements: - The RRM measurement framework (measurement object, measurement id, reporting config) in LTE as a baseline in NR. - At least events like LTE A1-A6 and periodic will be supported for NR (modification to the events may be considered). - Network based mobility shall reuse the same principles as LTE (Rel-13).

In addition, the following agreements about RLF were made in the RAN2 #97bis meeting.

Agreements: - For connected mode, UE declares RLF upon timer expiry due to DL OOS detection, random access procedure failure detection, and RLC failure detection. - FFS whether maximum ARQ retransmission is only criteria for RLF failure (needs to be discussed in common UP/CP session). - In NR RLM procedure, physical layer performs out of sync/in sync indication and RRC declares RLF. - For RLF purposes, RAN2 preference is that the in sync/out of sync indication should be a per cell indication, and we aim for a single procedure for both multi-beam and single beam operation.

In this contribution, we will focus on the situation where a UE experiences poor link quality in a serving cell while moving to a neighbor cell. Note that one of NR deployment scenarios can be dense small cells, where inter-cell interference is severe. Furthermore, in high frequency (above 6 GHz) cells, sudden drop in link quality may happen frequently due to a blockage or the UE’s rotation. In this context, it is valuable to investigate how the UE escapes from this situation efficiently. Therefore, we will discuss the following issues.

- Fast RLF recovery during mobility in LTE

- Necessity of fast RLF recovery in NR

Fast RLF Recovery during Mobility in LTE

In LTE, the RLF-related actions are defined for a UE to initiate the connection re-establishment procedure when the UE experiences poor link quality in a serving cell [1]. More specifically, the RLF-related timer T310 operates as described in Table 1.

Table 1 Description of T310 for RLF

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T310 | Upon detecting physical layer problems for the PCell, i.e., upon receiving N310 consecutive out-of-sync indications from lower layers | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure and upon initiating the connection re-establishment procedure | If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure |

Note that the out-of sync indication is issued if the link quality is worse than the threshold Qout, which is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to 10 % block error rate of a hypothetical PDCCH transmission. Accordingly, the UE may not receive (or transmit) control information and data with a high probability while T310 is running.

Observation 1: In LTE, the operation of T310 is defined as a method for a UE to declare RLF.

To reduce the service interruption during T310, the operation of early T310 termination was introduced to LTE Rel-12. This operation considers the situation where a UE finds a target cell for handover while T310 is running. In particular, the timer T312, which control when to declare RLF, operates as described in Table 2.

Table 2 Description of T312 for early T310 termination

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T312 | Upon triggering a measurement report for a measurement identity for which T312 has been configured, while T310 is running | Upon receiving N311 consecutive in-sync indications from lower layers, upon triggering the handover procedure, upon initiating the connection re-establishment procedure, and upon the expiry of T310 | If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure |

Figure 1 Operation of early T310 termination using T312

Fig. 1 shows the operation of early T310 termination using T312. We assume that the UE satisfies the A3 event (i.e., RSRPtarget > RSRPserving + offset) for TTT (Time-To-Trigger) while T310 is running. Then, the UE starts T312 and attempts handover. If handover is not triggered (i.e., the UE does not receive handover command) until the expiry of T312 and T310 is still running, the UE declares RLF without waiting the expiry of T310 and performs the connection re-establishment procedure towards the target cell. By doing so, the UE can avoid further service interruption during the remaining T310.

Observation 2: The operation of T312, in which a UE is allowed to declare RLF before the expiry of T310, is beneficial to reduce service interruption time.

Necessity of Fast RLF Recovery in NR

3.1	Handover Command

In the RAN2 #97bis meeting, it was agreed that a gNB transmits handover command to a UE in order to trigger a handover procedure (i.e., network-based mobility). The agreements on this issue are captured as follows.

Agreements: - Handover command can contain at least cell identity of the target cell and RACH configuration(s) associated to the beams of the target cell. RACH configuration(s) can include configuration for contention-free random access. - UE selects a suitable beam from all beams of the target cell. - UE performs CBRA on the UE's selected beam if CFRA resources are not provided for the UE's selected beam.

It is well known that handover failure mainly occurs because the UE fails to receive the handover command from the serving gNB, although the UE already detects the target gNB (e.g., A3 event). Moreover, this phenomenon (i.e., handover command reception failure) is primarily caused by the poor link quality between the UE and the serving gNB. As a result, it is highly probable that the handover failure occurs while T310 of the UE is running. Such reasoning is valid for any handover procedure that relies on the handover command sent by the serving gNB, regardless of whether the procedure is used in LTE or NR. In this context, we think that the early T310 termination scheme using T312, which is already specified in LTE Rel-12, should be a baseline as a method for fast RLF recovery and/or mobility performance enhancement in NR.

Observation 3: In LTE, we observed that the main cause of handover failure is because a UE fails to receive handover command from a serving cell. Since NR also relies on the handover command to initiate handover (i.e., network-based mobility), it is expected that NR will have the same problem as in LTE.

3.2	CU-DU Architecture

Let’s assume that a UE declares RLF at the expiry of T312 and performs the connection re-establishment procedure towards a target cell for handover. In LTE, depending on whether a context of the UE is available at the eNB of the target cell, one of the following two cases can happen [2].

- Case 1: If the eNB finds a context that matches the identity of the UE, or obtains this context from the previously serving eNB, it indicates to the UE that its connection can be resumed.

- Case 2: If the context is not found, RRC connection is released and the UE initiates procedure to establish new RRC connection. In this case, the UE is required to go via RRC_IDLE.

It is obvious that the UE experiences less service interruption time in the case 1 than in the case 2. In other words, if the target cell belongs to a prepared eNB (i.e., an eNB which has admitted the UE during an earlier executed handover preparation phase, or obtains the UE context from the previously serving eNB), the service interruption time becomes shorter.

In NR, it is expected that the CU-DU architecture will be implemented, where a CU (Centralized Unit) manages a large number of cells formed by DUs (Distributed Units). Hence, the CU may have context information of the UEs served by the cells under itself so that the frequency of the case 1 may dominate that of the case 2. Due to such an availability of the context information at the CU, we think that fast RLF recovery schemes such as the early T310 termination using T312 work efficiently.

Observation 4: In the CU-DU architecture, the target cell for handover can easily obtain the UE’s context information from the previously serving cell via the CU. This aspect can make the time from the UE’s RLF declaration to the connection re-establishment shorter.

Proposal: RAN2 should adopt fast RLF recovery schemes based on the T312 operation (i.e., RLF declaration before the expiry of T310), which is already specified in LTE.

Conclusion

Observation 1: In LTE, the operation of T310 is defined as a method for a UE to declare RLF.

Observation 2: The operation of T312, in which a UE is allowed to declare RLF before the expiry of T310, is beneficial to reduce service interruption time.

Observation 3: In LTE, we observed that the main cause of handover failure is because a UE fails to receive handover command from a serving cell. Since NR also relies on the handover command to initiate handover (i.e., network-based mobility), it is expected that NR will have the same problem as in LTE.

Observation 4: In the CU-DU architecture, the target cell for handover can easily obtain the UE’s context information from the previously serving cell via the CU. This aspect can make the time from the UE’s RLF declaration to the connection re-establishment shorter.

Proposal: RAN2 should adopt fast RLF recovery schemes based on the operation of T312 (i.e., RLF declaration before the expiry of T310), which is already specified in LTE.

Reference

[1]	3GPP TS 36.331 V13.3.0, “Radio Resource Control (RRC); Protocol Specification,” October 2016.

[2]	3GPP TS 36.300 V13.5.0, “E-UTRA and E-UTRAN; Overall Description; Stage 2,” September 2016.