3GPP TSG-RAN WG2 #99bis	R2-1711679

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item: 	10.2.7

Source:	Apple Inc., OPPO, ZTE

Title:	UE capability indication for single UL transmission of LTE-NR DC

Document for:		Discussion

# 1	Introduction

As explained in [1] [2] [3], depending on the band combinations, sending two concurrent uplinks in two different bands can produce undesirable inter-modulations (IMD) that degrades UE’s RX sensitivity. This creates challenges in RF implementations. In order to avoid serious receiver sensitivity degradations on any agreed LTE/NR band combinations, it is imperative that some other mechanisms to be specified in Rel-15 NR work item phase to avoid simultaneous LTE and NR uplink transmissions from a single UE perspective when such issue occurs.

As a result, 3GPP RAN1 has tackled this problem and has reached the following agreements in the past three RAN1 meetings. Specifically, an agreement was made in RAN1 #89 in May, that NR supports the case that when the UE is configured with multiple UL carriers on different frequencies, the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers [4]; Further in RAN1 Adhoc NR #2 in June, a TDM based solution was selected and agreed [5]; In RAN1#90 in August, specific TDM patterns were selected and further agreed [6].

In order to enable the single UL transmission operation, the indication from UE side on whether the UE allows 2 simultaneous UL transmission is demanded. In RAN plenary #77 meeting, the following agreements were made in [7].

Agreements:

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified in RAN4) within that band combination the UE is allowed to indicated that it does not support 2 simulataneous UL tx (this is not implying the granularity of the signaling)

- Signaling to be defined to support the “red text” from RAN4 part of the single tx discussion (i.e. as in RP-172064)

  - UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

Besides, RAN sent out an LS in [8], which presents explicit guidance and expectation on RAN2/RAN4 to complete their corresponding tasks on time.

[Quoted from [8] ]

RAN has discussed the issue of simultaneous dual Tx vs. single Tx for LTE-NR dual connectivity and endorsed the attached documents RP-172064, RP-172085. RAN agreed that all the RAN WGs should complete their tasks by December 2017 to allow the feature to be introduced to the specifications.

In this contribution, we propose our view on how to endorse the corresponding UE capability indication based on the guidance from RAN.

# 2    Discussion

It is observed that from RAN agreement, UE capability of whether 2 simultaneous UL transmission is supportted or not on specific band combination and channel allocation within that band combination could both be considered, and RAN2 and RAN4 should complete their task by December 2017. It’s the common understanding that LTE-NR band combinations are already defined, which is also the basis for general UE capability definition in both LTE and the ongoing design of NR. Channel allocation is expected to provide finer information within specific band combinations and maximize the opportunity to configure the simultaneous UL transmission. However, channel allocation may not be clear at this stage, due to operators’ unavailability on it.

It’s thus obvious that currently per band combination UE capability signaling for non-supporting of 2 simultaneous UL transmission for difficult (with IMD issues) band combinations should be designed as baseline. For RAN2 perspective, this is considered a straightforward task. Even without finer channel-level signaling, UE capability indication can still be based on the knowledge of channel allocation for known network deployments, pre-provisioned on the UE (an implementation and testing topic).

As for the channel allocation within one band combination, the related normalization work coud be discussed independently from baseline capability in RAN4 at this stage.

In addition, for information, as following the consensual MR-DC band combination structure [9] is illustrated. The 2 simultaneous UL transmissions could be simply implemented with one bit RF capability indicator. Which band combinations are allowed to indicate this capability will be decided by RAN4.

Figure 1: Common MR-DC band combination parameter structure

Proposal: To introduce 1-bit per band combination as part of the UE RF capability signalling to indicate that the UE does not support simultaneous UL transmission.

# 3    Conclusion

In this contribution, we discuss the UE capability indication for single uplink transmission so that the single UL transmission operation can be configured appropriately to handle the desense issues for LTE-NR dual connectibity. The proposals include following:

Proposal: To introduce 1-bit per band combination as part of the UE RF capability signalling to indicate that the UE does not support simultaneous UL transmission.

# 4    References

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R1-1709586, “WF on UL Sharing for NSA NR UE””, 3GPP RAN1 #89, Hangzhou, China, May 15th-19th, 2017.

- R1-1711878, “LS on Single UL transmission ”, 3GPP RAN1 NR ad-hoc#2, Qingdao, China, 27th – 30th June 2017.

- R1-1714827, “Way forward on remaining issues of single UL transmission”, 3GPP RAN1#90, Prague, Czech, 21st – 25th August 2017

- RP-172085 Signalling for Single/Dual UL Tx   RAN2 Chair (Intel)   RAN Plenary #77   Sep., 2017

- RP-172100  LS on single Tx switched UL  RAN Plenary #77  Sep., 2017

- R2-17xxxx Report on Email Disc: [99#25][NR] Capability coordination - Part 1  Intel  Oct., 2017