3GPP TSG-RAN WG2 Meeting #99bis	R2-1711749

Prague, Czech Republic, 9th – 13th October 2017        (Resubmission of R2-1709430)

Agenda item:	9.10.3

Source:	Samsung

Title:	Supporting reliability during resource sharing

Document for:	Discussion

- Introduction

The work item on 3GPP V2X phase 2 was approved with following objectives which have impact on RAN2 [1]:

- Specify solutions for the following PC5 functionalities, which can co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs: [RAN1, RAN2, RAN4]

  - Carrier aggregation (up to 8 PC5 carriers);

  - 64QAM;

  - Reduce the maximum time between packet arrival at Layer 1 and resource selected for transmission;

  - Radio resource pool sharing between UEs using mode 3 and UEs using mode 4;

This contribution will discuss how to solve the resource collision from radio resource pool sharing between mode 3 and mode 4 UEs.

- Discussion

According to the agreement of radio resource pool sharing between mode 3 and mode 4 [1], we are able to describe 2 usage scenarios. Scenario 1 is resource pool sharing of mode 3 UE with mode 4 UE and Scenario 2 is resource pool sharing of mode 4 UE with mode 3 UE.

2.1 Resource pool sharing of mode 3 UE with mode 4 UE

This scenario describes that mode 4 UEs are able to use unused resource pool for mode 3 UE where the resource pool for mode 3 UE is dedicatedly allocated by eNB.

For example, eNB has 100 PRBs resource pool for mode 3 UEs and eNB dedicatedly allocates 50 PRBs to some of mode 3 UE. And remaining 50 PRBs resource is used by mode 4 UEs by using sensing operation. In this case, the resource pool for mode 3 UE is kept because mode 4 UEs use remaining resources of resource pool for mode 3.

If eNB allocates remaining 50 PRBs resource to other mode 3 UE while mode 4 UE is using remaining resources for mode 3 UE, the resource collision could happen between mode 3 UEs and mode 4 UEs. Since eNB does not know whether the remaining resource is used by mode 4 UE or not. To avoid such resource collision, eNB should know the occupied resource by mode 4 UE before eNB allocates resource for mode 3 UE when resource pool for mode 3 UE is shared for mode 4 UE. Therefore, it needs that resource occupied status is reported from mode 4 UE. In addition, transmission reliability is degraded since it is possible to use same resource between mode 3 UE and mode 4 UE. It is necessary to study feasibility evaluation in terms of reliability requirement while mode 4 UE is using remaining resources for mode 3 UE.

Proposal 1: It is necessary to study feasibility evaluation in terms of reliability requirement while mode 4 UE is using remaining resources for mode 3 UE

2.2 Resource pool sharing of mode 4 UE with mode 3 UE

This scenario describes that resource pool for mode 4 UE is commonly allocated by eNB and mode 3 UEs are able to use the resource pool allocated for mode 4 UE.

According to this scenario, eNB is able to allocate resource pool for UE using mode 4 to mode 3 UEs. The resource pool can be used by mode 4 UEs as well as mode 3 UEs through eNB’s allocation. Mode 3 UE is able to transmit message by using the allocated resource which is selected from resource pool for mode 4 UEs.

A potential issue of this scenario is that eNB doesn’t know whether resource occupied by mode 4 UE or not before dedicatedly allocating resource to mode 3 UE. It can cause resource collision between mode 3 UE and mode 4 UE. In order to avoid resource collision between mode 3 UE and mode 4 UE, it is required that mode 4 UE be informed which resource is used by mode 3 UE.

Proposal 2: eNB or mode 3 UE should inform mode 4 UE which resource is used by mode 3 UE in order to avoid resource collision between mode 3 UE and mode 4 UE.

In this scenario, if mode 4 UEs share resource pool for mode 4 UEs with mode 3 UEs and mode 3 UEs use resource dedicatedly allocated from resource pool for mode 4 UEs, the available resource pool for mode 4UEs may be reduced. It means that probability of resource collision increases among mode 4 UEs since mode 4 UEs select resource using sensing operation on the reduced mode 4 UE resource pool. It is not sufficient to support reliability requirement.

Observation 1. If resource pool size for mode 4 UEs is reduced because of resource pool sharing between mode 4 UE and mode 3 UE, then resource collision probability among mode 4 UEs increases. It is not sufficient to support reliability requirement.

So, when resource pool sharing between mode 4 UEs and mode 3 UEs, we need to solve the resource collision problem for supporting reliability requirement. If mode 4 UEs detect that the probability of resource collision could be high, mode 4 UEs request eNB dedicated resource allocation as mode 3 or uplink resource (Uu). For example, mode 4 UE checks channel busy ratio (CBR) before sending V2X message and decides whether mode 4 resource pool to be used or not based on CBR result. If the mode 4 UE decides that current status (e.g., CBR) of mode 4 resource pool cannot guarantee PPPP for the V2X message, the mode 4 UE requests dedicated resource to eNB.

Proposal 3. There needs a dedicated resource allocation for mode4 UE to avoid suffering mode 4 UE from resource collision due to resource pool sharing with mode 3 UE when mode 4 UE’s V2X message requires high reliability.

3. Conclusion

Proposal 1: It is necessary to study feasibility evaluation in terms of reliability requirement while mode 4 UE is using remaining resources for mode 3 UE

Proposal 2: eNB or mode 3 UE should inform mode 4 UE which resource is used by mode 3 UE in order to avoid resource collision between mode 3 UE and mode 4 UE.

Observation 1. If resource pool size for mode 4 UEs is reduced because of resource pool sharing between mode 4 UE and mode 3 UE, then resource collision probability among mode 4 UEs increases. It is not sufficient to support reliability requirement.

Proposal 3. There needs a dedicated resource allocation for mode4 UE to avoid suffering mode 4 UE from resource collision due to resource pool sharing with mode 3 UE when mode 4 UE’s V2X message requires high reliability.

4. References

[1] RP-170798, “New WID on 3GPP V2X Phase 2”, Huawei, CATT, LG Electronics, HiSilicon, China Unicom.