3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711994

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.1.5.1

Source:	Ericsson

Title:	Offline discussion #41: Text proposal for MAC and RRC relating to beam selection during handover

Document for:	Discussion, Decision

# Introduction

This document contains text proposals for MAC and RRC relating to the beam selection during handover.

The MAC text proposal is based on R2-1711443. Note that the assumption is that the dedicated RACH configuration contains a list of random access resources associated with either SSB and CSI-RS. The entries in the list will contain SSB index, a dedicated sequence and the time frequency location of the random access occasion, e.g.

CFRA-SSB-Resource::= SEQUENCE {

ssb			SSB-ID,

ra-PreambleIndex			INTEGER (0..XX)

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

}

Similarly the common configuration is assumed to contain a list of SSB indeces, preamble group A and B configuration for each SSB and the time frequency location of the random access occasion

CBRA-SSB-Resource::= SEQUENCE {

ssb			SSB-ID,

preamblesGroupAConfig	SEQUENCE {

ra-PreamblesGroupA	SetOfPreambles,

}

preamblesGroupBConfig	SEQUENCE {

ra-PreamblesGroupB	SetOfPreambles,

} OPTIONAL

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

# Text Proposal for MAC

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if a list of contention free RA resources has been explicitly provided by RRC:

2> if the list of contention free RA resources contains a list of SSB indices, select a suitable SSB index from the set of explicitly provided SSB indeces, where an SSB index is considered suitable if it is corresponding to a SSB whose quality measured on the associated NR-SS is above a threshold;

2>	else if the list of contention free RA resources contains a list of CSI-RS indices, select a suitable CSI-RS index from the set of explicitly provided CSI-RS indices, where a CSI-RS index is considered suitable if it is corresponding to a CSI-RS whose quality measured on the associated NR-SS is above a threshold;

Editor's note: The name of the threshold is FFS.

2>	set the PREAMBLE_INDEX to the ra-PreambleIndex corresponding to the selected index;

1>	else if no contention free RA resources were explicitly provided or if none of the SSBs and CSI-RS from the list of explicitly provided contention free RA resources was suitable:

Editor's note: preamble selection for SI request using msg-1 based/ msg-3 based can be discussed further.

2>	if Msg3 has not yet been transmitted:

3> select one suitable SSB index from the set of common SSB indeces, where an SSB is considered suitable if the quality measured on the associated NR-SS is above a threshold;

Editor's note: The name of the threshold is FFS.

4>	if Random Access Preambles group B exists on the selected SSB; and

4>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA on the selected SSB:

5>	select the Random Access Preambles group B;

4> else:

5>	select the Random Access Preambles group A.

2> else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

2>	select a ra-PreambleIndex within the selected group randomly with equal probability;

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex;

Editor's note: the value/range of ra-PreambleIndex should be confirmed by RAN1.

Editor's note: when the RAP group A/B is selected, path loss (in LTE) is not captured since no agreements were made, and RAN2 further discussion is required.

Editor's note: above 0b000000 in selection was removed to correct Editor's mistake.

1>	determine the next available NR-UNIT containing PRACH permitted by the restrictions given by the prach-ConfigIndex;

Editor's note: depending on the RAN1 decision, selection of PRACH resource can be further elaborated.

Editor's note: depending on the RAN1 decision, the ra-PreambleIndex selection may influence when the next available NR unit is if the preamble definition includes time (slot/position within slot)

Editor's note: The term 'NR-UNIT' is used tentatively, and should be replaced later with e.g. subframe/TTI/slot after having RAN1 input.

1> perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

# Text Proposal for RRC

Possible ASN.1 for the random access parameters is provided below. It is expected that this is captured as part of the L2 text proposal.

RACH-ConfigDedicated ::=	SEQUENCE {

cfra-Resources				CFRA-Resources

}

CFRA-Resources ::= CHOICE {

cfra-ssb-ResourceList	CFRA-SSB-ResourceList,

cfra-csirs-ResourceList	CFRA-CSIRS-ResourceList

}

CFRA-SSB-ResourceList ::= SEQUENCE (SIZE(1..maxRAssbResources)OF CFRA-SSB-Resource

CFRA-CSIRS-ResourceList::= SEQUENCE (SIZE(1..maxRAcsirsResources)OF CFRA-CSIRS-Resource

CFRA-SSB-Resource::= SEQUENCE {

ssb			SSB-ID,

ra-PreambleIndex			INTEGER (0..XX)

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

}

CFRA-CSIRS-Resource ::= SEQUENCE {

csirs		CSIRS-ID,

ra-PreambleIndex			INTEGER (0..XX)

-- 	PRACH configuration for CSIRS configuration (i.e. time and frequency location)

}

PCellConfigCommon ::= SEQUENCE {

-- various IEs

rach-ConfigCommon		RACH-ConfigCommon,

-- other IEs

...

}

-- RACH-ConfigCommon should also be included in SI

RACH-ConfigCommon ::=	SEQUENCE {

cbra-Resources			CBRA-SSB-Resource,

-- Power control parameters for random access

}

CBRA-SSB-Resource ::= SEQUENCE {

ra-Msg3SizeGroupA			RA-Msg3Sizes,

messagePowerOffsetGroupB	MessagePowerOffsetGroupB OPTIONAL,

cbra-SSB-ResourceList		CBRA-SSB-ResourceList

CBRA-SSB-ResourceList ::= SEQUENCE (SIZE(1..maxRAssbResources)OF CBRA-SSB-Resource

CBRA-SSB-Resource::= SEQUENCE {

ssb			SSB-ID,

preamblesGroupAConfig	SEQUENCE {

ra-PreamblesGroupA	SetOfPreambles,

}

preamblesGroupBConfig	SEQUENCE {

ra-PreamblesGroupB	SetOfPreambles,

} OPTIONAL

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

}