3GPP TSG-RAN WG2 Rel-13 LTE ASN.1 review Ad-Hoc Meeting 	R2-160034

# Helsinki, Finland, 13 - 14 January 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RRC connection initiation conditions for sidelink discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-01-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | As agreed in RAN2, if the RRC_IDLE UE configured to transmit relay PS related sidelink discovery on other frequency than the camped frequency and if the SIB 19 broadcast by the camped cell indicates the sidelink discovery transmission resource on the other frequency should be requested through dedicated signalling, the UE should initiates the RRC Connection establishment. However, in section 5.3.3.1a, the current description does not capture this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is suggested to divide the RRC connection initiation conditions for PS related sidelink discovery into two cases: one is for non-relay PS related discovery announcement and the other one for the relay PS related discovery announcement. For the relay PS related sidelink discovery, both the camped frequency as well as the other frequency for the sidelink discovery transmission are considered to trigger the RRC Connection initiation if necessary. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One conditions for establishing RRC connection for sidelink discovery is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.3.3.1a	Conditions for establishing RRC Connection for sidelink communication/ discovery

For sidelink communication an RRC connection is initiated only in the following case:

1>	if configured by upper layers to transmit non-relay related sidelink communication and related data is available for transmission:

2>	if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 does not include commTxPoolNormalCommon;

1>	if configured by upper layers to transmit relay related sidelink communication:

2>	if the UE is acting as relay; or:

2>	if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the cell on which the UE camps and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE;

3>	if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 either does not include commTxPoolNormalCommon or does not include commTxAllowRelayCommon:

For sidelink discovery an RRC connection is initiated only in the following case:

1>	if configured by upper layers to transmit non-PS related sidelink discovery announcements:

2> if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps does not include discTxPoolCommon; or

2>	if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResources included and set to requestDedicated:

1>	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements:

2>	if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or

2>	if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated:

- if configured by upper layers to transmit relay PS related sidelink discovery announcements:

2>

if the UE is acting as relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19 of the cell on which the UE camps; and above discThreshLoRelayUE, if included in the same SystemInformationBlockType19; or

2>

if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19 of the cell on which the UE camps:

3>	if the frequency on which the UE is configured to transmit relay PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or

3>	if the frequency on which the UE is configured to transmit relay PS related sidelink discovery announcements is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated:

NOTE:	Upper layers initiate an RRC connection. The interaction with NAS is left to UE implementation.