3GPP TSG-RAN WG2 Rel-13 LTE ASN.1 review Ad-Hoc Meeting	R2-160037

# Helsinki, Finland, 13 - 14 January 2016

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev |  | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for the sidelink discovery announcement pool selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-1-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The poolToAddModList IE is only contained in the IE SL-DiscConfig, which specifies the dedicated configuration information for sidelink discovery. In another word, the poolToAddModList IE could only be configured via dedicated signalling. As the result, according to the current description under ASN.1 review, the RSRP-based pool selection mechanism is only applicable for the CONNECTED UEs, which is not aligned with the agreements. In addition, “the reference cell as indicated by discTxRefCarrier” in the description of sidelink discovery announcement pool selection is not accurate. According to the section 5.10.6b in the current 36.331 running CR, the reference cell could be 1) the primary carrier; or 2) a secondary carrier; or 3) the cell indicated by the discTxRefCarrierDedicated IE; or 4) the cell indicated by the discTxRefCarrierCommon IE; or 5) the DL frequency paired with the one used to transmit sidelink discovery announcements on as reference. So the current description excludes the case 1), 2) and 5) and is not accurate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) The description for discovery pool selection is modified to cover both configurations via dedicated signalling and SIB19 cases. “within poolToAddModList” is removed. 2) The description for reference cell is modified to cover all the cases specified in the section 5.10.6b in the current 36.331 running CR. “the reference cell as indicated by discTxRefCarrier” is replaced by “the reference cell as specified in section 5.10.6b”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of sidelink discovery announcement pool selection is inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.10.6a	Sidelink discovery announcement pool selection

A UE that is is configured with a list of resource pool entries for sidelink discovery announcement transmission shall:

1>	if poolSelection is set to rsrpBased:

2>	select an entry of the list of resource pools for which the RSRP measurement of the reference cell as specified in 5.10.6b, after applying the layer 3 filter defined by quantityConfig as specified in 5.5.3.2 , is in-between threshLow and threshHigh;

1>	else:

2>	randomly select, using a uniform distribution, an entry of the list of resource pools;

1>	configure lower layers to transmit the sidelink discovery announcement using the selected pool of resources;