3GPP TSG RAN WG2 ad hoc								R2-160491

Budapest, Hungary, 19-21 January 2016

Source:	Vodafone

Title:	“Solution 2”: Queuing of ‘signalling’ behind ‘data via MME’

Agenda item:	4.3

Document for:	Discussion and Agreement

# 1. Introduction

SA2 has technically endorsed a number of P-CRs for “solution 2” for NB-IoT based on the agreements reached by the group.

However, details of the handling of data sent via the MME have not been addressed in detail by SA2: in part, this is because SA2 has left the data handling mechanisms of the existing RATs to other committees (i.e. RAN 2 for the RLC and PDCP protocols for UTRAN and E-UTRAN, and CT 1 for the LLC and SNDCP protocols for GPRS).

This paper introduces an aspect that Vodafone believes RAN 2 can, and should, consider.

# 2. Key attributes of “Solution 2” from SA2’s TR23.720

# The rather rapid generation of TR 23.720 over the last 8 months has meant that some of the background to solution 2 in SA2’s earlier TR(s) has been omitted.

However, the key to its NB-IoT battery life gains (and its reasonable ‘exception reporting’ latency) over “release 12” EPS mechanisms lie in the ability to send/receive data by reusing the NAS security context in MME & UE, and maintaining the IP header compression context during long periods of inactivity.

Note:    there can be considerable confusion caused by different usages of terminology. The author prefers to try and use the term “data via the MME” rather than terms such as “data over NAS”.

# 3. Handling of signalling queued behind data packets in solution 2

In line with the existing EPS architecture, an undocumented (but discussed) SA2 assumption is that the use of “data via MME” should not normally cause IP-level IP packet fragmentation, i.e. “small data via MME” must handle a payload of “1500 bytes plus a small number of bytes” (e.g. 1600 bytes).

At extreme coverage, NB-IoT can have a rather low data rate, e.g. perhaps only 160 bit/s (Annex A, TR 45.820). At this data rate, a 1600 byte packet would take 80 seconds to be delivered!

The NB-IoT radio interface will (at least for “solution 2”) use a very simple RLC protocol that only transports one “thing” at a time – either ‘data’ or ‘signalling’, not both. (See the RAN 2 running CR to 36.300 which captures their agreements in R2-157187 on “solution 2” in section x.3.6.2 , “DL information transfer and UL information transfer messages are used to carry small data and carried over SRB1;…..; Data radio bearer (DRB) is not used”)

Having a signalling (or SMS) message queued for potentially 80 seconds behind one data packet does not lead to an elegant EMM or ESM protocol, and, can be expected to cause problems to SMS protocols running to nodes e.g. outside of the VPLMN.

One simple solution is for the MME and UE to cut the data packet up into smaller PDUs, and for the E-UTRAN to prioritise signalling over queued “data via MME” packets.

The use of a segmentation/reassembly in the MME and UE for the “data via MME” is discussed in C1-160358 submitted to their #95bis Nashville meeting (11-15 Jan 2016). C1-160358 suggests that (at least for NB-IoT) a SDU segmentation/reassembly function IS used with e.g. a default payload size of [200] bytes, and that the RAN (and UE’s lower layers) should prioritise transmission of signalling packets ahead of queued data PDUs.

Note that following segmentation of a large packet, the MME would promptly deliver all of the sub-packets to the eNB, i.e. they would NOT be buffered in the MME.

In the “NB-IoT software download case”, the problem/issue can be much worse because downlink data is buffered in the eNB and signalling could get queued behind multiple maximum size IP packets.

Observation 1) in line with the existing EPS architecture, in RRC connected state, downlink user data is not buffered in the core network (neither MME nor SGW) but is buffered in the eNB.

Observation 2) there is no feedback from the eNB to the MME when a downlink packet is delivered to the UE. Hence an MME has no way to know whether there is any (or no) queued data within the eNB. Hence once in RRC connected state, the MME will always promptly send “data via the MME” packets to the eNB.

Observation 3) from observation 2, unless signalling can ‘overtake’ queued data packets, no estimate can be made for the timer values that the NAS signalling protocol should use.

Observation 4) in line with the existing EPS architecture, support for IP packets aligned with the Ethernet  Maximum Transmission Unit (=1500 bytes) should be provided by the 3GPP system.

Observation 5) at the extreme coverage data rate of 160 bps, and allowing for some headers, the transmission of a non-segmented ‘full MTU size’ packet could take up to around 80 seconds.

Proposal 1a) TS 36.300 should specify that the eNB sends downlink NAS signalling packets ahead of queued data packets. (The MME timers should not have to be varied according to different internal eNB implementations, thus this internal eNB behaviour is “mandatory” in order to maintain an open RAN/CN interface.)

Proposal 1b) TS 36.300 should specify that uplink NAS signalling is scheduled ahead of data packets queued within the UE’s lower layers.

Proposal 2) to enable CT 1 design work to progress quickly, RAN 2 (in collaboration with RAN 1) should liaise to CT 1 to inform CT1 of the throughput that can be achieved at extreme NB-IoT coverage.