3GPP TSG-RAN WG2 NB-IOT	R2-160503

Budapest, Hungary, January 19 – 21 2016

Agenda Item	: 6.1 (NB_IOT-Core)

Source	: LG Electronics Inc.

Title	: Early use of DRX cycle for NB-IOT

Document for	: Discussion and Decision

1.	Introduction

In RAN2 #92 meeting, RAN2 agreed that it is beneficial to enable DRX also for short connections, as early as possible.

In LTE, DRX operation controls UE’s PDCCH monitoring in a way that the UE saves power consumption while being provided sufficient scheduling opportunities. The same principle can be applied to the NB-IOT UE but with a stronger intention of power saving and an assumption of small data transmssion.

In this document, we will analyze DRX operation for NB-IOT UE and discuss how to save the UE’s power consumption by considering the small data transmission.

2.	Discussion

2.1.	DRX operation in LTE

As per DRX operation,

- the UE starts/restarts drx-InactivityTimer when the UE receives a PDCCH indicating a new transmission.

- the UE starts using DRX cycle when drx-InactivityTimer expires or a DRX Command MAC control element is received.

- drx-InactivityTimer can be from 1 PDCCH-subframe up to 2560 PDCCH-subframes.

Note that drx-InactivityTimer would be configured by considering the opportunity for scheduling a new transmission.

2.2.	DRX operation in NB-IOT

In the scope of eMTC, RAN2 has discussed DRX operation. Considering repetition transmission/reception in eMTC, RAN2 decided that drx-InactivityTimer is started after the last subframe of M-PDCCH repetition. Similarly, for NB-IOT, we assume repetition transmission/reception and look into when DRX cycle starts:

- step0. The UE has a small data and requests an UL grant for the small data transmission.

- step1. The UE receives an UL grant by repetition reception, and starts drx-InactivityTimer at the end of PDCCH repetition for the UL grant.

- step2. The UE transmits an MAC PDU including the data by repetition transmission.

- step3. The UE keeps monitoring PDCCH while drx-InactivityTimer is running.

- step4. When drx-InactivityTimer expires, the UE starts using DRX cycle.

We can observe that the UE keeps monitoring PDCCH after transmission of the MAC PDU for additional scheduling opportunity. And then, the UE finally starts using DRX cycle after drx-InactivityTimer expiry. Even for the last MAC PDU transmission, the UE keeps monitoring PDCCH due to drx-InactivityTimer which has been started upon reception of PDCCH for the UL grant. Note that, in NB-IOT, we assume that drx-InactivityTimer would be a long value because of repetition transmission/reception.

In LTE, it can be assumed that the UE, in general, wouldn’t be provided an UL grant that can accommodate all data at once. Thus, the UE would need to wait for another scheduling in order to transmit the remaining data. In this case, applying the same UE behaviour even for the last data transmission, i.e., keeping monitoring PDCCH even after the last data transmission, would make the specification simple while not brining any critical problem.

In NB-IOT, however, the data would be typically a small data so that one shot transmission would be generally sufficient. Accordingly, mandating the UE to monitor the PDCCH even after the last data transmission would bring unnecessary PDCCH monitoring more frequently compared to LTE. Given that power consumption is one critical designing factor in NB-IOT, it seems not desirable.

Observation. As per the current DRX operation, the UE keeps monitoring PDCCH even after the last data transmission, which brings significant power consumption considering that a long drx-InacitivtyTimer value would be used for NB-IOT.

Based on the observation, it would be good to enhance DRX operation for NB-IOT.

One option is that the UE starts using the DRX cycle when the UE transmits the last data of a data burst to the eNB. The gain would be that UE saves power consumption by using the DRX cycle without waiting for drx-InactivityTimer expiry. Given that a long drx-InactivityTimer value would be used for NB-IOT, a significant power saving gain is expected.

Proposal 1. The NB-IOT UE starts using the DRX cycle when the UE transmits the last data to the eNB.

One issue with proposal 1 is that the UE autonomously goes into DRX cycle when it sends the last data of a data burst. This may cause desynchronization of DRX state between UE and eNB, which would bring waste of radio resource or unnecessary PDCCH monitoring.

Therefore, the UE needs to send a kind of indication to the eNB when it sends the last data of a data burst so that the eNB knows that the UE goes into DRX cycle. For example, BSR with zero buffer status could be used for this purpose.

Proposal 2. The NB-IOT UE sends a last data indication when the UE transmits the last data to the eNB.

One may say that the eNB can send a DRX command MAC CE upon reception of the last data indication from the UE. However, it costs the UE’s monitoring and processing power of the DRX command MAC CE. Having repetition transmission/reception, the cost would be increased for NB-IOT. With an intention of saving UE power consumption in NB-IOT, it would be helpful to start using DRX cycle even without the DRX command MAC CE in terms of power saving.

3.	Conclusion

In this document, we addressed the DRX aspect of NB-IOT UE. From power consumption point of view, we observe that

Observation. As per the current DRX operation, the UE keeps monitoring PDCCH even after the last data transmission, which brings significant power consumption considering that a long drx-InacitivtyTimer value would be used for NB-IOT.

Based on the observation we propose:

Proposal 1. The NB-IOT UE starts using the DRX cycle when the UE transmits the last data to the eNB.

Proposal 2. The NB-IOT UE sends a last data indication when the UE transmits the last data to the eNB.

4.	Reference

[1] TR23.720, Architecture enhancements for Cellular Internet of Things

[2] R2-157187, Running 36.300 CR to capture agreements on NB-IoT