3GPP TSG RAN WG2 NB-IoT		R2- 160523

Budapest, Hungary, 19th – 21st January 2016

Source:	Samsung Electronics

Title:	Discussions on access control for NB-IoT

Document for:	Discussion

Agenda item:	5.2

Introduction

At RAN2 #92, we discussed access control for NB-IoT and come to an agreement that one single mechanism (which is FFS) is expected to support access control. Further following agreements are described below [1]:

Access control follows the assumptions as below: - The access control mechanism shall be able to discriminate between different roaming UEs, i.e. the same roaming differentiation as for EAB; - Priority discrimination is needed. The priority discrimination classes can be hard-coded in the specification; normal reporting, high-priority/alarm/exception report. This needs to be provided by NAS. The final classes are FFS; - A barring bitmap is used. The barring bitmap is transmitted separately from other system information and only when access control is enabled; - Barring bitmap check is applicable to normal reports. A separate flag is broadcasted which indicates if exception reports are subject to barring bitmap check or not. The following access control aspects are FFS: - whether to introduce a third class of priority, the use case need to be better clarified; - whether to introduce barring time; - whether change of bitmap will trigger SI change indication; - how to spread the load after un-barring / barring change.

This contribution discusses the details on Access control for NB-IoT.

Discussion

In the last meeting, RAN2 agreed to support one single mechanism for access control in NB-IoT. Further detailed considerations were that it shall be able to discriminate between different roaming UEs as EAB, barring bitmap check is used, and barring parameters are transmitted separately from other system information. These assumptions reach to a conclusion that it would be better to develop a mechanism which is based on EAB. It can reuse barring bitmap check scheme of EAB and barring parameters including barring category and barring bitmap can be transmitted in legacy SIB 14.

Proposal 1	NB-IoT Access control mechanism can be enhanced based on EAB.

There still exist problems to optimize EAB-approach access control. For example, as ACB is absent, the roles it once handled such as controlling the access for AC 10-15 or barring time are needed to be applied in developed solution. Also, complicated update procedure of EAB barring parameters can be simplified to be suitable for NB-IoT.

Observation 1	Access control mechanism for NB-IoT needs to supplement the roles of ACB and the weak points of EAB.

First of all, the new EAB-approach scheme can adopt the barring time. In ACB, UEs are provided with barring time information and it starts when access to the cell is barred. While the timer is running, monitoring barring parameters and access to the cell is barred so that UEs can save power. If the new mechanism operates without barring time, there could exist repetition of unnecessary barring check and power consumption would be more than that of legacy UEs supporting both EAB and ACB.

If the barring time is considered to be supported, it needs to be long enough so that SIB carrying the barring parameters is updated before the expiry of the barring time. Then, barred UEs can retry barring check with the newly set barring bitmap. In general, the control of barring parameters and SI update is highly related with the congestion situation and it is figured out by the network. Thus, network can provide proper minimum value of barring time securing the chance to update System information. Also, by randomizing the barring time and wake-up time of UEs, loads of re-accessing UEs would be spread. Therefore, we suggest supporting network-configured barring time.

Proposal 2	The barring time is necessary for energy-efficient operation of NB-IoT devices. We prefer it to be configured by network so that it can properly reflect the congestion.

One of shortcomings of EAB is the complicated barring parameter update procedures. Whenever barring bitmap in SIB 14 changes, UEs are informed about the updates by receiving paging indication. Unlike usual SI updates in which UEs wait for the next modification period, special paging indication trigger UEs to update barring parameters immediately.

We prefer not to trigger paging or value tag. Frequent paging or value tag update would cause more power consumption on devices. Rather, UEs can check the barring bitmap when it initiates connection establishment. For the energy-efficient operation, using barring time is emphasized once again. Also, the barring parameters are needed to be updated immediately spreading the access traffic of unbarring UEs.

Proposal 3	SIB carrying the barring parameters can be updated immediately without triggering SI change indication.

We briefly described overall access control scheme of the contribution in Figure 1.

Figure 1 	Suggesting access control scheme for NB-IoT

At the last meeting, RAN2 agreed that the barring bitmap is transmitted separately from other system information and only when access control is enabled. By controlling the enablement of access control, UEs can save power consumed for checking SIB 14 and skip access barring check. There exist several methods to enable/disable access control, but we suggest practically using legacy scheduling information in SIB 1.

Proposal 4	Enablement of access control can be informed by the scheduling information in SIB 1.

Conclusions

In this contribution we have discussed access control for NB-IoT. Following proposals are made in this contribution:

Proposal 1	NB-IoT Access control mechanism can be enhanced based on EAB.

Proposal 2	The barring time is necessary for energy-efficient operation of NB-IoT devices. We prefer it to be configured by network so that it can properly reflect the congestion.

Proposal 3	SIB carrying the barring parameters can be updated immediately without triggering SI change indication.

Proposal 4	Enablement of access control can be informed by the scheduling information in SIB 1.

References

[1] R2-157187, Running 36.300 CR to capture agreements on NB-IoT, Huawei, RAN2#92, Budapest, Hungary, January 2016.