3GPP TSG-RAN WG2 Meeting NB-IOT ad-hoc	R2-163257

Sophia-Antipolis, France, 3-4 May, 2016

Agenda Item:	6.1

Source:	Ericsson

Title:	Connected mode DRX timers for NB-IoT

Document for:	Discussion, Decision

# Introduction

This contribution discusses the value ranges needed for DRX timers in RRC connected state in NB-IoT.

At the RAN2#93bis meeting ([1]) the following was agreed with regards to DRX signalling:

- The drx-InactivityTimer should be started at the end of the transmission/re-transmission of each MAC PDU

- The “UL HARQ RTT Timer” is constant and started on the last subframe of the NPUSCH transmission and the “HARQ RTT Timer” is variable and started on the last subframe of the NPDSCH transmission

- The onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer and the HARQ RTT timer in NB-IoT should be extended, compared to eMTC/LTE

- drx-ULRetransmissionTimer in eMTC should be re-used in NB-IoT because of the asynchronous and adaptive UL HARQ process. While the values should be extended

- We use PDCCH period instead of PDCCH subframe as basic unit of corresponding DRX parameters, except for the DRX cycle

- We use table below as baseline for value ranges, but aim to have fewer start-offset-values and fewer on-duration-values

| Parameters | Analysis | LTE (not updated) | NB-IoT |
| --- | --- | --- | --- |
| onDurationTimer | Using pp instead of psf directly | psf {1, 2, 3, 4, 5, 6, 8, 10, 20, 30, 40, 50, 60, 80, 100, 200} | pp {1, 2, 3, 4, 5, 6, 8, 10, 20, 30, 40, 50, 60, 80, 100, 200} |
| drx-InactivityTimer | As analysized in proposal 3 and using pp instead of psf. Remove some big values | psf {1, 2, 3, 4, 5, 6, 8, 10, 20, 30, 40, 50, 60, 80, 100, 200, 300, 500, 750, 1280, 1920, 2560} | pp {0, 1, 2, 3, 4, 5, 6, 8, 10} |
| drx-RetransmissionTimer | Using pp instead of psf directly | psf {1, 2, 4, 6, 8, 16, 24, 33} | pp {0, 1, 2, 4, 6, 8, 16, 24, 33} |
| HARQ RTT Timer | Re-use eMTC | 8 subframes | Re-use: 8 subframes |
| longDRX-CycleStartOffset | Using pp instead of sf directly | sf10 INTEGER(0..9), sf20 INTEGER(0..19), sf32 INTEGER(0..31), sf40 INTEGER(0..39), sf64 INTEGER(0..63), sf80 INTEGER(0..79), sf128 INTEGER(0..127), sf160 INTEGER(0..159), sf256 INTEGER(0..255), sf320 INTEGER(0..319), sf512 INTEGER(0..511), sf640 INTEGER(0..639), sf1024 INTEGER(0..1023), sf1280 INTEGER(0..1279), sf2048 INTEGER(0..2047), sf2560 INTEGER(0..2559) sf5120 INTEGER(0..1), sf10240 INTEGER(0..3) | pp 10 INTEGER(0..9), pp 20 INTEGER(0..19), pp 32 INTEGER(0..31), pp 40 INTEGER(0..39), pp 64 INTEGER(0..63), pp 80 INTEGER(0..79), pp 128 INTEGER(0..127), pp 160 INTEGER(0..159), pp 256 INTEGER(0..255), pp 320 INTEGER(0..319), pp 512 INTEGER(0..511), pp 640 INTEGER(0..639), pp1024 INTEGER(0..1023), pp1280 INTEGER(0..1279), pp2048 INTEGER(0..2047), pp2560 INTEGER(0..2559) pp5120 INTEGER(0..1), pp10240 INTEGER(0..3) |
| drx-ULRetransmissionTimer | Same as drx-RetransmissionTimer | none | pp {0, 1, 2, 4, 6, 8, 16, 24, 33} |
| UL HARQ RTT Timer | Samilar to HARQ RTT Timer | eMTC: 4 subframes | 4 subframes |

# Discussion

In our view the long DRX cycle should in NB-IoT primarily be used by the UE while waiting for a response to a previously sent/received request message, or while waiting for scheduling to continue for the UE in case of a system that is loaded and where the eNB does not currently have time to schedule the UE. This means that the duration of the long DRX cycle in NB-IoT in many cases should be measured in seconds. It would seem that the maximum long DRX cycle should be the same as in legacy LTE, which is 10240 ms, because this will allow a reasonable long sleep period for the UE, while at the same time the eNB can schedule the UE at least once every 10 seconds. A reasonable minimum value of the long DRX cycle could be 256 ms, and it is suggested to use fewer long DRX cycle durations. A suggestion of suitable values is:

Proposal 1: Use the following long DRX cycles: sf256, sf512, sf1024, sf2048, sf5120, sf10240, spare2, spare1

Since the number of UEs that will be in RRC connected mode at the same time is rather limited in NB-IoT, and since configuring many different DRX offsets results in unnecessary signalling , we propose to reduce the number of offsets that can be used for a given long DRX cycle duration. This also has the advantage that there will be less overlap between different OnDuration periods for different UEs.

Proposal 2: Use only every 8:th subframe as starting position of an OnDuration period. In the case of sf256 long DRX cycle, the following OnDuration start offsets would be applicable: INTEGER(0, 8, 16, 24, .. 248), and similar for other DRX cycles.

The OnDurationTimer is measured in PDCCH periods (pp). For NB-IoT it is important that the UE will get a chance to be scheduled at the next OnDuration opportunity but it is also important to ensure that the UE is not wasting a lot of energy while waiting at an OnDuration period. This is important for UEs in poor coverage due to the long transmission times so for these a rather small pp should be used. However, for good coverage UEs scheduling flexibility is desired so some medium sized values should be present. A suggestion of suitable values is:

Proposal 3: Use the following OnDurationTimer values: pp{1, 2, 3, 4, 8, 16, 32, spare1}

The drx-InactivityTimer is measured in PDCCH periods (pp). For NB-IoT this timer is primarily used while waiting for a subsequent transmission of the next TB or a possible retransmission of the most recent sent/received TB. In a low loaded system it is expected that the eNB should be able to schedule the UE at the next possible scheduling occasion, but in case there is other traffic in the same cell, it may mean that the eNB cannot schedule the UE immediately but has to delay the scheduling for a number of PDCCH periods. It is, however, not optimal to expect the UE to be awake for too long time because this will just waste battery and it is then probably better to wait until the next OnDuration for poor coverage UEs. For good coverage UEs scheduling flexibility is desired so some medium sized values should be present also here:

Proposal 4: Use the following drx-InactivityTimer values: pp{1, 2, 3, 4, 8, 16, 32, spare1}

The purpose of the drx-RetransmissionTimer and the drx-UlRetransmissionTimer timers is to ensure that the UE is awake for some time to receive a subsequent re-transmission request. But since the UE will anyhow be awake due to the drx-InactivityTimer and this timer is started at the end of a HARQ procedure, it would seem that this usage is not really needed in NB-IoT, and therefore we propose that the default values of these timers should be set to 0.

Proposal 5: The default value used for the drx-RetransmissionTimer and the drx-UlRetransmissionTimer is suggested to be set to 0.

# Conclusion

Proposal 1: Use the following long DRX cycles: sf256, sf512, sf1024, sf2048, sf5120, sf10240, spare2, spare1.

Proposal 2: Use only every 8:th subframe as starting position of an OnDuration period. In the case of sf256 long DRX cycle, the following OnDuration start offsets would be applicable: INTEGER(0, 8, 16, 24, .. 248), and similar for other DRX cycles.

Proposal 3: Use the following OnDurationTimer values: pp{1, 2, 3, 4, 8, 16, 32 spare1}.

Proposal 4: Use the following drx-InactivityTimer values: pp{1, 2, 3, 4, 8, 16, 32 spare1}.

Proposal 5: The default value used for the drx-RetransmissionTimer and the drx-UlRetransmissionTimer is suggested to be set to 0.

# References

- R2-163133, “Report of the LTE breakout session (NB-IoT)”, RAN2#93bis