3GPP TSG-RAN WG2 NR ASN.1 Ad-Hoc	Tdoc R2-2204335

Electronic meeting, April 20-22, 2022

Agenda Item:	4

Source:	Ericsson

Title:	RIL E007, Set Modify Release structure

Document for:	Discussion, Decision

# 1	Introduction

This document is partly a re-submission of R2-2110778 submitted to RAN2#116e, and discussed in AT email discussion 048 (R2-2111606 Summary of offline 048 Rel-17 RRC SetModifyRelease). The outcome was this:

[048] For Rel-17, RAN2 can consider adding the possibility to “release and add” larger IEs as required on a case by case basis. FFS how to release it in ASN.1.

Section 2.1 gives some background.

Section 2.2 discusses the usefulness of SetModifyRelease in the context of [E007]

Text proposal is provided in Annex.

# 2	Discussion

## 2.1 Background

From the first NR release it was agreed to have a SetupRelease structure to enable an easier release of a group of fields. This contribution discusses the issues with the current structure and possible improvement for Rel-17.

The SetupRelease structure allow us to

- release an entire structure if it was configured before,

- setup the entire configuration if the field was not configured before

- apply delta configuration if it was configured before

- keep the configuration entirely unchanged if the SetupRelease structure is marked Need M.

However, it is not possible to replace a previously provided configuration by a new one, i.e., to explicitly disable delta signalling for the contained information element and do a local full configuration instead.

We consider this option desirable since delta configuration is sometimes difficult and sometimes impossible to generate for a branch of IEs. For example if the gNB wants to release a field that is marked as “Need M” but also change or set another field, the only option is to fall back to a full-config. Currently, this must be carried out on the top level and hence also the L2 stacks must be reset. The only alternative would be to release the field in a first RRCReconfiguration and to add it again in a subsequent RRCReconfiguration. However, this is not possible either in many cases since a configuration without e.g. pdcchConfig or pdschConfig would not be valid.

The AddMod/Release lists allow a local full configuration of the list elements by adding the element’s index to the release list and re-adding the new element with the same index to the AddMod list. The SetupRelease structure does unfortunately not offer this option.

The SetupRelease structure does not allow to replace a previously provided configuration entirely by a new one, i.e., to perform a local full configuration of the contained IE.

As shown below the current structure treats setup and delta configuration in the same way (i.e. via including a SetupRelease structure set to value "setup"):

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

If the field had been configured previously, a subsequent use of "setup" is treated as a delta - with all benefits and drawbacks. If the network wants to replace the previous configuration (i.e., not delta but rather a "local full configuration") it must send a first message where the field is set to "release" and a subsequent message where it is set to "setup" again. Obviously, such a two-step procedure is undesirable in general and often not possible at all (e.g. the UE would not be able to have pdcch-Config simply released while no configuration is added in the same message). Hence, the network will practically fall back to a regular "fullConfig".

The current SetupRelease structure does not differentiate delta configuration from the case where the network may want to replace entirely the configuration under the SetupRelease structure (i.e. a “local full configuration”).

To enable a "local full config" we suggest to define a new parameterized type as follows:

SetModifyRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

set           ElementTypeParam     // sets or replaces the previous values

delta           ElementTypeParam     // a delta to a previous configuration

}

In this manner, the set field can be used any time the network wants to provide new variant of the contained IE. The above structure should anyway be used only for large IEs. Small fields should generally be "mandatory present" or "optional Need R".

For Rel-17, where a SetupRelease structure would be applicable, a SetModifyRelease structure should be used instead, i.e. a structure that allow release, delta and a full replacement of a previous configuration of the contained IE.

## 2.2	E007, beamFailureRecoveryServingCellConfig-r17

One example that would benefit from the new SetModifyRelease is the beamFailureRecoveryServingCellConfig-r17.

With the existing SetupRelease, delta configuration becomes complex. E.g., to configure the UE with a beamFailureRecoveryServingCellConfig-r17 with only 2 of the 3 fields (e.g. omitting candidateBeamRSList2-r17, while the UE was previously configured with all 3 fields, would require removal and addition of the BWP-Downlink, that is a full configuration of the BWP-Downlink.

With the SetModifyRelease, this would be accomplished by the “set” option, to get a “local full configuration” of the beamFailureRecoveryServingCellConfig-r17, instead of a full configuration of the whole BWP-Downlink.

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16                                        OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16                                       OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16                               OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoverySCellConfig-r16}                 OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                                     OPTIONAL    -- Need M

]],

[[

deactivatedMeasGapList-r17          SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17            OPTIONAL,   -- Cond PreConfigMG

beamFailureRecoveryServingCellConfig-r17  SetupRelease { BeamFailureRecoveryServingCellConfig-r17}    OPTIONAL,   -- Need M

harq-FeedbackEnablingforSPSactive-r17 BOOLEAN                                                         OPTIONAL,   -- Need R

cfr-ConfigMulticast-r17             SetupRelease { CFR-ConfigMulticast-r17 }                          OPTIONAL,   -- Need M

dl-PRS-ProcessingWindowPreConfigAddModList-r17  DL-PRS-ProcessingWindowPreConfigAddModList-r17        OPTIONAL,   -- Need N

dl-PRS-ProcessingWindowPreConfigReleaseList-r17 DL-PRS-ProcessingWindowPreConfigReleaseList-r17       OPTIONAL,   -- Need N

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17                                            OPTIONAL    -- Need R

]]

BeamFailureRecoveryServingCellConfig-r17 ::= SEQUENCE {

rsrp-ThresholdBFR-r17             RSRP-Range                                                                    OPTIONAL, -- Need M

candidateBeamRSList-r17           SEQUENCE (SIZE(1..maxNrofCandidateBeams-r17)) OF CandidateBeamRS-r16          OPTIONAL, -- Need M

candidateBeamRSList2-r17          SEQUENCE (SIZE(1..maxNrofCandidateBeams-r17)) OF CandidateBeamRS-r16          OPTIONAL, -- Need M

...

}

The existing alternative to use SetupRelease is to use Need R, but this completely abandons the delta signalling principle.

Since all cases that currently uses SetupRelease would clearly benefit in similar way for delta signalling. Hence, we propose:

Introduce SetModifyRelease in all Rel-17 fields that currently use SetupRelease.

Draft TP is provided in Annex

# 3	Conclusion

In the previous sections we made the following observations:

- The SetupRelease structure does not allow to replace a previously provided configuration entirely by a new one, i.e., to perform a local full configuration of the contained IE.

The current SetupRelease structure does not differentiate delta configuration from the case where the network may want to replace entirely the configuration under the SetupRelease structure (i.e. a “local full configuration”).

Based on the discussion in the previous sections we propose the following:

- For Rel-17, where a SetupRelease structure would be applicable, a SetModifyRelease structure should be used instead, i.e. a structure that allow release, delta and a full replacement of a previous configuration of the contained IE.

Introduce SetModifyRelease in all Rel-17 fields that currently use SetupRelease.

A TP is provided in Annex.

# Annex: TP on SetupModifyRelease

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUPRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUPRELEASE-STOP

-- ASN1STOP

#### –	SetupModifyRelease

SetupModifyRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.x for guidelines.

-- ASN1START

-- TAG-SETUPMODIFYRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

set             ElementTypeParam     // sets or replaces the previous values

delta           ElementTypeParam     // a delta to a previous configuration

}

-- TAG-SETUPMODIFYRELEASE-STOP

-- ASN1STOP

<cut>

## A.3.x	Guidelines on use of parameterised SetModifyRelease type

The usage of the parameterised SetModifyRelease type is like a function call in programming languages where the element type parameter is passed as a parameter. The parameterised type only implies a textual change in abstract syntax where all references to the parameterised type are replaced by the compiler with the release/set/delta choice. Two examples of the usage are shown below:

-- /example/ ASN1START

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX               SetModifyRelease { IE-rX }                   OPTIONAL,      --  Need M

...

}

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX               SetModifyRelease { Element-rX }

}                                                                   OPTIONAL,       -- Need M

Element-rX ::= SEQUENCE {

field1-rX                  IE1-rX,

field2-rX                  IE2-rX                               OPTIONAL        -- Need N

}

-- /example/ ASN1STOP

The SetModifyRelease is always be used with only named IEs, i.e. the example below is not allowed:

-- /example/ ASN1START

RRCMessage-rX-IEs ::= SEQUENCE {

field-rX       SetModifyRelease { SEQUENCE {   -- Unnamed SEQUENCEs are not allowed!

field1-rX                  IE1-rX,

field2-rX                  IE2-rX                         OPTIONAL        -- Need N

}

}                                                                     OPTIONAL,   -- Need M

}

-- /example/ ASN1STOP

If a field defined using the parameterized SetModifyRelease type requires procedural text, the field is referred to using the values defined for the type itself, namely, "set", " delta" and "release". For example, procedural text for field-rX above could be as follows:

1> if field-rX is set to "set":

2> do something;

1> else if field-rX is set to "delta":

2> do something;

1> else (field-rX is set to "release"):

2> release field-rX (if appropriate).