3GPP TSG-RAN WG2 NR ASN.1 adhoc meeting	R2-2204347

Online, 20 - 22 April 2022

Title: 	Discussion on merged handling of Meas Gaps and Pos gaps

Source: 	vivo

Agenda item:	4

Document Type:	Discussion and Decision

1. Introduction

During ASN.1 review, the following proposed changes were made regarding the handling of Meas Gaps and Pos gaps:

| ID | Description | Proposed Change |
| --- | --- | --- |
| M607 | Clarification on pre-configured positioning gap and pre-configured gap is needed | Related to Z141 and H566. There is “pre-configured gap” introduced by MGE WI and “pre-configured positioning gap” from positioning WI. There are different activation/deactivation procedure for these two “pre-configured” gaps. Whether to harmonize ASN.1 code for the two gaps could be discussed. When to use which gap should also be discussed. In addition, there is an FFS point from MGE WI that “FFS whether and how to capture the UE behavior on PRS measurements within measurement gaps when a Pre-configured MG is provided by the network (as indicated in RAN4 LS R4-2206789)”. We will have a Tdoc to discuss the needed change. |
| H566 | It should be” preconfig-MG for positioning is not activated”. With concurrent gap, there can be activated preconfiguration gap for positioning and other purpose. While if a preconfigured gap not for the purpose for positioning is activated/deactivated should not affect the spec here | change the condition to "there is no activated preconfigured measurement gap for positioning". |
| H567 | The UE does not need to send the deactivation request when a new preconfiguraton measurement gap is needed or the current meas gap needs to be modified. The UE only needs to send another UL MAC CE for the activation of preconfig MG or another LocationMeasurementIndication to update the request. The reason is that there can be only one acticated MG for positioning at a certain time with the agremment from the dicsussin on concurrent gap. | Remove the condition “2> if a request from upper layers to transmit either a new preConfigGapID or to modify the current measGapConfig is received; or” Also, change the “if preconfigGapID is activated” to If there is activated preconfigmeasurement gap for positioning” |
| Z141 | Although R17 preconfigured gap can be configured for positioning, once one PosEnh gap is activated, we think it is same as the PosEnh gap configured in R16, and one gap can be used for both positioning measurement and normal RRM measurement. So we think there is no need to define separate “addModList” for positioning, it is better to use a unified signalling for both R17 positioning gap and MGE. Otherwise, it is hard to capture the association between the two structures. | Delete these two IEs, and reuse gapUEToAddModList-r17 to provide pre-configured PoS gap configurations, we will bring discussion paper for this issue. |
| H581 | In MGenh, the measGapConfig can already associate a MG to a PRS. We should reuse the signalling introduced in MGenh for positioning. |  |

In this document, we further discuss how the two new MGS can co-exist.

2. Discussion

2.1 Issue 1: separate or combined gapUEToAddModList-r17

In the current mega CR, the pre-configuration for MGenh and PosEnh are not aligned, i.e., the pre-configuration for MGenh is placed in MeasGapConfig while the pre-configuration for PosEnh is placed in MeasConfig.

MeasGapConfig ::= SEQUENCE { gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M ..., [[ gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M gapUE SetupRelease { GapConfig } OPTIONAL -- Need M ]], [[ gapUEToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig OPTIONAL, -- Need N gapUEToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17 OPTIONAL, -- Need N gapFR1ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig OPTIONAL, -- Need N gapFR1ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17 OPTIONAL, -- Need N gapFR2ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF GapConfig OPTIONAL, -- Need N gapFR2ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-1-r17)) OF MeasGapId-r17 OPTIONAL -- Need N ]] }

MeasConfig ::= SEQUENCE { measObjectToRemoveList MeasObjectToRemoveList OPTIONAL, -- Need N measObjectToAddModList MeasObjectToAddModList OPTIONAL, -- Need N reportConfigToRemoveList ReportConfigToRemoveList OPTIONAL, -- Need N reportConfigToAddModList ReportConfigToAddModList OPTIONAL, -- Need N measIdToRemoveList MeasIdToRemoveList OPTIONAL, -- Need N measIdToAddModList MeasIdToAddModList OPTIONAL, -- Need N s-MeasureConfig CHOICE { ssb-RSRP RSRP-Range, csi-RSRP RSRP-Range } OPTIONAL, -- Need M quantityConfig QuantityConfig OPTIONAL, -- Need M measGapConfig MeasGapConfig OPTIONAL, -- Need M measGapSharingConfig MeasGapSharingConfig OPTIONAL, -- Need M ..., [[ interFrequencyConfig-NoGap-r16 ENUMERATED {true} OPTIONAL -- Need R ]], [[ posMeasGapPreConfigToAddModList-r17 PosMeasGapPreConfigToAddModList-r17 OPTIONAL, -- Need N posMeasGapPreConfigToRemoveList PosMeasGapPreConfigToRemoveList-r17 OPTIONAL -- Need N ]] }

From our perspective, the two pre-MGs are not identical, i.e. the pre-MG for MGenh is automatically activated and the pre-MG for PosEnh requires MAC CE activation. However, both are used to perform measurements and therefore can be placed in the same place. Our preference is that the pre-MG for PosEnh shall be moved into MeasGapConfig and shall use a separate gapToAddModList.

Proposal 1: The pre-MG for PosEnh shall be moved into MeasGapConfig and shall use a separate gapToAddModList from that for MGenh.

2.2 Issue 2: relation between pre-MGs

The MG request process for positioning is as follows.

1> if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient: 2> if preconfigured measurement gaps are configured and the UE considers that at least one of the preconfigured gaps meets the measurement gap requirements: Editor’s Note: check if Reference for TS 38.133 on measurement gap requirements is needed. 3> trigger the lower layers to initiate the measurement gap activation request using UL MAC CE as specified in TS 38.321 [6];

In our understanding, the UE behavior when configured with both pre-MG for MGenh and PosEnh is:

- The per-MG for MGenh is activated when the corresponding BWP/CA is activated;

- The UE requires measurement gaps for performing the requested location measurements while the activated measurement gaps are not sufficient;

- If one of the pre-MG for PosEnh meets the measurement gap requirements, the UE sends the MAC CE to activate that pre-MG. Otherwise, the UE sends RRC LocationMeasurementIndication message to request for new MG.

Proposal 2: Clarify that the pre-condition to request for per-MG of PosEnh is that the measurement gaps are either not configured or the activated measurement gaps are not sufficient.

3. Conclusion

Proposal 1: The pre-MG for PosEnh can be moved into MeasGapConfig and shall use a separate gapToAddModList from that for MGenh.

Proposal 2: Clarify that the pre-condition to request for per-MG of PosEnh is that the measurement gaps are either not configured or the activated measurement gaps are not sufficient.

4. Reference