3GPP TSG-RAN WG2 NR ASN.1 Ad-Hoc Electronic	R2-2204351

Elbonia, 20 – 22 April 2022

Agenda item:	4

Source:	Nokia, Nokia Shanghai Bell

Title:	[N011] [N016] SIB extensions

WID/SID:	Multiple - Release 17

Document for:	Discussion and Decision

# 1	SIB1 size [N016]

Since SIB1 is a mandatory for all SA deployments, its size needs to be kept optimized. At the same time, it's good to ensure it's understandable. This means that

- Related fields should be grouped if they are used together (to save on optionality bits and improve readability)

- Use of ellipsis should be avoided (as that causes at least 1-3 bytes of overhead due to the length field addition)

- Number of bits in a field should be minimized (self-evident)

All of these have some issues in SIB1:

- Related RedCap fields are not grouped: there are 4 fields related to RedCap, all of which are about "enabling flags". They should be grouped together.

- Ellipsis: Ellipsis is used in featurePriorities-r17 (as noted in I412) and SDT-ConfigCommonSIB-r17 (as noted in I416), which creates overhead. Those should be just removed and extensions can use regular NCE mechanism.

- Bit savings: Some fields could perhaps be further optimized:

  - The SEQUENCE uac-BarringInfo-v1700 only needs the outer optionality bit. The inner one seems to have been introduced to allow release of the MINT configuration, but this can be handled by the need code absence condition "Otherwise, the field is absent, Need R".

  - The field hyperSFN seems only needed if eDRX is allowed - hence, it could be discussed if those can be grouped to save one bit.

- SI scheduling: The field si-SchedulingInfo-v1700 is quite large, so it would be good to check if anything can be optimized there. There have been additions to e.g. the CellAccessRelatedInfo, BWP for RedCap, which might benefit from closer look.

Observation 1: Rel-17 additions to SIB1 have not been size-optimized.

Proposal 1: Group the RedCap-related fields in SIB1 under the same SEQUENCE and remove optionality bits from ENUMERATED whose other value is equal to field behaviour on absence.

Proposal 2: Remove ellipsis from in featurePriorities-r17 and SDT-ConfigCommonSIB-r17.

Proposal 3: Remove inner optionality bit of uac-BarringInfo-v1700, i.e. remove OPTIONAL from uac-BarringInfoSetList-v1700, move the Cond MINT to the outer field and add "Need R" to the "if absent" part of the condition.

Proposal 4: Discuss whether the fields hyperSFN-r17 and eDRX-Allowed-r17 could be grouped under one SEQUENCE.

Proposal 5: Discuss whether anything else (e.g. SI-SchedulingInfor-v1700) can be optimized in SIB1 signalling.

# 2	SIB segmentation [N011]

SIB segmentation was first introduced in LTE for the SIB11/12 (i.e. ETWS/CMAS SIBs), and later also to SIB28 (sidelink communication configuration SIB), since the payload of those SIBs can be larger than the maximum size. For the same reasons, NR introduced segmentation to SIB7/8 (i.e. ETWS/CMAS SIBs), SIB12 (sidelink communication), and now in Rel-17 also SIB17 (TRS configuration for IDLE/INACTIVE modes).

Observation 2: Rel-17 is adding yet another SIB that can be segmented.

However, it seems that the SIB segmentation in SIB17 only considers two segments: 0 and 1. But the SIB is still extendible, so if the SIB17 size becomes larger than could fit in two SIB segments, there is a problem. Hence, even if it's not needed now, it seems preferable to just allow up to 64 segments also for SIB17.

Proposal 6: Allow up to 64 segments for the SIB17.

In general, the question is that when SIB segmentation is allowed, should there be a template for avoiding similar mistake? Same as with SetupRelease, the Annex B illusrtates how a "generic" SIB segmentation structure could be defined (including procedural text). However, it can be argued that SIB segmentation should normally be a rare operation, so this may be unlikely to be needed, so whether it's a good idea to introduce such a parameterized type is something to discuss. There could also be just a guideline for segmented SIBs as part of RRC Annex A.

Proposal 7: Discuss whether to introduced parameterized type for segmented SIBs or guideline for how segmented SIBs should be defined in NR RRC.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This documents has made the following observations:

Observation 1: Rel-17 additions to SIB1 have not been size-optimized.

Observation 2: Rel-17 is adding yet another SIB that can be segmented.

And proposed the following:

Proposal 1: Group the RedCap-related fields in SIB1 under the same SEQUENCE and remove optionality bits from ENUMERATED whose other value is equal to field behaviour on absence.

Proposal 2: Remove ellipsis from in featurePriorities-r17 and SDT-ConfigCommonSIB-r17.

Proposal 3: Remove inner optionality bit of uac-BarringInfo-v1700, i.e. remove OPTIONAL from uac-BarringInfoSetList-v1700, move the Cond MINT to the outer field and add "Need R" to the "if absent" part of the condition.

Proposal 4: Discuss whether the fields hyperSFN-r17 and eDRX-Allowed-r17 could be grouped under one SEQUENCE.

Proposal 5: Discuss whether anything else (e.g. SI-SchedulingInfor-v1700) can be optimized in SIB1 signalling.

Proposal 6: Allow up to 64 segments for the SIB17.

Proposal 7: Discuss whether to introduced parameterized type for segmented SIBs or guideline for how segmented SIBs should be defined in NR RRC.

Annex A illustrates the results of proposals 1-5, and Annex B shows the result of proposal 6 and annex C illustrates example of the parameterized type for segmented SIBs.

# Annex A: SIB1 in RRC v.17.0.0

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                              OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                                              OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                           OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                                               OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                                               OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                               OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants                                           OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                           OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                         OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                                   OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs                                                  OPTIONAL

}

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}                                                   OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16                                           OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1630-IEs                                                     OPTIONAL

}

SIB1-v1630-IEs ::=               SEQUENCE {

uac-BarringInfo-v1630            SEQUENCE {

uac-AC1-SelectAssistInfo-r16     SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16

}                                                                                                   OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1700-IEs                                                     OPTIONAL

}

SIB1-v1700-IEs ::=               SEQUENCE {

hsdn-Cell-r17                        ENUMERATED {true}                                              OPTIONAL,  -- Need R

ue-TimersAndConstants-RemoteUE-r17   UE-TimersAndConstants-RemoteUE-r17                             OPTIONAL,  -- Need R

uac-BarringInfo-v1700                SEQUENCE {

uac-BarringInfoSetList-v1700         UAC-BarringInfoSetList-v1700

}                                                                                                   OPTIONAL,  -- Cond MINT

sdt-ConfigCommon-r17                 SDT-ConfigCommonSIB-r17                                        OPTIONAL,  -- Need R

featurePriorities-r17        SEQUENCE {

redCapPriority-r17           FeaturePriority-r17                                                OPTIONAL,

slicingPriority-r17          FeaturePriority-r17                                                OPTIONAL,

ce-Priority-r17              FeaturePriority-r17                                                OPTIONAL,

sdt-Priority-r17             FeaturePriority-r17                                                OPTIONAL,

}                                                                                                   OPTIONAL,  -- Need R

si-SchedulingInfo-v1700      SI-SchedulingInfo-v1700                                                OPTIONAL,  -- Need R

eDRX-Config                  SEQUENCE {

hyperSFN-r17                 BIT STRING (SIZE (10)),

eDRX-Allowed-r17             ENUMERATED {allowed, notAllowed}

}                                                                                                   OPTIONAL,  -- Need R

-- FFS whether eDRX-allowed is one or two bits, i.e. separate for idle/inactive

redCap-Config-r17            SEQUENCE {

halfDuplexRedCap-r17           ENUMERATED {allowed, notAllowed},

-- FFS whether halfDuplexRedCapAllowed is kept, remove also from related procedure

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred},

intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed}

}                                                                                                   OPTIONAL,  -- Need R

nonCriticalExtension         SEQUENCE {}                                                            OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range,

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

}

FeaturePriority-r17 ::= INTEGER (0..15)

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellSelectionInfo Parameters for cell selection related to the serving cell.

eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

eDRX-Allowed The presence of this field indicates that extended DRX is allowed in the cell for UEs in RRC_IDLE and in RRC_INACTIVE. The UE shall stop using extended DRX if eDRX-Allowed is not present.

featurePriorities Indicates priorities for features, such as RedCap, Slicing, etc. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

halfDuplexRedCap-Allowed FFS: The presence of this field indicates whether the cell supports half-duplex FDD RedCap UEs.

hsdn-Cell This field indicates this is a HSDN cell as specified in TS 38.304 [20].

hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around.

idleModeMeasurementsEUTRA This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements.

idleModeMeasurementsNR This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements.

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

intraFreqReselectionRedCap Controls cell selection/reselection to intra-frequency cells for RedCap UEs when this cell is barred, or treated as barred by the RedCap UE, as specified in TS 38.304 [20]. If not present, a RedCap UE treats the cell as barred, i.e.,the UE considers that the cell does not support RedCap.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is absent, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

sdt-RSRP-Threshold RSRP threshold for UE to determine whether to perform SDT procedure, as specified in TS 38.321 [3].

sdt-DataVolumeThreshold Data volume threshold used to determine whether SDT can be initiated, as specified in TS 38.321 [3]. Value byte32 corresponds to 32 bytes, value byte100 corresponds to 100 bytes, and so on.

sdt-LogicalChannelSR-DelayTimer The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT, as specified in TS 38.321 [3]. Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on. If this field is not configured, then logicalChannelSR-DelayTimer is not applied for SDT logical channels.

servingCellConfigCommon Configuration of the serving cell.

t319a Initial value of the timer T319a. Value ms100 corresponds to 100 milliseconds, value ms200 corresponds to 200 milliseconds and so on.

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. If plmnCommon is chosen, the UAC-AccessCategory1-SelectionAssistanceInfo is applicable to all the PLMNs and SNPNs in plmn-IdentityInfoList and npn-IdentityInfoList. If individualPLMNList is chosen, the 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. If uac-AC1-SelectAssistInfo-r16 is present, the UE shall ignore the uac-AccessCategory1-SelectionAssistanceInfo.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. The 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. Value notConfigured indicates that Access Category1 is not configured for the corresponding PLMN/SNPN.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs/SNPNs, unless overwritten by the PLMN/SNPN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. The cell operating as PCell always provides this field.

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses fullI-RNTI and RRCResumeRequest1 if the field is present, or shortI-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| MINT | The field is optionally present, Need R, in a cell that provides a configuration for disaster roaming, otherwise it is absent, Need R. |
| Standalone | The field is mandatory present in a cell that supports standalone operation, otherwise it is absent. |

# Annex B: Correction to SIB17 segmentNumber

#### –	SIB17

SIB17 contains configurations of TRS resources for idle/inactive UEs.

SIB17 information element

-- ASN1START

-- TAG-SIB17-START

SIB17-r17 ::=               SEQUENCE {

segmentNumber-r17           INTEGER (0..63),

segmentType-r17             ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r17        OCTET STRING

}

# Annex C: Parameterized type for Segmented SIBs

#### –	SegmentedSIB

SegmentedSIB allows the SIB-Param to be used as the referenced data type for the defining SIBs that can be transmitted in sgments.

-- ASN1START

-- TAG-SEGMENTED-SIB-START

SegmentedSIB { SIB-Param } ::= SEQUENCE {

segmentNumber-r17           INTEGER (0..63),

segmentType-r17             ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r17        OCTET STRING

}

-- TAG-SEGMENTED-SIB-STOP

-- ASN1STOP

SegmentedSIB field descriptions

segmentContainer This field includes a segment of the encoded SIB-Param. The size of the included segment in this container should be small enough that the SIB message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when the SIB is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB-Param. A segment number of zero corresponds to the first segment, A segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

##### 5.2.2.4.X	Actions upon reception of SegmentedSIB

Upon receiving SegmentedSIB, the UE shall:

1>	if the UE has stored at least one segment of SegmentedSIB and the value tag of SegmentedSIB has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble Param-SIB from the received segments.