# 3GPP TSG-RAN WG3#100	R3-182558

# Busan, South Korea, 21-25 May 2018

3GPP TSG-RAN WG2 #101bis			R2-1806503

Sanya, China, 16th-20th April 2018

Title:	LS on different CGIs for different SSBs

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

#### Name:	Sergio Parolari

E-mail Address:	  Sergio.Parolari@zte.com.cn

1. Overall Description:

RAN2 discussed the implications of the agreement that there can be multiple SS/PBCH blocks (SSBs) within a carrier, which can have the same or different PCI.

RAN2 agreed that different SSBs with RMSI define different individual cells (regardless of whether the same or different PCI is used) characterized by different Cell Global Identifiers (CGIs), which are globally unique.

RAN2 would also like to highlight that, for a UE in RRC Connected, the radio resources used by the cell accessed by the UE via a given SSB might overlap in the frequency domain with the radio resources of other cells accessed via other SSBs within a carrier.

RAN2 invites RAN3 to take this into account where needed, e.g. for Xn and F1 procedures.

2. Actions:

RAN2 invites RAN3 to take the above RAN2 agreements into account.

3. Date of Next RAN2 Meetings:

RAN2#102	21 - 25 May 2018 	Busan, South Korea

RAN2#AH1807	2 - 6 July 2018		Montreal, Canada