3GPP TSG-RAN WG3 #100	R3-182999

Busan, South Korea, 21-25 May 2018

Agenda Item:	10.5.1

Source:	NEC

Title:	(TP for SA BL CR for TS 38.473): solving the FFS on 5G-S-TMSI

Document for:	Agreement

# Introduction

This contribution presents a text proposal to address the FFS on the range of 5G-S-TMSI.

# Discussion

In the LS from SA2 to RAN2 [1], SA2 asked RAN2 whether it is possible to extend the encoding space for the 5G-S-TMSI from 40 bits to e.g. 48 bits.

In RAN2#101bis, RAN2 analysed the impact of increasing the size of 5G-S-TMSI (specifically to 48 bits) for both NR and LTE connected to 5GC, and agreed the following:

RAN2#101bis Agreements 1 Final decision on 5G-S-TMSI size is an SA2 decision 2 Reply to SA2 that it is feasible to increase the 5G-S-TMSI size (from 40 bits to 48bits) for LTE and NR. Indicate that there are some impacts to LTE and NR specs to support it and RAN2 is continuing to discuss to conclude the final approach to be taken.

RAN2 provided the above agreements in reply LS to SA2 [2]. Then, shortly after sending the reply LS to SA2, RAN2 received another LS from SA2 [3], in which SA2 asks RAN2 to extend the size of 5G-S-TMSI 48 bits.

Proposal 1: The size of 5G S-TMSI for both NR and LTE connected to 5GC is 48 bits.

Proposal 2: RAN3 is kindly asked to agree the TP (below) for TS 38.473.

# References

[1] R2-1804256/S2-182964, LS_on_5G-S-TMSI-space, SA2 to RAN2

[2] R2-1806394, Reply LS on 5G-S-TMSI, RAN2 to SA2

[3] R2-1806448, Reply LS on 5G-S-TMSI, SA2 to RAN2

# Text Proposal for TS 38.473

#### ----------------------------------------Start of the First Change -----------------------------------------------

#### 9.3.1.x9	CN UE Paging Identity

The 5G-S-TMSI is used as UE identifier for CN paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE CN UE paging identity | M |  |  |  |
| >5G-S-TMSI |  |  |  |  |
| >>5G-S-TMSI | M |  | BIT STRING (SIZE(48)) |  |
| >FFS |  |  |  |  |

#### ----------------------------------------End of the First Change -----------------------------------------------

#### ----------------------------------------Start of the second Change (ASN.1)--------------------------------

### 9.4.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[skip parts no change]

CNUEPagingIdentity ::= CHOICE {

g5-S-TMSI			 BIT STRING (SIZE(48)),

...

}

#### ----------------------------------------End of the second Change (ASN.1)--------------------------------