3GPP TSG RAN WG3 #100	Tdoc R3-183202

# Busan, Korea, 21st- 25th May 2018

Agenda Item:	10.10.7.3

Source:	Ericsson

Title:	(TP for SA BL CR for TS 38.473): Measurement configuration for SA

Document for:	TP for SA BL CR for TS 38.473

# Introduction

At the last meeting, it was agreed to introduce the MeasConfig IE in the direction from gNB-CU to gNB-DU. The MeasConfig IE contains a list of frequencies for which the gNB-CU requests the gNB-DU to generate gaps. An LS was sent to RAN2 for confirmation. Even if RAN2 did not confirm the validity of the solution yet, since this is the last meeting before functional freeze, we suggest removing the FFS in TS 38.473. If RAN2 identifies issues with the solution, then the necessary corrections can be applied later. Note that eventual corrections will not require non-backward compatible changes.

Proposal 1:	Remove the FFS related to measurement configuration.

Proposal 2:	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for SA BL CR for TS 38.473

<<<<<<<<<<<<<<<<<<<< Requested Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | CG-ConfigInfo, as defined in TS 38.331 [8]. |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | UE-CapabilityRAT-ContainerList, as defined in TS 38.331 [8]. |
| MeasConfig | O |  | OCTET STRING | MeasConfig, as defined in TS 38.331 (without MeasGapConfig). For EN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. For NG-RAN, includes the list of FR1 and/or FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps and the gap type (per-UE or per-FR). |
| Handover Preparation Information | O |  | OCTET STRING | HandoverPreparationInformation, as defined in TS 38.331 [8]. |

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE and according to the requested gap type (per-UE or per-FR). |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>